Discussion Question 11E
P212, Week 11
Low-Pass and High-Pass Filters


[image: image1.wmf],max

max

,maxmax

R

,max

0

as .  From figure  V

and 

C

C

R

CR

V

IX

R

VIRC

VV

==®

w

w®¥»

E

E

==

The circuit shown at right has an AC generator of peak EMF Emax and frequency  connected in series with a resistor R and a capacitor C.  Let’s analyze the behavior of this circuit at extreme values of the frequency . 
Important note: Since there is no inductor present, you may think that we’re back to the material of “RC Circuits” that we studied a month ago.  But not so!  Those “RC circuits” were connected to DC batteries of constant voltage!  This one is being driven by an AC generator, which makes it behave in a totally different way.  This is an RLC circuit like any other … the fact that L = 0 just simplifies things a little bit.

[image: image2.wmf]R,maxR,maxR,max

C,maxC,maxC,max

For the resistor, which is is true?

a) V   b) V    c)V

For the capacitor, which is is true?

 a) V   b) V    c)V

»

»

=?

=?

EEE

EEE

(a)  Draw a phasor diagram for this circuit, showing the relative phases of the voltages VR, VC, and E.

(b)  Now draw phasor diagrams for the cases when the driving frequency  is extremely small, and when it is extremely large.  In both cases, please answer the following questions:

· How does the peak voltage VR,max across the resistor compare with the generator Emax? 
(Are they about the same? Or is one very-much smaller than the other?)

· How does the peak voltage VC,max across the capacitor compare with the generator Emax? 
(Are they about the same? Or is one very-much smaller than the other?)

[image: image3.wmf]R,maxR,maxR,max

C,maxC,maxC,max

For the resistor, which is is true?

a) V   b) V    c)V

For the capacitor, which is is true?

 a) V   b) V    c)V

»

»

=?

=?

EEE

EEE

[image: image4.wmf]In Circuit A, the loud speaker measures 

the voltage across R which 

is very low when Thus Circuit A is a low

 pass filter.

In Circuit B, the loud speaker measures 

the voltage across L which 

is very lo

w®¥

w when 0.Thus Circuit B is a high pass f

ilter.

w®


[image: image5.wmf],max

,max,max

,max

,max,max

 so as  

and as 0 . The phases look 

like this

L

L

LR

R

LR

V

X

L

VV

VRR

VV

w

==w®¥

w®

?

=

[image: image6.wmf],max

max

,maxmax

C

,max,max

0

as 0.  From figure  V

and 

R

CC

RC

V

IR

RC

VIX

VV

==w®

w®»

E

E

==


[image: image7.wmf],max

,max,max

,max

,max,max

 so as  

and as 0 . The phases look 

like this

L

L

LR

R

LR

V

X

L

VV

VRR

VV

w

==w®¥

w®

?

=

[image: image8.emf]V

R

V

C



V

R

V

C




[image: image9.emf]V

C

V

R



V

C

V

R



V

R




[image: image10.emf]V

R



V

C

V

R



V

C


[image: image11.emf]C

R

E

C

R

E


[image: image19.emf]V

L

V

R



V

R



V

L

High 



limit

V

R 

<<  



V

L 

<<  



Low 



limit

V

L

V

R



V

L

V

R



V

R



V

L

High 



limit

V

R 

<<  



V

L 

<<  



Low 



limit


A common application of the circuit we’ve been considering is in audio amplifiers, which process music signals containing many different frequencies.  In this application, the generator EMF is produced by something like a CD player, or an old-fashioned record player.  This input signal Vin is sent through the resistor and capacitor, and an output signal Vout is produced by connecting an output device (like a speaker) in parallel across either R or C.  Here are the two configurations:

[image: image12.wmf]R,maxR,maxR,max

C,maxC,maxC,max

For the resistor, which is is true?

a) V   b) V    c)V

For the capacitor, which is is true?

 a) V   b) V    c)V

»

»

=?

=?

EEE

EEE


Circuit  A

[image: image13.emf]R

C

V

in

V

out

R

C

V

in

V

out


Circuit  B
Suppose our music input comes from an old record-player, and is subject to hiss-and-pop scratch noise.  These distortions of the music are very short-lived, and so are of very high frequency.  Thus, we would like our amplifier to remove signals of very high frequency.  This is called a low-pass filter (or “scratch filter”) ( when the input signal is of low frequency, the input signal Vin is passed through to Vout without much change … but high-frequency input signals are blocked.

(c)  Based on your phasor diagrams, which of the circuits shown would provide a low-pass filter?

The loud speaker in Circuit B records the voltage across the capacitor. In the high  limit , VC,max approaches zero and hence the high frequency components are blocked.  Circuit B is then the low pass filter.

(d)  Would the other one provide a high-pass filter (also called a “rumble filter”) which deletes very-low frequency bass noise?

The loud speaker in Circuit A records the voltage across the resistor. In the low  limit, VR,max ( 0 and hence the low frequency components are blocked.  Circuit A is thus a high pass filter.

(e)  Low-pass and high-pass filters can also be constructed using inductors rather than capacitors.  Use your expertise with phasor diagrams to determine which of these two circuits would provide a low-pass filter and which would provide a high-pass filter.

[image: image14.emf]C

R

V

in

V

out

C

R

V

in

V

out


Circuit  A

Circuit  B

[image: image15.emf]L

R

V

in

V

out

R

L

V

in

V

out

L

R

V

in

V

out

R

L

V

in

V

out

R

L

V

in

V

out


[image: image16.wmf],max

max

,maxmax

C

,max,max

0

as 0.  From figure  V

and 

R

CC

RC

V

IR

RC

VIX

VV

==w®

w®»

E

E

==

[image: image17.wmf],max

,max,max

,max

,max,max

 so as  

and as 0 . The phases look 

like this

L

L

LR

R

LR

V

X

L

VV

VRR

VV

w

==w®¥

w®

?

=


Super-low frequency case








Super-high frequency case


























� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���


























� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���








PAGE  
3

[image: image18.wmf]In Circuit A, the loud speaker measures 

the voltage across R which 

is very low when Thus Circuit A is a low

 pass filter.

In Circuit B, the loud speaker measures 

the voltage across L which 

is very lo

w®¥

w when 0.Thus Circuit B is a high pass f

ilter.

w®

_1348814578.unknown

_1348820211.unknown

_1192274964.unknown

_1348814377.unknown

_1192273445.unknown

_1192274771.unknown

