ENSC-283

Assignment #1

Assignment date: Monday Jan. 12, 2009

Due date: Monday Jan. 19, 2009

Problem:  (Newtonian fluid shear stress)

The velocity distribution for the flow of a Newtonian fluid between two wide, parallel plates (see Figure) is given by the equation

|  | u=3Um21-yh2 |  |
| --- | --- | --- |

where Um is the mean velocity. The fluid has the viscosity of 0.04 lb.s/ft2. If Um=2 ft/s and h=0.2 in, determine:

- The shearing stress acting on the bottom wall.

- The shearing stress acting on a plane parallel to the walls and passing through the centerline (midplane).

u

h

h

y

Solution

For this type of parallel flow the shearing stress is obtained from

|  | τ=μdudy | (1) |
| --- | --- | --- |

Thus, if the velocity distribution u=u(y) is known, the shearing stress can be determined at all points by evaluating the velocity gradient, du/dy. For the distribution given

|  | dudy=-3Uyh2 | (2) |
| --- | --- | --- |

- The bottom wally=-h so that

|  | dudy=3Uh | (3) |
| --- | --- | --- |

And therefore the shearing stress is

τbottom wall=μ3Uh=0.04 lb.sft232fts 0.2 in.1ft12in.=14.4 lb/ft2(in direction of flow)

Always use units in  your calculations

This stress creates a drag on the wall. Since the velocity distribution is symmetrical, the shearing stress along the upper wall would have the same magnitude and direction.

- Along the midplane where y=0 it follows from Eq. (2) that

|  | dudy=0 | (4) |
| --- | --- | --- |

And thus the shearing stress is

τmidplane=0

Note (1):

In this problem, we calculated the shearing stress acting on the walls. Same shearing stress but in the opposite direction is acting on the fluid. Why?

Note (2):

From Eq. (2) we see that the velocity gradient is a linear function of y i.e. τ=αy with  α=-3U/h2. Hence, the shearing stress (see Eq. (1)) varies linearly with y and in this particular problem varies from 0 at the center of the channel to
14.4 lb/ft2 at the walls. For the more general case, the actual variation will depend on the nature of the velocity distribution.

τmidplane=0

τwall=14.4 lb.ft2

y, in

0.1

0.2

-0.1

0

-0.2

0

10

5

15

τ, lb/ft2