ENSC-283

Assignment #6

Assignment date: Monday Feb. 23, 2009

Due date: Monday Mar. 2, 2009

Problem:  (Flow under a sluice gate: hydrostatic pressure force)

Water in an open channel is held in by a sluice gate. Compare the horizontal force of the water on the gate (a) when the gate is closed and (b) when it is open (assuming steady flow, as shown). Assume the flow at sections 1 and 2 is incompressible and uniform, and that (because the streamlines are straight there) the pressure distributions are hydrostatic.

Solution:

The forces acting on the control volume include:

- Force of gravity, W

- Friction force, Ff

- Components of Rx and  Ry of reaction force from gate

- Hydrostatic pressure distribution on vertical surface

- Pressure distribution along the bottom surface (not shown)

CV

Ff

W

Rx

p2(y)

p1(y)

Ry

Assumptions:

- Friction force Ff is negligible.

- No body force in x direction.

- Steady flow.

- Incompressible flow.

- Uniform flow at each section.

- Hydrostatic pressure distribution at sections 1 and 2.

Then,

FR1+FR2+Rx=-ρV12wD1+ρV22wD2

The surface forces acting on the CV are due to the pressure distributions and the unknown force, Rx. From our assumptions, we can integrate the gage pressure distribution on each side to compute the hydrostatic forces FR1 and FR2,

FR1=0D1p1ydA=w0D1ρgydy=ρgwD122

FR2=02p2(y)dA=w0D2ρgydy=ρgwD222

Hence,

Rx=ρwV22D2-V12D1-ρgw2D12-D22

The horizontal force per unit width of the gate is

Rxw=ρV22D2-V12D1-ρg2D12-D22

Part (a):

When gate is closed, D2=0 and V2=0. From mass conservation V1=0

Rxw=-ρgD122

Rxw=-999kgm3×9.81ms2×3m22=-44.1 kN

Part (b):

V1=1 [m/s], and V2=7 [m/s], hence,

Rxw=999kgm3×7 ms2×0.429m-1 ms2×3m-12×999kgm3×9.81ms2×3m2-0.429m2=-25.2 kN

Note: As can be seen, the force on the open gate is significantly less than the force on the closed gate as the water accelerates out under the gate.