## Integral Relations for a Control Volume

Control volume approach is accurate for any flow distribution but is often based on the “one-dimensional” property values at the boundaries.

## System and control volume

A system is defined as a fixed quantity of matter or a region in space chosen for study. The mass or region outside the system is called the surroundings.

SYSTEM

BOUNDARY

SURROUNDINGS

Fig.1: System, surroundings, and boundary.

msys=const.      dmsysdt=0

Boundary: the real or imaginary surface that separates the system from its surroundings. The boundaries of a system can be fixed or movable. Mathematically, the boundary has zero thickness, no mass, and no volume.

Open system or control volume is a properly selected region in space. It usually encloses a device that involves mass flow such as a compressor. Both mass and energy can cross the boundary of a control volume.

Note: control volume is an abstract concept and does not hinder the flow in any way.

Fig. 2: Examples of fixed, moving, and deformable control volume.

## Volume and mass flow rate

Fig.3: Volume flow rate through arbitrary surface.

Let n be defined as the unit vector normal to dA. Then the amount of fluid swept through dA in time dt is:

dV=V dt dA cosθ=V.ndA dt

The integral of dV/dt is the total volume rate of flow Q through the surface S:

Q=SV.ndA=SVn dA

where Vn is the normal component of the velocity. We consider n to be the outward normal unit vector. Volume can be multiplied by density to obtain the mass flow m.

m=SρV.ndA=SρVn dA

If density and velocity are constant over the surface S, a simple expression results:

m=ρQ=ρAV

## The Reynolds Transport Theorem

To convert a system analysis to control volume analysis, we must convert our mathematics to apply to a specific region rather than to individual masses. This conversion is called the Reynolds transport theorem.

Consider a fixed control volume with an arbitrary flow pattern through. In general, each differential area dA of surface will have a different velocity V with a different angle θ with the normal to dA.  One can find: In flow volume: VA cosθindt and outflow volume VA cosθoutdt.

Fig. 4: Control volume, Reynolds transport theorem.

Let B be any property of the fluid (energy, momentum, enthalpy, etc.) and β=dBdm be the intensive value of the amount B per unit mass in any small element of the fluid.

| The total amount of B in the control volume is: | BCV=CVβdm=CVβρdV β=dBdm |
| --- | --- |
| A change within the control volume: | ddtCVβρdV |
| Outflow of β from the control volume: | CSβρV cosθ dAout |
| Inflow of β to the control volume: | CSβρV cosθ dAin |

CV and CS refer to control volume and control surface, respectively.

For the system shown in Fig. 4, the instantaneous change of B in the system is sum of the change within, plus the outflow, minus the inflow:

ddtBsys=ddtCVβρdV+CSβρV cosθ dAout-CSβρV cosθ dAin

Note the control volume is fixed in space, the elemental volume do not vary with time. Also we note that V cosθ is the component of V normal to the area element of the control surface. Thus we can write:

Flux term=CSβρVn dAout-CSβρVn dAin=CSβ dmout-CSβ dmin

The vector form of the above equation is:

Flux term=CSβρV.n dA

And the Reynolds transport theorem, in the vector form, becomes:

ddtBsys=ddtCVβρdV+CSβρV.n dA

## One-dimensional flux term approximation

In many situations, the flow crosses the boundaries of the control surface at simplified inlets and exits that are approximately one-dimensional (the velocity can be considered uniform across each control surface). For a fixed control volume, the surface integral reduces to:

ddtBsys=ddtCVβρdV+outletsβimiout-inletsβimiin    where   mi=ρiAiVi

Example 1

A fixed control volume has three one-dimensional boundary sections, as shown in the figure below. The flow within the control volume is steady. The flow properties at each section are tabulated below. Find the rate of change of energy that occupies the control volume at this instant.

| Control surface | type | ρ, kg/m3 | V, m/s | A, m2 | e, J/kg |
| --- | --- | --- | --- | --- | --- |
| 1 | inlet | 800 | 5.0 | 2.0 | 300 |
| 2 | inlet | 800 | 8.0 | 3.0 | 100 |
| 3 | outlet | 800 | 17.0 | 2.0 | 150 |

## Conservation of mass

For conservation of mass, B=m and β=dmdm=1. The Reynolds transport equation becomes:

CV∂ρ∂tdV+CSρV.n dA=0

If the control volume only has a number of one-dimensional inlets and outlets, we can write:

CV∂ρ∂tdV+iρiAiViout-iρiAiViin=0

Note: for steady-state flow, ∂ρ∂t=0, and the conservation of mass becomes:

iρiAiViout=iρiAiViin

This means, in steady flow, the mass flows entering and leaving the control volume must balance.

## Average velocity

In cases that fluid velocity varies across a control surface, it is often convenient to define an average velocity.

Vav=QA=1AV.ndA

The average velocity is only a concept, i.e., when it is multiplied by the area gives the volume flow.

If the density varies across the cross-section, we similarly can define an average density:

ρav=1AρdA

Example 2

In a grinding and polishing operation, water at 300 K is supplied at a flow rate of 4.264×10-3 kg/s through a long, straight tube having an inside diameter of D=2R=6.35 mm. Assuming the flow within the tube is laminar and exhibits a parabolic velocity profile:

ur=umax1-rR2

where umax is the maximum fluid velocity at the center of the tube. Using the definition of the mass flow rate and the concept of average velocity, show that:

uavg=umax2

R

u(r)

uavg

R

r

L

r

## The linear momentum equation

For Newton’s second law, the property being differentiated is the linear momentum, mV. Thus B =mV and β=dBdm=V. The Reynolds transport theorem becomes:

ddtmVsys=F=ddtCVVρdV+CSVρV.n dA

Note that this is a vector equation and has three components.

Momentum flux term,

MCS=CSVρV.n dA

If cross-section is one-dimensional, V and ρ are uniform and over the area, momentum flux simplifies:

Mi=ρiAiVi=miVi

For one-dimensional inlets and outlets, we have:

∑F=ddtCVVρdV+imiViout-imiViin

## Net pressure force on a closed CV

Recall that the external pressure force on a surface is normal and inward.

Since the unit vector n is outward, we can write:

Fpress=CSp-ndA

If the pressure has a uniform value pa all around the surface, the net pressure force is zero.

Funiform press=-paCSndA=0

This is independent of the shape of the surface. Thus pressure force problems can be simplified by subtracting any convenient uniform pressure pa and working only with the pieces of gage pressure that remain:

Fpress=CSp-pa-ndA=CSpgage-ndA