- ENSC 283 Quiz #1

Jan. 27, 2009

Name: ……………………………………… Student ID:………………………………..

Time: 45 minutes or less. Develop answers on available place. The quiz has 5% (bonus) of the total mark. Closed books & closed notes.

Problem 1 (50%):

A square, side dimension a (m), has its top edge H (m) below the water surface. It is on angle θ and its bottom is hinged as shown in the figure below. Develop a relationship for the force F needed to just open the gate.

Hinge

Water

F

a

H

θ

Hint: start with drawing a free-body-diagram of the gate. Also:

yCP=-γsinθIxxpCGA

Ixx=a412             A=a2

Solution:

The first step is to sketch a free-body diagram of the gate so the forces and distances are clearly identified. It is done in the following figure.

F

H

θ

FR

Fx

Fy

yCP

.

CG

The force FR is calculated to be

| FR=γhCGA=γ(H+asinθ2)a2 | (Eq.1) |
| --- | --- |

We will take moments about the hinge so that it will not be necessary to calculate the forces Fx and Fy.

| FR×a2-yCP=F×a | (Eq.2) |
| --- | --- |

where, yCP is the distance between the center of pressure (CP) and the center of gravity (CG). yCP can be written as:

| yCP=γsinθIxxpCGA=IxxsinθhCGA=a412sinθ(H+asinθ2)a2=a2sinθ12(H+asinθ2) | (Eq.3) |
| --- | --- |

Substituting yCP into Eq.2, the force F is found.

| F=FR×a2-yCPa=γ(H+asinθ2)a2a2-yCPa | (Eq.4) |
| --- | --- |

Simplifying the above equation, we get:

| F=γa2(3H+asinθ)6 | (Eq.5) |
| --- | --- |

Problem 2 (50%):

It is said that Archimedes discovered the buoyancy laws when asked by King Hiero of Syracuse to determine whether his new crown was pure gold (SG = 19.3). Archimedes measured the weight of the crown in air to be 11.8 N and its weight in water to be 10.9 N. Was it pure gold?

Hint: the buoyancy is the difference between air weight and underwater weight.

FB=γV

Solution:

The buoyancy is the difference between air weight and underwater weight:

| FB=Win air-Win water=γwaterVcrown=11.8 N-10.9 N=0.9 N | (Eq.1) |
| --- | --- |

where, Win air and Win water are the weight of the crown in air and water, respectively. The weight of the crown in air can be expressed as:

| Win air=(SG)γwaterVcrown | (Eq.2) |
| --- | --- |

Substituting Eq.2 into Eq.1, we get:

| Win water=γwaterVcrownSG-1=FBSG-1 | (Eq.3) |
| --- | --- |

Thus, the specific gravity of the crown can be written as:

| SG=1+Win waterFB=1+10.90.9=13.11 (not pure gold) | (Eq.4) |
| --- | --- |