# ENSC 283 Week # 2, Tutorial # 1– Dimensionless Quantities

ρ, μ

D

V

Problem 1: A dimensionless combination of variables that is important in the study of viscous flow through pipes is called the Reynolds number, Re, defined asρVD/μ where, as indicated in the figure, ρ is the fluid density, V the mean fluid velocity, D the pipe diameter, and μ the fluid viscosity. A Newtonian fluid having a viscosity of 0.38 N.s/m2 and a specific gravity of 0.91 flows through a 25-mm-diameter pipe with a velocity of 2.6 m/s. Determine the value of the Reynolds number using (a) SI units, and (b) BG units.

Solution

# Step 1: Write out what you are required to solve for (this is so you don’t forget to answer everything the question is asking for)

Find:

- Reynolds number, Re, in SI and BG units

# Step 2: Prepare a data table

| Data | Value | ## Unit |
| --- | --- | --- |
| V | 2.6 | m/s |
| D | 25 | mm |
| μ | 0.38 | N.s/m2 |
| SG | 0.91 | - |

# Step 3: State your assumptions (you may have to add to your list of assumptions as you proceed in the problem)

Assumptions:

- Water is considered as a Newtonian fluid.

# Step 4: Calculations

- SI Units

The fluid density is calculated from the specific gravity as

| ρ=SG ρH2O@4°C=0.91 1000kgm3=910 kg/m3 | (Eq1) |
| --- | --- |
| From the definition of the Reynolds number, |  |

| Re=ρVDμ=910kgm32.6 ms25 mm(10-3mmm)0.38 N.s/m2=156 (kg.ms2)/N | (Eq2) |
| --- | --- |

However, since 1N=1 kg.m/s2 it follows that the Reynolds number is unitless, therefore

| Re=156 | (Eq3) |
| --- | --- |

- BG Units

We first convert all the SI values of the variables appearing in the Reynolds number to BG values by using the conversion factors.

Since, then the maximum electrical power generation per year become

| ρ=910kgm31.940×10-3slugsft3kgm3=1.77 slugs/ft3 | (Eq4) |
| --- | --- |
| V=2.6 ms 3.281ftm=8.53 ft/s | (Eq5) |

| D=0.025 m 3.281ftm=8.20×10-2 ft | (Eq6) |
| --- | --- |

| μ=0.38 N.s/m2 2.089lb.s/ft2N.s/m2=7.94×10-3 lb.s/ft2 | (Eq7) |
| --- | --- |

Using the definition of the Reynolds number, we get

| Re=ρVDμ=1.77slugsft38.53fts(8.20×10-2 ft)7.94×10-3 lb.s/ft2=156 | (Eq8) |
| --- | --- |

# Step 5: Concluding Statement

The value of any dimensionless quantity does not depend on the system of units used if all variables that make up the quantity are expressed in a consistent set of units.