# ENSC 283 Week # 4, Tutorial # 3– Hydrostatic Forces

Problem 1: In the figure the surface AB is a circular arc with a radius of 2 m. The distance DB is 4 m. If water is the liquid supported by the surface and if atmospheric pressure prevails on the other side of AB, determine the magnitude and line of action of the resultant hydrostatic force on AB per unit length.

Solution

# Step 1: Write out what you are required to solve for (this is so you don’t forget to answer everything the question is asking for)

Find:

- the magnitude and line of action of the hydrostatic force F on AB

# Step 2: Prepare a data table

| Data | Value | ## Unit |
| --- | --- | --- |
| DB | 4 | m |
| r | 2 | m |

# Step 3: Calculations

The vertical component is equal to the weight of water in volume AOCDB:

| WOCDB=γ AOCDB×1= 9810Nm34 m2 m1 m=78.480 kN |  | (Eq1) |
| --- | --- | --- |
| WAOB=γ AAOB×1=γ14πr2×1= 9810 Nm3×π42 m21 m=30.819 kN |  | (Eq2) |

Therefore, the vertical component is:

| FV=WOCDB+WAOB=109.299 kN | (Eq3) |
| --- | --- |

The line of action of the vertical component acts through the centroid of the volume of water considered above, and this is calculated by taking moments about the z-axis.

| FV.xCG=WOCDB. xCG,1+WAOB.xCG,2= 78.480 kN1 m+30.819 kN4×23π m=104.640 kNm | (Eq4) |
| --- | --- |

| xCG=104.640 kNm109.299 kN=0.9574 m | (Eq5) |
| --- | --- |

Note:  xCG,1 and xCG,2=4r/(3π) are the centroidal distances of OCDB and AOB with respect to the z-axis, respectively.

The magnitude of the horizontal component can be expressed as:

| FH=γhCGAproj= 9810Nm35 m2 m2=98.1 kN | (Eq6) |
| --- | --- |

The location of the line of action of the horizontal component is given by:

| yCP,proj=-IxxsinθhCGAproj=-1121 m(2 m)3sin90°5 m(2 m2)=-0.0667 m | (Eq7) |
| --- | --- |

Note that yCP,proj is calculated with respect to the center of Aproj, therefore,

| YCP,proj=-1 m-0.0667 m= -1.0667 m | (Eq8) |
| --- | --- |

where, YCP,proj is the location with respect to the x-axis.

The resultant hydrostatic force is:

| F=(FH2+FV2)1/2= (98.1 kN)2+(109.299 kN)21/2=146.867 kN | (Eq9) |
| --- | --- |

This resultant force is shown in the following figure.

It should be considered that the resultant force acts along z=-(tanθ)x.