# ENSC 283 Week # 12, Tutorial # 8 – Flow in an Irrigation System

Problem: Spray heads in an agricultural spraying system are to be supplied with water through 50 m of drawn aluminum tubing from an engine-driven pump. In its most efficient operating range, the pump output is 0.1 m3/s at a discharge pressure not exceeding 450 kPa. For satisfactory operation, the sprinklers must operate at 200 kPa or higher pressure. Minor losses and elevation changes may be neglected. Assuming the roughness of 0.01mm for the pipe, determine the smallest standard pipe size that can be used.

Solution

# Step 1: Write out what you are required to solve for (this is so you don’t forget to answer everything the question is asking for)

Find:

- D, the smallest standard pipe size

# Step 2: Prepare a data table

| Data | Value | ## Unit |
| --- | --- | --- |
| Q | 0.1 | m3/s |
| p1max | 450 | kPa |
| p2min | 200 | kPa |
| L | 50 | m |
| ϵ | 0.01 | mm |

# Step 3: State your assumptions (you may have to add to your list of assumptions as you proceed in the problem)

Assumptions:

- Steady state

- Incompressible flow

- Negligible minor losses i.e., hf=hpump

- V1=V2=V; α1=α2

# Step 4: Calculations

∆p, L, and Q are known. D is unknown, so iteration is needed to determine the minimum standard diameter that satisfies the pressure drop constraint at the given flow rate. The maximum allowable pressure drop over the length, L, is

| ∆pmax=p1max-p2min=450-200kPa=250kPa | (Eq1) |
| --- | --- |

Neglecting elevation changes and the energy equation can be expressed as:

| p1ρg+α1V122g-p2ρg+α2V222g=hf | (Eq2) |
| --- | --- |

| hpump=hf=fLDV22g | (Eq3) |
| --- | --- |

Applying the assumption (4), we get

| ∆p=p1-p2=fLDρV22 | (Eq4) |
| --- | --- |

This equation is difficult to solve for D because both V and f depend on D. The best approach is to use a computer application such as Excel to automatically solve for D. Fore completeness here we show the manual iteration procedure. The first step is to express Eq4 and the Reynolds number in terms of Q instead of V (Q is constant but V varies with D). We have V=Q/A=4Q/πD2, so that

| ∆p=fLDρ24QπD22 | (Eq5) |
| --- | --- |

The Reynolds number in terms of Q is

| Re=ρVDμ=VDν=4QπDν=40.1 m3/sπ10-6m2/s=1.273×105D | (Eq6) |
| --- | --- |

For an initial guess, take nominal pipe 4 in pipe:

| Re=1.273×1054 in1 in2.54×10-2m=1.253×106 | (Eq7) |
| --- | --- |

For the drawn tubing, ϵ=0.01mm and hence ϵD=9.84×10-5. Using the Colebrook relationship, the friction factor can be found.

| 1f=-2logϵ/D3.7+2.51Re∙f→f=0.01318 | (Eq8) |
| --- | --- |

Substituting f into Eq5, we get:

| ∆p=8fLρQ2π2D5=8×0.01318×50 m1000 kg/m30.1 m3/s2π210.16×10-2m5=493.409 kPa>∆pmax | (Eq9) |
| --- | --- |

Since ∆p>∆pmax, we should try a larger diameter pipe. Let consider D=5 in, thus

| Re=1.273×1055 in1 in2.54×10-2m=1.002×106 | (Eq10) |
| --- | --- |

For the drawn tubing with D=5 in, ϵD=7.87×10-5. Using the Colebrook relationship, we get f=0.01312. Substituting f into Eq5, we get:

| ∆p=8fLρQ2π2D5=8×0.01312×50 m1000 kg/m30.1 m3/s2π212.7×10-2m5=160.944 kPa<∆pmax | (Eq11) |
| --- | --- |

Thus, the criterion for pressure drop is satisfied for a minimum nominal diameter of 5 in pipe.