Solutions Manual

for

Introduction to Thermodynamics and Heat Transfer

Yunus A. Cengel

2nd Edition, 2008

Chapter 6

MASS AND ENERGY ANALYSIS OF CONTROL VOLUMES

# PROPRIETARY AND CONFIDENTIAL

This Manual is the proprietary property of The McGraw-Hill Companies, Inc. (“McGraw-Hill”) and protected by copyright and other state and federal laws.  By opening and using this Manual the user agrees to the following restrictions, and if the recipient does not agree to these restrictions, the Manual should be promptly returned unopened to McGraw-Hill:  This Manual is being provided only to authorized professors and instructors for use in preparing for the classes using the affiliated textbook.  No other use or distribution of this Manual is permitted.  This Manual may not be sold and may not be distributed to or used by any student or other third party.  No part of this Manual may be reproduced, displayed or distributed in any form or by any means, electronic or otherwise, without the prior written permission of McGraw-Hill.

Conservation of Mass

6-1C Mass, energy, momentum, and electric charge are conserved, and volume and entropy are not conserved during a process.

6-2C Mass flow rate is the amount of mass flowing through a cross-section per unit time whereas the volume flow rate is the amount of volume flowing through a cross-section per unit time.

6-3C The amount of mass or energy entering a control volume does not have to be equal to the amount of mass or energy leaving during an unsteady-flow process.

6-4C Flow through a control volume is steady when it involves no changes with time at any specified position.

6-5C No, a flow with the same volume flow rate at the inlet and the exit is not necessarily steady (unless the density is constant). To be steady, the mass flow rate through the device must remain constant.

6-6E A garden hose is used to fill a water bucket. The volume and mass flow rates of water, the filling time, and the discharge velocity are to be determined.

Assumptions 1 Water is an incompressible substance. 2 Flow through the hose is steady. 3 There is no waste of water by splashing.

Properties We take the density of water to be 62.4 lbm/ft3 (Table A-3E).

Analysis (a) The volume and mass flow rates of water are

V=AV=(πD24)V=[π(112 ft)24](8 ft/s)=0.04363 ft3/s

m=ρV=(62.4 lbm/ft3)(0.04363 ft3/s)=2.72 lbm/s

(b) The time it takes to fill a 20-gallon bucket is

Δt=VV=20  gal0.04363 ft3/s1 ft37.4804 gal=61.3 s

(c) The average discharge velocity of water at the nozzle exit is

Ve=VAe=VπDe24=0.04363 ft3/s[π(0.512 ft)24]=32 ft/s

Discussion Note that for a given flow rate, the average velocity is inversely proportional to the square of the velocity. Therefore, when the diameter is reduced by half, the velocity quadruples.

6-7 Air is accelerated in a nozzle. The mass flow rate and the exit area of the nozzle are to be determined.

AIR

V2 = 180 m/s

V1 = 40 m/s

A1 = 90 cm2

Assumptions Flow through the nozzle is steady.

Properties The density of air is given to be 2.21 kg/m3 at the inlet, and 0.762 kg/m3 at the exit.

Analysis (a) The mass flow rate of air is determined from the inlet conditions to be

m=ρ1A1V1=(2.21 kg/m3)(0.009  m2)(40  m/s)=0.796  kg/s

(b) There is only one inlet and one exit, and thus . Then the exit area of the nozzle is determined to be

m=ρ2A2V2  A2=mρ2V2=0.796  kg/s(0.762 kg/ m3)(180 m/s)=0.0058 m2=58 cm2

6-8E Steam flows in a pipe. The minimum diameter of the pipe for a given steam velocity is to be determined.

Assumptions Flow through the pipe is steady.

Steam, 200 psia

600F, 59 ft/s

Properties The specific volume of steam at the given state is (Table A-6E)

P=200 psia  v1=3.0586 ft3/lbm

Analysis The cross sectional area of the pipe is

m=1vAcVA=mvV=(200 lbm/s)(3.0586 ft3/lbm)59 ft/s=10.37 ft2

Solving for the pipe diameter gives

A=πD24D=4Aπ=4(10.37 ft2)π=3.63 ft

- Therefore, the diameter of the pipe must be at least 3.63 ft to ensure that the velocity does not exceed 59 ft/s.

6-9  A water pump increases water pressure. The diameters of the inlet and exit openings are given. The velocity of the water at the inlet and outlet are to be determined.

Assumptions 1 Flow through the pump is steady. 2 The specific volume remains constant.

Properties The inlet state of water is compressed liquid. We approximate it as a saturated liquid at the given temperature. Then, at 15°C and 40°C, we have (Table A-4)

Water

70 kPa

15C

700 kPa

T=15°C     v1=0.001001 m3/kg

T=40°C     v1=0.001008 m3/kg

Analysis The velocity of the water at the inlet is

V1=mv1A1=4mv1πD12=4(0.5 kg/s)(0.001001 m3/kg)π(0.01 m)2=6.37 m/s

Since the mass flow rate and the specific volume remains constant, the velocity at the pump exit is

V2=V1A1A2=V1D1D22=(6.37 m/s)0.01 m0.015 m2=2.83 m/s

Using the specific volume at 40°C, the water velocity at the inlet becomes

V1=mv1A1=4mv1πD12=4(0.5 kg/s)(0.001008 m3/kg)π(0.01 m)2=6.42 m/s

which is a 0.8% increase in velocity.

6-10 Air is expanded and is accelerated as it is heated by a hair dryer of constant diameter. The percent increase in the velocity of air as it flows through the drier is to be determined.

Assumptions Flow through the nozzle is steady.

V2

V1

Properties The density of air is given to be 1.20 kg/m3 at the inlet, and 1.05 kg/m3 at the exit.

Analysis There is only one inlet and one exit, and thus . Then,

m1=m2ρ1AV1=ρ2AV2V2V1=ρ1ρ2=1.20 kg/m31.05 kg/m3=1.14     (or, and increase of 14%)

Therefore, the air velocity increases 14% as it flows through the hair drier.

6-11 A rigid tank initially contains air at atmospheric conditions. The tank is connected to a supply line, and air is allowed to enter the tank until the density rises to a specified level. The mass of air that entered the tank is to be determined.

V1 = 1 m3

1 =1.18 kg/m3

Properties The density of air is given to be 1.18 kg/m3 at the beginning, and 7.20 kg/m3 at the end.

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary.  The mass balance for this system can be expressed as

Mass balance:

min−mout=Δmsystem   →    mi=m2−m1=ρ2V−ρ1V

Substituting,

mi=(ρ2−ρ1)V=[(7.20-1.18) kg/m3](1 m3)=6.02 kg

Therefore, 6.02 kg of mass entered the tank.

- 6-12 A smoking lounge that can accommodate 15 smokers is considered. The required minimum flow rate of air that needs to be supplied to the lounge and the diameter of the duct are to be determined.

Assumptions  Infiltration of air into the smoking lounge is negligible.

Properties The minimum fresh air requirements for a smoking lounge is given to be 30 L/s per person.

Analysis The required minimum flow rate of air that needs to be supplied to the lounge is determined directly from

Smoking Lounge

15 smokers

30 L/s person

Vair=Vair per person(No. of persons)(30 L/s⋅person)(15 persons)=450 L/s=0.45 m3/s

The volume flow rate of fresh air can be expressed as

V=VA=V(πD24)

Solving for the diameter D and substituting,

D=4VπV=4(0.45 m3/s)π(8 m/s)=0.268 m

Therefore, the diameter of the fresh air duct should be at least 26.8 cm if the velocity of air is not to exceed 8 m/s.

6-13 The minimum fresh air requirements of a residential building is specified to be 0.35 air changes per hour. The size of the fan that needs to be installed and the diameter of the duct are to be determined.

Analysis   The volume of the building and the required minimum volume flow rate of fresh air are

Vroom=(2.7 m)(200 m2)=540 m3V=Vroom×ACH=(540 m3)(0.35/h)=189 m3h=189,000 L/h=3150  L/min

House

200 m2

0.35 ACH

The volume flow rate of fresh air can be expressed as

V=VA=V(πD24)

Solving for the diameter D and substituting,

D=4VπV=4(1893600 m3/s)π(6 m/s)=0.106 m

Therefore, the diameter of the fresh air duct should be at least 10.6 cm if the velocity of air is not to exceed 6 m/s.

6-14 A cyclone separator is used to remove fine solid particles that are suspended in a gas stream. The mass flow rates at the two outlets and the amount of fly ash collected per year are to be determined.

Assumptions  Flow through the separator is steady.

Analysis Since the ash particles cannot be converted into the gas and vice-versa, the mass flow rate of ash into the control volume must equal that going out, and the mass flow rate of flue gas into the control volume must equal that going out. Hence, the mass flow rate of ash leaving is

mash=yashmin=(0.001)(10 kg/s)=0.01 kg/s

The mass flow rate of flue gas leaving the separator is then

mflue gas=min−mash=10−0.01=9.99 kg/s

The amount of fly ash collected per year is

mash=mashΔt=(0.01 kg/s)(365×24×3600 s/year)=315,400 kg/year

6-15 Air flows through an aircraft engine. The volume flow rate at the inlet and the mass flow rate at the exit are to be determined.

Assumptions 1 Air is an ideal gas. 2 The flow is steady.

Properties The gas constant of air is R = 0.287 kPam3/kgK (Table A-1).

Analysis  The inlet volume flow rate is

V1=A1V1=(1 m2)(180 m/s)=180 m3/s

The specific volume at the inlet is

v1=RT1P1=(0.287 kPa⋅m3/kg⋅K)(20+273 K)100 kPa=0.8409 m3/kg

Since the flow is steady, the mass flow rate remains constant during the flow. Then,

m=V1v1=180 m3/s0.8409 m3/kg=214.1 kg/s

6-16 A spherical hot-air balloon is considered. The time it takes to inflate the balloon is to be determined.

Assumptions 1 Air is an ideal gas.

Properties The gas constant of air is R = 0.287 kPam3/kgK (Table A-1).

Analysis  The specific volume of air entering the balloon is

v=RTP=(0.287 kPa⋅m3/kg⋅K)(35+273 K)120 kPa=0.7366 m3/kg

The mass flow rate at this entrance is

m=AcVv=πD24Vv=π(1 m)242 m/s0.7366 m3/kg=2.132 kg/s

The initial mass of the air in the balloon is

mi=Viv=πD36v=π(3 m)36(0.7366 m3/kg)=19.19 kg

Similarly, the final mass of air in the balloon is

mf=Vfv=πD36v=π(15 m)36(0.7366 m3/kg)=2399 kg

The time it takes to inflate the balloon is determined from

Δt=mf−mim=(2399−19.19) kg2.132 kg/s=1116 s=18.6 min

6-17 Water flows through the tubes of a boiler. The velocity and volume flow rate of the water at the inlet are to be determined.

Assumptions Flow through the boiler is steady.

Properties The specific volumes of water at the inlet and exit are (Tables A-6 and A-7)

6 MPa, 450C

80 m/s

Steam

7 MPa

65C

P1=7 MPa     v1=0.001017 m3/kg

P2=6 MPa     v2=0.05217 m3/kg

Analysis The cross-sectional area of the tube is

Ac=πD24=π(0.13 m)24=0.01327 m2

The mass flow rate through the tube is same at the inlet and exit. It may be determined from exit data to be

m=AcV2v2=(0.01327 m2)(80 m/s)0.05217 m3/kg=20.35 kg/s

The water velocity at the inlet is then

V1=mv1Ac=(20.35 kg/s)(0.001017 m3/kg)0.01327 m2=1.560 m/s

The volumetric flow rate at the inlet is

V1=AcV1=(0.01327 m2)(1.560 m/s)=0.0207 m3/s

6-18 Refrigerant-134a flows through a pipe. Heat is supplied to R-134a. The volume flow rates of air at the inlet and exit, the mass flow rate, and the velocity at the exit are to be determined.

R-134a

200 kPa

20C

5 m/s

Q

180 kPa

40C

Properties The specific volumes of R-134a at the inlet and exit are (Table A-13)

P1=200 kPav1=0.1142 m3/kg		P1=180 kPav2=0.1374 m3/kg

Analysis (a) (b) The volume flow rate at the inlet and the mass flow rate are

V1=AcV1=πD24V1=π(0.28 m)24(5 m/s)=0.3079 m3/sm=1v1AcV1=1v1πD24V1=10.1142 m3/kgπ(0.28 m)24(5 m/s)=2.696 kg/s

(c) Noting that mass flow rate is constant, the volume flow rate and the velocity at the exit of the pipe are determined from

V2=mv2=(2.696 kg/s)(0.1374 m3/kg)=0.3705 m3/sV2=V2Ac=0.3705 m3sπ(0.28 m)24=6.02 m/s

# Flow Work and Energy Transfer by Mass

6-19C  Energy can be transferred to or from a control volume as heat, various forms of work, and by mass.

6-20C  Flow energy or flow work is the energy needed to push a fluid into or out of a control volume. Fluids at rest do not possess any flow energy.

6-21C  Flowing fluids possess flow energy in addition to the forms of energy a fluid at rest possesses. The total energy of a fluid at rest consists of internal, kinetic, and potential energies. The total energy of a flowing fluid consists of internal, kinetic, potential, and flow energies.

6-22E Steam is leaving a pressure cooker at a specified pressure. The velocity, flow rate, the total and flow energies, and the rate of energy transfer by mass are to be determined.

Assumptions 1 The flow is steady, and the initial start-up period is disregarded. 2 The kinetic and potential energies are negligible, and thus they are not considered. 3 Saturation conditions exist within the cooker at all times so that steam leaves the cooker as a saturated vapor at 30 psia.

Properties The properties of saturated liquid water and water vapor at 30 psia are vf = 0.01700 ft3/lbm, vg = 13.749 ft3/lbm, ug = 1087.8 Btu/lbm, and hg = 1164.1 Btu/lbm (Table A-5E).

Analysis (a) Saturation conditions exist in a pressure cooker at all times after the steady operating conditions are established. Therefore, the liquid has the properties of saturated liquid and the exiting steam has the properties of saturated vapor at the operating pressure. The amount of liquid that has evaporated, the mass flow rate of the exiting steam, and the exit velocity are

H2O

Sat. vapor

P = 30 psia

Q

m=ΔVliquidvf=0.4 gal0.01700 ft3/lbm0.13368 ft31 gal=3.145 lbmm=mΔt=3.145 lbm45 min=0.0699 lbm/min=1.165×10-3 lbm/sV=mρgAc=mvgAc=(1.165×10-3 lbm/s)(13.749 ft3/lbm)0.15 in2144 in21 ft2=15.4 ft/s

(b) Noting that h = u + Pv and that the kinetic and potential energies are disregarded, the flow and total energies of the exiting steam are

eflow=Pv=h−u=1164.1−1087.8=76.3  Btu/lbmθ=h+ke+pe≃h=1164.1  Btu/lbm

Note that the kinetic energy in this case is ke = V2/2 = (15.4 ft/s)2 = 237 ft2/s2 = 0.0095 Btu/lbm, which is very small compared to enthalpy.

(c) The rate at which energy is leaving the cooker by mass is simply the product of the mass flow rate and the total energy of the exiting steam per unit mass,

Emass=mθ=(1.165×10−3 lbm/s)(1164.1  Btu/lbm)=1.356  Btu/s

Discussion The numerical value of the energy leaving the cooker with steam alone does not mean much since this value depends on the reference point selected for enthalpy (it could even be negative). The significant quantity is the difference between the enthalpies of the exiting vapor and the liquid inside  (which is hfg) since it relates directly to the amount of energy supplied to the cooker.

6-23 Air flows steadily in a pipe at a specified state. The diameter of the pipe, the rate of flow energy, and the rate of energy transport by mass are to be determined. Also, the error involved in the determination of energy transport by mass is to be determined.

300 kPa

77C

Air

25 m/s

18 kg/min

Properties The properties of air are R = 0.287 kJ/kg.K and cp = 1.008 kJ/kg.K (at 350 K from Table A-2b)

Analysis (a) The diameter is determined as follows

v=RTP=(0.287 kJ/kg.K)(77+273 K)(300 kPa)=0.3349 m3/kg

A=mvV=(1860 kg/s)(0.3349 m3/kg)25 m/s=0.004018 m2

D=4Aπ=4(0.004018 m2)π=0.0715 m

(b) The rate of flow energy is determined from

Wflow=mPv=(1860 kg/s)(300 kPa)(0.3349 m3/kg)=30.14 kW

(c) The rate of energy transport by mass is

Emass=m(h+ke)=mcpT+12V2(18/60 kg/s)(1.008 kJ/kg.K)(77+273 K)+12(25 m/s)21 kJ/kg1000 m2/s2105.94 kW

(d) If we neglect kinetic energy in the calculation of energy transport by mass

Emass=mh=mcpT=(18/60 kg/s)(1.005 kJ/kg.K)(77+273 K)=105.84 kW

Therefore, the error involved if neglect the kinetic energy is only 0.09%.

6-24E A water pump increases water pressure. The flow work required by the pump is to be determined.

Assumptions 1 Flow through the pump is steady. 2 The state of water at the pump inlet is saturated liquid. 3 The specific volume remains constant.

Properties The specific volume of saturated liquid water at 10 psia is

Water

10 psia

50 psia

v= vf@ 10 psia=0.01659 ft3/lbm  (Table A-5E)

Then the flow work relation gives

wflow=P2v2−P1v1=v(P2−P1)(0.01659 ft3/lbm)(50−10)psia1 Btu5.404 psia⋅ft30.1228 Btu/lbm

6-25 An air compressor compresses air. The flow work required by the compressor is to be determined.

Compressor

1 MPa

300°C

120 kPa

20°C

Assumptions 1 Flow through the compressor is steady. 2 Air is an ideal gas.

Properties Combining the flow work expression with the ideal gas equation of state gives

wflow=P2v2−P1v1R(T2−T1)(0.287 kJ/kg⋅K)(300−20)K80.36 kJ/kg

# Steady Flow Energy Balance: Nozzles and Diffusers

6-26C  A steady-flow system involves no changes with time anywhere within the system or at the system boundaries

6-27C  No.

6-28C  It is mostly converted to internal energy as shown by a rise in the fluid temperature.

6-29C  The kinetic energy of a fluid increases at the expense of the internal energy as evidenced by a decrease in the fluid temperature.

6-30C  Heat transfer to the fluid as it flows through a nozzle is desirable since it will probably increase the kinetic energy of the fluid. Heat transfer from the fluid will decrease the exit velocity.

6-31 Air is accelerated in a nozzle from 30 m/s to 180 m/s. The mass flow rate, the exit temperature, and the exit area of the nozzle are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Air is an ideal gas with constant specific heats. 3 Potential energy changes are negligible. 4 The device is adiabatic and thus heat transfer is negligible. 5 There are no work interactions.

AIR

P2 = 100 kPa

V2 = 180 m/s

P1 = 300 kPa

T1 = 200C

V1 = 30 m/s

A1 = 80 cm2

Properties The gas constant of air is 0.287 kPa.m3/kg.K (Table A-1). The specific heat of air at the anticipated average temperature of 450 K is cp = 1.02 kJ/kg.C (Table A-2).

Analysis (a) There is only one inlet and one exit, and thus . Using the ideal gas relation, the specific volume and the mass flow rate of air are determined to be

v1=RT1P1=(0.287  kPa⋅m3/kg⋅K)(473  K)300  kPa=0.4525  m3/kg

m=1v1A1V1=10.4525  m3/kg(0.008  m2)(30  m/s)=0.5304  kg/s

(b) We take nozzle as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=m(h2+V22/2)   (since  {Q≃W≃Δpe≃0)0=h2−h1+V22−V122  0=cp,aveT2−T1+V22−V122

Substituting, 	0=(1.02  kJ/kg⋅K)(T2−200C)+(180  m/s)2−(30  m/s)221  kJ/kg1000  m2/s2

It yields		T2 = 184.6C

(c)  The specific volume of air at the nozzle exit is

v2=RT2P2=(0.287  kPa⋅m3/kg⋅K)(184.6+273  K)100  kPa=1.313  m3/kg

m=1v2A2V2  0.5304  kg/s=11.313  m3/kgA2180  m/s → A2 = 0.00387 m2 = 38.7 cm2

6-32 EES Problem 6-31 is reconsidered. The effect of the inlet area on the mass flow rate, exit velocity, and the exit area as the inlet area varies from 50 cm2 to 150 cm2 is to be investigated, and the final results are to be plotted against the inlet area.

Analysis The problem is solved using EES, and the solution is given below.

Function HCal(WorkFluid$, Tx, Px)

"Function to calculate the enthalpy of an ideal gas or real gas"

If   'Air' = WorkFluid$  then

HCal:=ENTHALPY('Air',T=Tx) "Ideal gas equ."

else

HCal:=ENTHALPY(WorkFluid$,T=Tx, P=Px)"Real gas equ."

endif

end  HCal

"System: control volume for the nozzle"

"Property relation: Air is an ideal gas"

"Process: Steady state, steady flow, adiabatic, no work"

"Knowns - obtain from the input diagram"

WorkFluid$ = 'Air'

T[1] = 200  [C]

P[1] = 300   [kPa]

Vel[1] = 30  [m/s]

P[2] = 100  [kPa]

Vel[2] = 180  [m/s]

A[1]=80 [cm^2]

Am[1]=A[1]*convert(cm^2,m^2)

"Property Data - since the Enthalpy function has different parameters

for ideal gas and real fluids, a function was used to determine h."

h[1]=HCal(WorkFluid$,T[1],P[1])

h[2]=HCal(WorkFluid$,T[2],P[2])

"The Volume function has the same form for an ideal gas as for a real fluid."

v[1]=volume(workFluid$,T=T[1],p=P[1])

v[2]=volume(WorkFluid$,T=T[2],p=P[2])

"Conservation of mass: "

m_dot[1]= m_dot[2]

"Mass flow rate"

m_dot[1]=Am[1]*Vel[1]/v[1]

m_dot[2]= Am[2]*Vel[2]/v[2]

"Conservation of Energy - SSSF energy balance"

h[1]+Vel[1]^2/(2*1000) = h[2]+Vel[2]^2/(2*1000)

"Definition"

A_ratio=A[1]/A[2]

A[2]=Am[2]*convert(m^2,cm^2)

| A1 [cm2] | A2 [cm2] | m1 | T2 |
| --- | --- | --- | --- |
| 50 | 24.19 | 0.3314 | 184.6 |
| 60 | 29.02 | 0.3976 | 184.6 |
| 70 | 33.86 | 0.4639 | 184.6 |
| 80 | 38.7 | 0.5302 | 184.6 |
| 90 | 43.53 | 0.5964 | 184.6 |
| 100 | 48.37 | 0.6627 | 184.6 |
| 110 | 53.21 | 0.729 | 184.6 |
| 120 | 58.04 | 0.7952 | 184.6 |
| 130 | 62.88 | 0.8615 | 184.6 |
| 140 | 67.72 | 0.9278 | 184.6 |
| 150 | 72.56 | 0.9941 | 184.6 |

6-33E Air is accelerated in an adiabatic nozzle. The velocity at the exit is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Air is an ideal gas with constant specific heats. 3 Potential energy changes are negligible. 4 There are no work interactions. 5 The nozzle is adiabatic.

Properties The specific heat of air at the average temperature of (700+645)/2=672.5°F is cp = 0.253 Btu/lbmR (Table A-2Eb).

Analysis There is only one inlet and one exit, and thus m1=m2=m. We take nozzle as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

AIR

300 psia

700F

80 ft/s

250 psia

645F

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=m(h2+V22/2)h1+V122=h2+V22/2

Solving for exit velocity,

V2=V12+2(h1−h2)0.5=V12+2cp(T1−T2)0.5(80 ft/s)2+2(0.253 Btu/lbm⋅R)(700−645)R25,037  ft2/s21 Btu/lbm0.5838.6 ft/s

6-34 Air is decelerated in an adiabatic diffuser. The velocity at the exit is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Air is an ideal gas with constant specific heats. 3 Potential energy changes are negligible. 4 There are no work interactions. 5 The diffuser is adiabatic.

Properties The specific heat of air at the average temperature of (20+90)/2=55°C =328 K is cp = 1.007 kJ/kgK (Table A-2b).

Analysis There is only one inlet and one exit, and thus m1=m2=m. We take diffuser as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

AIR

100 kPa

20C

500 m/s

200 kPa

90C

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=m(h2+V22/2)h1+V122=h2+V22/2

Solving for exit velocity,

V2=V12+2(h1−h2)0.5=V12+2cp(T1−T2)0.5(500 m/s)2+2(1.007 kJ/kg⋅K)(20−90)K1000 m2/s21 kJ/kg0.5330.2 m/s

6-35 Steam is accelerated in a nozzle from a velocity of 80 m/s. The mass flow rate, the exit velocity, and the exit area of the nozzle are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy changes are negligible. 3 There are no work interactions.

Steam

1

2

120 kJ/s

Properties From the steam tables (Table A-6)

P1=5  MPa  v1=0.057838  m3/kgh1=3196.7  kJ/kg

and

P2=2  MPa  v2=0.12551  m3/kgh2=3024.2  kJ/kg

Analysis (a)  There is only one inlet and one exit, and thus . The mass flow rate of steam is

m=1v1V1A1=10.057838  m3/kg(80  m/s)(50×10−4 m2)=6.92  kg/s

(b) We take nozzle as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=Qout+m(h2+V22/2)   (since  {W≃Δpe≃0)−Qout=mh2−h1+V22−V122

Substituting, the exit velocity of the steam is determined to be

−120 kJ/s=6.916  kg/s3024.2−3196.7+V22−(80  m/s)221  kJ/kg1000  m2/s2

It yields				V2 = 562.7 m/s

(c) The exit area of the nozzle is determined from

m=1v2V2A2  A2=mv2V2=6.916  kg/s0.12551  m3/kg562.7  m/s=15.42×10−4 m2

6-36 CD EES Steam is accelerated in a nozzle from a velocity of 40 m/s to 300 m/s. The exit temperature and the ratio of the inlet-to-exit area of the nozzle are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy changes are negligible. 3 There are no work interactions. 4 The device is adiabatic and thus heat transfer is negligible.

Properties From the steam tables (Table A-6),

P1=3  MPa  v1=0.09938  m3/kgh1=3231.7  kJ/kg

Analysis  (a) There is only one inlet and one exit, and thus . We take nozzle as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Steam

P2 = 2.5 MPa

V2 = 300 m/s

P1 = 3 MPa

T1 = 400C

V1 = 40 m/s

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=m(h2+V22/2)   (since  {Q≃W≃Δpe≃0)0=h2−h1+V22−V122

or,

h2=h1−V22−V122=3231.7  kJ/kg−(300  m/s)2−(40  m/s)221 kJ/kg1000  m2/s2=3187.5  kJ/kg

Thus,	P2=2.5  MPa T2=376.6°Cv2=0.11533  m3/kg

(b)  The ratio of the inlet to exit area is determined from the conservation of mass relation,

1v2A2V2=1v1A1V1  A1A2=v1v2V2V1=(0.09938  m3/kg)(300  m/s)(0.11533  m3/kg)(40  m/s)=6.46

6-37 Air is decelerated in a diffuser from 230 m/s to 30 m/s. The exit temperature of air and the exit area of the diffuser are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Air is an ideal gas with variable specific heats. 3 Potential energy changes are negligible. 4 The device is adiabatic and thus heat transfer is negligible. 5 There are no work interactions.

Properties The gas constant of air is 0.287 kPa.m3/kg.K (Table A-1). The enthalpy of air at the inlet temperature of 400 K is h1 = 400.98 kJ/kg (Table A-21).

Analysis  (a) There is only one inlet and one exit, and thus . We take diffuser as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

AIR

1

2

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=m(h2+V22/2)   (since  {Q≃W≃Δpe≃0)0=h2−h1+V22−V122,

or,

h2=h1−V22−V122=400.98  kJ/kg−30  m/s2−230  m/s221  kJ/kg1000  m2/s2=426.98  kJ/kg

From Table A-21,	T2 = 425.6 K

(b)  The specific volume of air at the diffuser exit is

v2=RT2P2=0.287  kPa⋅m3/kg⋅K425.6  K100  kPa=1.221  m3/kg

From conservation of mass,

m=1v2A2V2  A2=mv2V2=(6000/3600  kg/s)(1.221  m3/kg)30  m/s=0.0678   m2

6-38E Air is decelerated in a diffuser from 600 ft/s to a low velocity. The exit temperature and the exit velocity of air are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Air is an ideal gas with variable specific heats. 3 Potential energy changes are negligible. 4 The device is adiabatic and thus heat transfer is negligible. 5 There are no work interactions.

Properties The enthalpy of air at the inlet temperature of 20F is h1 = 114.69 Btu/lbm (Table A-21E).

Analysis  (a) There is only one inlet and one exit, and thus . We take diffuser as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

AIR

1

2

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=m(h2+V22/2)   (since  {Q≃W≃Δpe≃0)0=h2−h1+V22−V122,

or,

h2=h1−V22−V122=114.69  Btu/lbm−0−600  ft/s221  Btu/lbm25,037  ft2/s2=121.88  Btu/lbm

From Table A-21E,		T2 = 510.0 R

(b)  The exit velocity of air is determined from the conservation of mass relation,

1v2A2V2=1v1A1V1  1RT2P2A2V2=1RT1P1A1V1

Thus,

V2=A1T2P1A2T1P2V1=15(510  R)(13  psia)(480  R)(14.5  psia)(600  ft/s)=114.3  ft/s

6-39 CO2 gas is accelerated in a nozzle to 450 m/s. The inlet velocity and the exit temperature are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 CO2 is an ideal gas with variable specific heats. 3 Potential energy changes are negligible. 4 The device is adiabatic and thus heat transfer is negligible. 5 There are no work interactions.

Properties The gas constant and molar mass of CO2 are 0.1889 kPa.m3/kg.K and 44 kg/kmol (Table A-1). The enthalpy of CO2 at 500C is 30,797 kJ/kmol (from CO2 ideal gas tables -not available in this text).

Analysis (a) There is only one inlet and one exit, and thus . Using the ideal gas relation, the specific volume is determined to be

CO2

1

2

v1=RT1P1=0.1889  kPa⋅m3/kg⋅K773  K1000  kPa=0.146  m3/kg

Thus,

m=1v1A1V1  V1=mv1A1=6000/3600  kg/s0.146  m3/kg40×10−4 m2=60.8 m/s

(b)  We take nozzle as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=m(h2+V22/2)   (since  {Q≃W≃Δpe≃0)0=h2−h1+V22−V122

Substituting,

h2=h1−V22−V122M30,797  kJ/kmol−450  m/s2−60.8  m/s221  kJ/kg1000  m2/s244  kg/kmol26,423  kJ/kmol

Then the exit temperature of CO2 from the ideal gas tables is obtained to be      T2 = 685.8 K

Alternative Solution Using constant specific heats for CO2 at an anticipated average temperature of 700 K (Table A-2b), from the energy balance we obtain

0=cp(T2−T1)+V22−V1220=(1.126 kJ/kg⋅K)(T2−500)°C+450  m/s2−60.8  m/s221  kJ/kg1000  m2/s2T2=411.7°C=684.7 K

The result is practically identical to the result obtained earlier.

6-40 R-134a is accelerated in a nozzle from a velocity of 20 m/s. The exit velocity of the refrigerant and the ratio of the inlet-to-exit area of the nozzle are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy changes are negligible. 3 There are no work interactions. 4 The device is adiabatic and thus heat transfer is negligible.

Properties From the refrigerant tables (Table A-13)

R-134a

1

2

P1=700  kPa  v1=0.043358  m3/kgh1=358.90  kJ/kg

and

P2=400  kPa  v2=0.056796  m3/kgh2=275.07  kJ/kg

Analysis (a) There is only one inlet and one exit, and thus . We take nozzle as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=m(h2+V22/2)   (since  {Q≃W≃Δpe≃0)0=h2−h1+V22−V122

Substituting,

0=275.07−358.90 kJ/kg+V22−20  m/s221  kJ/kg1000  m2/s2

It yields		 V2 = 409.9 m/s

(b)  The ratio of the inlet to exit area is determined from the conservation of mass relation,

1v2A2V2=1v1A1V1  A1A2=v1v2V2V1=0.043358  m3/kg409.9  m/s0.056796  m3/kg20  m/s=15.65

6-41 Nitrogen is decelerated in a diffuser from 200 m/s to a lower velocity. The exit velocity of nitrogen and the ratio of the inlet-to-exit area are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Nitrogen is an ideal gas with variable specific heats. 3 Potential energy changes are negligible. 4 The device is adiabatic and thus heat transfer is negligible. 5 There are no work interactions.

Properties The molar mass of nitrogen is M = 28 kg/kmol (Table A-1). The enthalpies are (from nitrogen ideal gas tables-not available in this text)

T1=7°C=280 K   →    {h1=8141 kJ/kmolT2=22°C=295 K  →    {h2=8580 kJ/kmol

Analysis  (a) There is only one inlet and one exit, and thus . We take diffuser as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

N2

1

2

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=m(h2+V22/2)   (since  {Q≃W≃Δpe≃0)0=h2−h1+V22−V122=h2−h1M+V22−V122,

Substituting,

0=8580−8141 kJ/kmol28 kg/kmol+V22−200  m/s221  kJ/kg1000  m2/s2

It yields				       V2 = 93.0 m/s

(b)  The ratio of the inlet to exit area is determined from the conservation of mass relation,

or,	1v2A2V2=1v1A1V1  A1A2=v1v2V2V1=RT1P1RT2P2V2V1A1A2=T1P1T2P2V2V1=280  K/60  kPa93.0  m/s295  K/85  kPa200  m/s=0.625

Alternative Solution Using constant specific heats for N2 at room temperature (Table A-2a), from the energy balance we obtain

0=cp(T2−T1)+V22−V1220=(1.039 kJ/kg⋅K)(22−7)°C+V22−200  m/s221  kJ/kg1000  m2/s2V2=94.0 m/s

The result is practically identical to the result obtained earlier.

6-42 EES Problem 6-41 is reconsidered. The effect of the inlet velocity on the exit velocity and the ratio of the inlet-to-exit area as the inlet velocity varies from 180 m/s to 260 m/s is to be investigated. The final results are to be plotted against the inlet velocity.

Analysis The problem is solved using EES, and the solution is given below.

Function HCal(WorkFluid$, Tx, Px)

"Function to calculate the enthalpy of an ideal gas or real gas"

If   'N2' = WorkFluid$  then

HCal:=ENTHALPY(WorkFluid$,T=Tx) "Ideal gas equ."

else

HCal:=ENTHALPY(WorkFluid$,T=Tx, P=Px)"Real gas equ."

endif

end  HCal

"System: control volume for the nozzle"

"Property relation: Nitrogen is an ideal gas"

"Process: Steady state, steady flow, adiabatic, no work"

"Knowns"

WorkFluid$ = 'N2'

T[1] = 7  [C]

P[1] = 60   [kPa]

{Vel[1] = 200  [m/s]}

P[2] = 85  [kPa]

T[2] = 22 [C]

"Property Data - since the Enthalpy function has different parameters

for ideal gas and real fluids, a function was used to determine h."

h[1]=HCal(WorkFluid$,T[1],P[1])

h[2]=HCal(WorkFluid$,T[2],P[2])

"The Volume function has the same form for an ideal gas as for a real fluid."

v[1]=volume(workFluid$,T=T[1],p=P[1])

v[2]=volume(WorkFluid$,T=T[2],p=P[2])

"From the definition of mass flow rate, m_dot = A*Vel/v and conservation of mass the area ratio A_Ratio = A_1/A_2 is:"

A_Ratio*Vel[1]/v[1] =Vel[2]/v[2]

"Conservation of Energy - SSSF energy balance"

h[1]+Vel[1]^2/(2*1000) = h[2]+Vel[2]^2/(2*1000)

| ARatio | Vel1 [m/s] | Vel2 [m/s] |
| --- | --- | --- |
| 0.2603 | 180 | 34.84 |
| 0.4961 | 190 | 70.1 |
| 0.6312 | 200 | 93.88 |
| 0.7276 | 210 | 113.6 |
| 0.8019 | 220 | 131.2 |
| 0.8615 | 230 | 147.4 |
| 0.9106 | 240 | 162.5 |
| 0.9518 | 250 | 177 |
| 0.9869 | 260 | 190.8 |

6-43 R-134a is decelerated in a diffuser from a velocity of 120 m/s. The exit velocity of R-134a and the mass flow rate of the R-134a are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy changes are negligible. 3 There are no work interactions.

Properties From the R-134a tables (Tables A-11 through A-13)

R-134a

1

2

2 kJ/s

P1=800  kPa  v1=0.025621 m3/kgh1=267.29  kJ/kg

and

P2=900  kPa  v2=0.023375  m3/kgh2=274.17  kJ/kg

Analysis (a)  There is only one inlet and one exit, and thus . Then the exit velocity of R-134a is determined from the steady-flow mass balance to be

1v2A2V2=1v1A1V1  V2=v2v1A1A2V1=11.8(0.023375  m3/kg)(0.025621  m3/kg)120  m/s=60.8  m/s

(b) We take diffuser as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

Qin+m(h1+V122)=m(h2+V22/2)   (since  {W≃Δpe≃0)Qin=mh2−h1+V22−V122

Substituting, the mass flow rate of the refrigerant is determined to be

2  kJ/s=m(274.17−267.29)kJ/kg+60.8  m/s2−(120  m/s)221  kJ/kg1000  m2/s2

It yields			m=1.308  kg/s

6-44 Heat is lost from the steam flowing in a nozzle. The velocity and the volume flow rate at the nozzle exit are to be determined.

400C

800 kPa

10 m/s

STEAM

Q

300C

200 kPa

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy change is negligible. 3 There are no work interactions.

Analysis We take the steam as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Energy balance:

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutmh1+V122=mh2+V222+Qout     since   {W≃Δpe≃0)

or	                h1+V122=h2+V222+Qoutm

The properties of steam at the inlet and exit are (Table A-6)

P1=800  kPa  v1=0.38429 m3/kgh1=3267.7 kJ/kg

P2=200  kPa  v2=1.31623 m3/kgh2=3072.1 kJ/kg

The mass flow rate of the steam is

m=1v1A1V1=10.38429 m3/s(0.08 m2)(10 m/s)=2.082 kg/s

Substituting,

3267.7 kJ/kg+(10 m/s)221 kJ/kg1000 m2/s2=3072.1 kJ/kg+V2221 kJ/kg1000 m2/s2+25 kJ/s2.082 kg/sV2=606 m/s

The volume flow rate at the exit of the nozzle is

V2=mv2=(2.082 kg/s)(1.31623 m3/kg)=2.74 m3/s

# Turbines and Compressors

6-45C  Yes.

6-46C  The volume flow rate at the compressor inlet will be greater than that at the compressor exit.

6-47C  Yes. Because energy (in the form of shaft work) is being added to the air.

6-48C  No.

6-49 Air is expanded in a turbine. The mass flow rate and outlet area are to be determined.

Assumptions 1 Air is an ideal gas. 2 The flow is steady.

Turbine

1 MPa

600°C

30 m/s

100 kPa

200°C

10 m/s

Properties The gas constant of air is R = 0.287 kPam3/kgK (Table A-1).

Analysis  The specific volumes of air at the inlet and outlet are

v1=RT1P1=(0.287 kPa⋅m3/kg⋅K)(600+273 K)1000 kPa=0.2506 m3/kg

v2=RT2P2=(0.287 kPa⋅m3/kg⋅K)(200+273 K)100 kPa=1.3575 m3/kg

The mass flow rate is

m=A1V1v1=(0.1 m2)(30 m/s)0.2506 m3/kg=11.97 kg/s

The outlet area is

A2=mv2V2=(11.97 kg/s)(1.3575 m3/kg)10 m/s=1.605 m2

6-50E Air is expanded in a gas turbine. The inlet and outlet mass flow rates are to be determined.

Assumptions 1 Air is an ideal gas. 2 The flow is steady.

Properties The gas constant of air is R = 0.3704 psiaft3/lbmR (Table A-1E).

Turbine

150 psia

700°F

5 lbm/s

15 psia

100°F

Analysis  The specific volumes of air at the inlet and outlet are

v1=RT1P1=(0.3704 psia⋅ft3/lbm⋅R)(700+460 R)150 psia=2.864 ft3/lbm

v2=RT2P2=(0.3704 psia⋅ft3/lbm⋅R)(100+460 R)15 psia=13.83 ft3/lbm

The volume flow rates at the inlet and exit are then

V1=mv1=(5 lbm/s)(2.864 ft3/lbm)=14.32 ft3/s

V2=mv2=(5 lbm/s)(13.83 ft3/lbm)=69.15 ft3/s

6-51 Air is compressed at a rate of 10 L/s by a compressor. The work required per unit mass and the power required are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible.  3 Air is an ideal gas with constant specific heats.

Properties The constant pressure specific heat of air at the average temperature of (20+300)/2=160°C=433 K is cp = 1.018 kJ/kg·K (Table A-2b). The gas constant of air is R = 0.287 kPam3/kgK (Table A-1).

Analysis (a) There is only one inlet and one exit, and thus m1=m2=m. We take the compressor as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Compressor

1 MPa

300°C

120 kPa

20°C

10 L/s

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

Win+mh1=mh2   (since Δke≃Δpe≃0)Win=m(h2−h1)=mcp(T2−T1)

Thus,

win=cp(T2−T1)=(1.018 kJ/kg⋅K)(300−20)K=285.0 kJ/kg

(b) The specific volume of air at the inlet and the mass flow rate are

v1=RT1P1=(0.287 kPa⋅m3/kg⋅K)(20+273 K)120 kPa=0.7008 m3/kg

m=V1v1=0.010 m3/s0.7008 m3/kg=0.01427 kg/s

Then the power input is determined from the energy balance equation to be

Win=mcp(T2−T1)=(0.01427 kg/s)(1.018 kJ/kg⋅K)(300−20)K=4.068 kW

6-52 Steam expands in a turbine. The change in kinetic energy, the power output, and the turbine inlet area are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible.

Properties From the steam tables (Tables A-4 through 6)

STEAM

m = 12 kg/s

·

P1 = 10 MPa

T1 = 450C

V1 = 80 m/s

P2 = 10 kPa

x2 = 0.92

V2 = 50 m/s

W

·

P1=10  MPa  v1=0.029782  m3/kgh1=3242.4  kJ/kg

and

P2=10  kPa  h2=hf+x2hfg=191.81+0.92×2392.1=2392.5  kJ/kg

Analysis (a) The change in kinetic energy is determined from

Δke=V22−V122=50  m/s2−(80  m/s)221  kJ/kg1000  m2/s2=−1.95 kJ/kg

(b) There is only one inlet and one exit, and thus . We take the turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=Wout+m(h2+V22/2)   (since  {Q≃Δpe≃0)Wout=−mh2−h1+V22−V122

Then the power output of the turbine is determined by substitution to be

Wout=−(12  kg/s)(2392.5−3242.4−1.95) kJ/kg=10.2  MW

(c)  The inlet area of the turbine is determined from the mass flow rate relation,

m=1v1A1V1  A1=mv1V1=(12  kg/s)(0.029782  m3/kg)80  m/s=0.00447   m2

6-53 EES Problem 6-52 is reconsidered. The effect of the turbine exit pressure on the power output of the turbine as the exit pressure varies from 10 kPa to 200 kPa is to be investigated. The power output is to be plotted against the exit pressure.

Analysis The problem is solved using EES, and the solution is given below.

"Knowns "

T[1] = 450 [C]

P[1] = 10000  [kPa]

Vel[1] = 80  [m/s]

P[2] = 10  [kPa]

X_2=0.92

Vel[2] = 50  [m/s]

m_dot[1]=12 [kg/s]

Fluid$='Steam_IAPWS'

"Property Data"

h[1]=enthalpy(Fluid$,T=T[1],P=P[1])

h[2]=enthalpy(Fluid$,P=P[2],x=x_2)

T[2]=temperature(Fluid$,P=P[2],x=x_2)

v[1]=volume(Fluid$,T=T[1],p=P[1])

v[2]=volume(Fluid$,P=P[2],x=x_2)

"Conservation of mass: "

m_dot[1]= m_dot[2]

"Mass flow rate"

m_dot[1]=A[1]*Vel[1]/v[1]

m_dot[2]= A[2]*Vel[2]/v[2]

"Conservation of Energy - Steady Flow energy balance"

m_dot[1]*(h[1]+Vel[1]^2/2*Convert(m^2/s^2, kJ/kg)) = m_dot[2]*(h[2]+Vel[2]^2/2*Convert(m^2/s^2, kJ/kg))+W_dot_turb*convert(MW,kJ/s)

DELTAke=Vel[2]^2/2*Convert(m^2/s^2, kJ/kg)-Vel[1]^2/2*Convert(m^2/s^2, kJ/kg)

| P2 [kPa] | Wturb [MW] | T2 [C] |
| --- | --- | --- |
| 10 | 10.22 | 45.81 |
| 31.11 | 9.66 | 69.93 |
| 52.22 | 9.377 | 82.4 |
| 73.33 | 9.183 | 91.16 |
| 94.44 | 9.033 | 98.02 |
| 115.6 | 8.912 | 103.7 |
| 136.7 | 8.809 | 108.6 |
| 157.8 | 8.719 | 112.9 |
| 178.9 | 8.641 | 116.7 |
| 200 | 8.57 | 120.2 |

6-54 Steam expands in a turbine. The mass flow rate of steam for a power output of 5 MW is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible.

Properties From the steam tables (Tables A-4 through 6)

H2O

1

2

P1=10  MPa  h1=3375.1  kJ/kg

P2=10  kPa  h2=hf+x2hfg=191.81+0.90×2392.1=2344.7  kJ/kg

Analysis There is only one inlet and one exit, and thus . We take the turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

Substituting, the required mass flow rate of the steam is determined to be

5000  kJ/s=−m(2344.7−3375.1) kJ/kgm=4.852  kg/s

6-55E Steam expands in a turbine. The rate of heat loss from the steam for a power output of 4 MW is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible.

H2O

1

2

Properties From the steam tables (Tables A-4E through 6E)

P1=1000  psia  h1=1448.6  Btu/lbm

P2=5  psia  h2=1130.7  Btu/lbm

Analysis There is only one inlet and one exit, and thus . We take the turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

Substituting,

Qout=−(45000/3600  lbm/s)(1130.7−1448.6) Btu/lbm−4000 kJ/s1  Btu1.055  kJ=182.0  Btu/s

6-56 Steam expands in a turbine. The exit temperature of the steam for a power output of 2 MW is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible.

Properties From the steam tables (Tables A-4 through 6)

P1=8  MPa  h1=3399.5  kJ/kg

Analysis  There is only one inlet and one exit, and thus . We take the turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

H2O

1

2

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

Substituting,

2500  kJ/s=3  kg/s3399.5−h2 kJ/kgh2=2566.2  kJ/kg

Then the exit temperature becomes

P2=20  kPa T2=60.1°C

6-57 Argon gas expands in a turbine. The exit temperature of the argon for a power output of 250 kW is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible. 4 Argon is an ideal gas with constant specific heats.

Properties The gas constant of Ar is R = 0.2081 kPa.m3/kg.K. The constant pressure specific heat of Ar is cp = 0.5203 kJ/kg·C (Table A-2a)

Analysis There is only one inlet and one exit, and thus . The inlet specific volume of argon and its mass flow rate are

ARGON

A1 = 60 cm2

P1 = 900 kPa

T1 = 450C

V1 = 80 m/s

P2 = 150 kPa

V2 = 150 m/s

250 kW

v1=RT1P1=0.2081  kPa⋅m3/kg⋅K723  K900  kPa=0.167  m3/kg

Thus,

m=1v1A1V1=10.167 m3/kg0.006  m280  m/s=2.874  kgs

We take the turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

m(h1+V122)=Wout+m(h2+V22/2)   (since  {Q≃Δpe≃0)Wout=−mh2−h1+V22−V122

Substituting,

250  kJ/s=−(2.874  kg/s)(0.5203  kJ/kg⋅C)(T2−450C)+(150  m/s)2−(80  m/s)221 kJ/kg1000  m2/s2

It yields

T2 = 267.3C

6-58 Helium is compressed by a compressor. For a mass flow rate of 90 kg/min, the power input required is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible.  3 Helium is an ideal gas with constant specific heats.

Properties The constant pressure specific heat of helium is cp = 5.1926 kJ/kg·K (Table A-2a).

He

90 kg/min

·

P2 = 700 kPa

T2 = 430 K

P1 = 120 kPa

T1 = 310 K

W

·

Q

Analysis There is only one inlet and one exit, and thus . We take the compressor as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

Win+mh1=Qout+mh2   (since Δke≃Δpe≃0)Win−Qout=m(h2−h1)=mcp(T2−T1)

Thus,

Win=Qout+mcpT2−T1(90/6 0  kg/s)(20  kJ/kg)+(90/60  kg/s)(5.1926  kJ/kg⋅K)(430−310)K965 kW

6-59 CO2 is compressed by a compressor. The volume flow rate of CO2 at the compressor inlet and the power input to the compressor are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible.  3 Helium is an ideal gas with variable specific heats. 4 The device is adiabatic and thus heat transfer is negligible.

Properties The gas constant of CO2 is R = 0.1889 kPa.m3/kg.K, and its molar mass is M = 44 kg/kmol (Table A-1). The inlet and exit enthalpies of CO2 are (from CO2 ideal gas tables –not available in this text)

CO2

2

1

Analysis (a) There is only one inlet and one exit, and thus . The inlet specific volume of air and its volume flow rate are

v1=RT1P1=0.1889  kPa⋅m3/kg⋅K300  K100  kPa=0.5667  m3/kg

V=mv1=(0.5  kg/s)(0.5667  m3/kg)=0.283 m3/s

(b) We take the compressor as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

Substituting

Win=0.5  kg/s15,483−9,431  kJ/kmol44  kg/kmol=68.8 kW

Alternative Solution Using constant specific heats for CO2 at room temperature (Table A-2a), from the energy balance we obtain

Win=mcp(T2−T1)=0.5  kg/s(1.039 kJ/kg⋅K)(450−300)K=63.5 kW

The result is close to the result obtained earlier.

6-60 Air is expanded in an adiabatic turbine. The mass flow rate of the air and the power produced are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 The turbine is well-insulated, and thus there is no heat transfer.  3 Air is an ideal gas with constant specific heats.

Properties The constant pressure specific heat of air at the average temperature of (500+150)/2=325°C=598 K is cp = 1.051 kJ/kg·K (Table A-2b). The gas constant of air is R = 0.287 kPam3/kgK (Table A-1).

Analysis (a) There is only one inlet and one exit, and thus m1=m2=m. We take the turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Turbine

1 MPa

500°C

40 m/s

100 kPa

150°C

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

mh1+V122=mh2+V222+WoutWout=mh1−h2+V12−V222=mcp(T1−T2)+V12−V222

The specific volume of air at the inlet and the mass flow rate are

v1=RT1P1=(0.287 kPa⋅m3/kg⋅K)(500+273 K)1000 kPa=0.2219 m3/kg

m=A1V1v1=(0.2 m2)(40 m/s)0.2219 m3/kg=36.06 kg/s

Similarly at the outlet,

v2=RT2P2=(0.287 kPa⋅m3/kg⋅K)(150+273 K)100 kPa=1.214 m3/kg

V2=mv2A2=(36.06 kg/s)(1.214 m3/kg)1 m2=43.78 m/s

(b) Substituting into the energy balance equation gives

Wout=mcp(T1−T2)+V12−V222(36.06 kg/s)(1.051 kJ/kg⋅K)(500−150)K+(40 m/s)2−(43.78 m/s)221 kJ/kg1000 m2/s213,260 kW

6-61 Air is compressed in an adiabatic compressor. The mass flow rate of the air and the power input are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 The compressor is adiabatic.  3 Air is an ideal gas with constant specific heats.

Properties The constant pressure specific heat of air at the average temperature of (20+400)/2=210°C=483 K is cp = 1.026 kJ/kg·K (Table A-2b). The gas constant of air is R = 0.287 kPam3/kgK (Table A-1).

Analysis (a) There is only one inlet and one exit, and thus m1=m2=m. We take the compressor as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Compressor

1.8 MPa

400°C

100 kPa

20°C

30 m/s

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

mh1+V122+Win=mh2+V222Win=mh2−h1+V22−V122=mcp(T2−T1)+V22−V122

The specific volume of air at the inlet and the mass flow rate are

v1=RT1P1=(0.287 kPa⋅m3/kg⋅K)(20+273 K)100 kPa=0.8409 m3/kg

m=A1V1v1=(0.15 m2)(30 m/s)0.8409 m3/kg=5.351 kg/s

Similarly at the outlet,

v2=RT2P2=(0.287 kPa⋅m3/kg⋅K)(400+273 K)1800 kPa=0.1073 m3/kg

V2=mv2A2=(5.351 kg/s)(0.1073 m3/kg)0.08 m2=7.177 m/s

(b) Substituting into the energy balance equation gives

Win=mcp(T2−T1)+V22−V122(5.351 kg/s)(1.026 kJ/kg⋅K)(400−20)K+(7.177 m/s)2−(30 m/s)221 kJ/kg1000 m2/s22084 kW

6-62E Air is expanded in an adiabatic turbine. The mass flow rate of the air and the power produced are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 The turbine is well-insulated, and thus there is no heat transfer.  3 Air is an ideal gas with constant specific heats.

Properties The constant pressure specific heat of air at the average temperature of (800+250)/2=525°F is cp = 0.2485 Btu/lbm·R (Table A-2Eb). The gas constant of air is R = 0.3704 psiaft3/lbmR (Table A-1E).

Analysis There is only one inlet and one exit, and thus m1=m2=m. We take the turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Turbine

500 psia

800°F

60 psia

250°F

50 ft3/s

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

mh1+V122=mh2+V222+WoutWout=mh1−h2+V12−V222=mcp(T1−T2)+V12−V222

The specific volume of air at the exit and the mass flow rate are

v2=RT2P2=(0.3704 psia⋅ft3/lbm⋅R)(250+460 R)60 psia=4.383 ft3/lbm

m=V2v2=50 ft3/s4.383 ft3/lbm=11.41 kg/s

V2=mv2A2=(11.41 lbm/s)(4.383 ft3/lbm)1.2 ft2=41.68 ft/s

Similarly at the inlet,

v1=RT1P1=(0.3704 psia⋅ft3/lbm⋅R)(800+460 R)500 psia=0.9334 ft3/lbm

V1=mv1A1=(11.41 lbm/s)(0.9334 ft3/lbm)0.6 ft2=17.75 ft/s

Substituting into the energy balance equation gives

Wout=mcp(T1−T2)+V12−V222(11.41 lbm/s)(0.2485 Btu/lbm⋅R)(800−250)R+(17.75 ft/s)2−(41.68 m/s)221 Btu/lbm25,037 ft2/s21559 kW

6-63 Steam expands in a two-stage adiabatic turbine from a specified state to another state. Some steam is extracted at the end of the first stage. The power output of the turbine is to be determined.

STEAM

20 kg/s

12.5 MPa

550°C

20 kg/s

II

1 MPa

200°C

1 kg/s

100 kPa

100°C

I

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The turbine is adiabatic and thus heat transfer is negligible.

Properties From the steam tables (Table A-6)

P1=12.5 MPa  h1=3476.5 kJ/kgP2=1 MPa  h2=2828.3 kJ/kgP3=100 kPa  h3=2675.8 kJ/kg

Analysis The mass flow rate through the second stage is

m3=m1−m2=20−1=19 kg/s

We take the entire turbine, including the connection part between the two stages, as the system, which is a control volume since mass crosses the boundary. Noting that one fluid stream enters the turbine and two fluid streams leave, the energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m1h1=m2h2+m3h3+WoutWout=m1h1−m2h2−m3h3

Substituting, the power output of the turbine is

Wout=(20 kg/s)(3476.5 kJ/kg)−(1 kg/s)(2828.3 kJ/kg)−(19 kg/s)(2675.8 kJ/kg)15,860 kW

# Throttling Valves

6-64C  Yes.

6-65C  No. Because air is an ideal gas and h = h(T) for ideal gases.  Thus if h remains constant, so does the temperature.

6-66C  If it remains in the liquid phase, no. But if some of the liquid vaporizes during throttling, then yes.

6-67C The temperature of a fluid can increase, decrease, or remain the same during a throttling process. Therefore, this claim is valid since no thermodynamic laws are violated.

6-68 Refrigerant-134a is throttled by a capillary tube. The quality of the refrigerant at the exit is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 Heat transfer to or from the fluid is negligible. 4 There are no work interactions involved.

Analysis There is only one inlet and one exit, and thus m1=m2=m. We take the throttling valve as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

50°C

Sat. liquid

-12°C

R-134a

Ein−Eout=ΔEsystemä0  (steady)=0   Ein=Eoutmh1=mh2h1=h2

since Q≃W=Δke≃Δpe≃0.

The inlet enthalpy of R-134a is, from the refrigerant tables (Table A-11),

T1=50°C  h1=hf=123.49 kJ/kg

The exit quality is

T2=−12°C   x2=h2−hfhfg=123.49−35.92207.38=0.422

6-69 Steam is throttled from a specified pressure to a specified state. The quality at the inlet is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 Heat transfer to or from the fluid is negligible. 4 There are no work interactions involved.

Analysis There is only one inlet and one exit, and thus m1=m2=m. We take the throttling valve as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Steam

2 MPa

100 kPa

120C

Throttling valve

Ein−Eout=ΔEsystemä0  (steady)=0   Ein=Eoutmh1=mh2h1=h2

since Q≃W=Δke≃Δpe≃0.

The enthalpy of steam at the exit is (Table A-6),

P2=100 kPa  h2=2716.1 kJ/kg

The quality of the steam at the inlet is (Table A-5)

P2=2000 kPa   x1=h2−hfhfg=2716.1−908.471889.8=0.957

6-70 CD EES Refrigerant-134a is throttled by a valve. The pressure and internal energy after expansion are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 Heat transfer to or from the fluid is negligible. 4 There are no work interactions involved.

Properties The inlet enthalpy of R-134a is, from the refrigerant tables (Tables A-11 through 13),

P1=0.8  MPa  h1≃hf @ 25C=86.41  kJ/kg

Analysis There is only one inlet and one exit, and thus . We take the throttling valve as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

P1 = 0.8 MPa

T1 = 25C

T2 = -20C

R-134a

Ein−Eout=ΔEsystemä0  (steady)=0   Ein=Eoutmh1=mh2h1=h2

since . Then,

T2=−20°C  hf=25.49  kJ/kg,uf=25.39  kJ/kghg=238.41  kJ/kgug=218.84  kJ/kg

Obviously hf < h2 <hg, thus the refrigerant exists as a saturated mixture at the exit state, and thus

P2  =  Psat @ -20°C  =  132.82 kPa

Also,

x2=h2−hfhfg=86.41−25.49212.91=0.2861

Thus,

u2=uf+x2ufg=25.39+0.2861×193.45=80.74  kJ/kg

6-71 Steam is throttled by a well-insulated valve. The temperature drop of the steam after the expansion is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 Heat transfer to or from the fluid is negligible. 4 There are no work interactions involved.

P1 = 8 MPa

T1 = 500C

P2 = 6 MPa

H2O

Properties The inlet enthalpy of steam is (Tables A-6),

P1=8  MPa  h1=3399.5  kJ/kg

Analysis There is only one inlet and one exit, and thus . We take the throttling valve as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−Eout=ΔEsystemä0  (steady)=0   Ein=Eoutmh1=mh2h1=h2

since . Then the exit temperature of steam becomes

P2=6  MPa  T2=490.1°C

6-72 EES Problem 6-71 is reconsidered. The effect of the exit pressure of steam on the exit temperature after throttling as the exit pressure varies from 6 MPa to 1 MPa is to be investigated.  The exit temperature of steam is to be plotted against the exit pressure.

Analysis The problem is solved using EES, and the solution is given below.

"Input information from Diagram Window"

{WorkingFluid$='Steam_iapws' "WorkingFluid: can be  changed to ammonia or other fluids"

P_in=8000 [kPa]

T_in=500  [C]

P_out=6000  [kPa]}

$Warning off

"Analysis"

m_dot_in=m_dot_out  "steady-state mass balance"

m_dot_in=1  "mass flow rate is arbitrary"

m_dot_in*h_in+Q_dot-W_dot-m_dot_out*h_out=0  "steady-state energy balance"

Q_dot=0  "assume the throttle to operate adiabatically"

W_dot=0  "throttles do not have any means of producing power"

h_in=enthalpy(WorkingFluid$,T=T_in,P=P_in)  "property table lookup"

T_out=temperature(WorkingFluid$,P=P_out,h=h_out)  "property table lookup"

x_out=quality(WorkingFluid$,P=P_out,h=h_out)  "x_out is the quality at the outlet"

P[1]=P_in;  P[2]=P_out;  h[1]=h_in;  h[2]=h_out  "use arrays to place points on property plot"

| Pout [kPa] | Tout [C] |
| --- | --- |
| 1000 | 463.1 |
| 2000 | 468.8 |
| 3000 | 474.3 |
| 4000 | 479.7 |
| 5000 | 484.9 |
| 6000 | 490.1 |

6-73E  High-pressure air is throttled to atmospheric pressure. The temperature of air after the expansion is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 Heat transfer to or from the fluid is negligible. 4 There are no work interactions involved. 5 Air is an ideal gas.

Analysis There is only one inlet and one exit, and thus . We take the throttling valve as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

P1 = 200 psia

T1 = 90F

P2 = 14.7 psia

Air

Ein−Eout=ΔEsystemä0  (steady)=0   Ein=Eoutmh1=mh2h1=h2

since .

For an ideal gas,

h = h(T).

Therefore,

T2 = T1 = 90°F

6-74 Carbon dioxide flows through a throttling valve. The temperature change of CO2 is to be determined if CO2 is assumed an ideal gas and a real gas.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 Heat transfer to or from the fluid is negligible. 4 There are no work interactions involved.

Analysis There is only one inlet and one exit, and thus . We take the throttling valve as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

CO2

5 MPa

100C

100 kPa

Ein−Eout=ΔEsystemä0  (steady)=0   Ein=Eoutmh1=mh2h1=h2

since .

(a) For an ideal gas,  h = h(T), and therefore,

T2=T1=100°CΔT=T1−T2=0°C

(b) We obtain real gas properties of CO2 from EES software as follows

P1=5 MPa  h1=34.77 kJ/kg

P2=100 kPa T2=66.0°C

Note that EES uses a different reference state from the textbook for CO2 properties. The temperature difference in this case becomes

ΔT=T1−T2=100−66.0=34.0°C

That is, the temperature of CO2 decreases by 34C in a throttling process if its real gas properties are used.

# Mixing Chambers and Heat Exchangers

6-75C  Yes, if the mixing chamber is losing heat to the surrounding medium.

6-76C Under the conditions of no heat and work interactions between the mixing chamber and the surrounding medium.

6-77C Under the conditions of no heat and work interactions between the heat exchanger and the surrounding medium.

6-78 A hot water stream is mixed with a cold water stream. For a specified mixture temperature, the mass flow rate of cold water is to be determined.

Assumptions 1 Steady operating conditions exist. 2 The mixing chamber is well-insulated so that heat loss to the surroundings is negligible. 3 Changes in the kinetic and potential energies of fluid streams are negligible. 4 Fluid properties are constant. 5 There are no work interactions.

H2O

(P = 250 kPa)

T3 = 42C

T1 = 80C

m1 = 0.5 kg/s

T2 = 20C

m2

·

·

Properties Noting that T < Tsat @ 250 kPa = 127.41°C, the water in all three streams exists as a compressed liquid, which can be approximated as a saturated liquid at the given temperature.  Thus,

h1    hf @ 80C  =	335.02 kJ/kg

h2    hf @ 20C  =	  83.915 kJ/kg

h3    hf @ 42C  =	175.90 kJ/kg

Analysis We take the mixing chamber as the system, which is a control volume. The mass and energy balances for this steady-flow system can be expressed in the rate form as

Mass balance:

min−mout=Δmsystemä0  (steady)=0  m1+m2=m3

Energy balance:

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutm1h1+m2h2=m3h3   (since  {Q=W=Δke≃Δpe≃0)

Combining the two relations and solving for  gives

m1h1+m2h2=m1+m2h3

Substituting, the mass flow rate of cold water stream is determined to be

m2=335.02−175.90 kJ/kg175.90−83.915 kJ/kg0.5  kg/s=0.865   kg/s

6-79E Liquid water is heated in a chamber by mixing it with saturated water vapor. If both streams enter at the same rate, the temperature and quality (if saturated) of the exit stream is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 There are no work interactions. 4 The device is adiabatic and thus heat transfer is negligible.

Properties From steam tables (Tables A-5E through A-6E),

h1    hf @ 50F   =   18.07 Btu/lbm

h2  =  hg @ 50 psia = 1174.2 Btu/lbm

Analysis   We take the mixing chamber as the system, which is a control volume since mass crosses the boundary. The mass and energy balances for this steady-flow system can be expressed in the rate form as

Mass balance:

H2O

(P = 50 psia)

T3, x3

T1 = 50F

Sat. vapor

m2 = m1

·

·

min−mout=Δmsystemä0  (steady)=0  min=mout m1+m2=m3=2m m1=m2=m

Energy balance:

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutm1h1+m2h2=m3h3   (since  {Q=W=Δke≃Δpe≃0)

Combining the two gives

mh1+mh2=2mh3 or h3=h1+h22

Substituting,

h3  =  (18.07 + 1174.2)/2 = 596.16 Btu/lbm

At 50 psia, hf = 250.21 Btu/lbm and hg = 1174.2 Btu/lbm.  Thus the exit stream is a saturated mixture since hf < h3 < hg.  Therefore,

T3 = Tsat @ 50 psia = 280.99F

and

x3=h3−hfhfg=596.16−250.21924.03=0.374

6-80 Two streams of refrigerant-134a are mixed in a chamber. If the cold stream enters at twice the rate of the hot stream, the temperature and quality (if saturated) of the exit stream are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 There are no work interactions. 4 The device is adiabatic and thus heat transfer is negligible.

Properties From R-134a tables (Tables A-11 through A-13),

h1    hf @ 12C   =  68.18 kJ/kg

h2  =  h @ 1 MPa,  60C = 293.38 kJ/kg

Analysis   We take the mixing chamber as the system, which is a control volume since mass crosses the boundary. The mass and energy balances for this steady-flow system can be expressed in the rate form as

Mass balance:

R-134a

(P = 1 MPa)

T3,  x3

T2 = 60C

T1 = 12C

m1 = 2m2

·

·

min−mout=Δmsystemä0  (steady)=0  min=mout m1+m2=m3=3m2   since   {m1=2m2

Energy balance:

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutm1h1+m2h2=m3h3   (since  {Q=W=Δke≃Δpe≃0)

Combining the two gives	   2m2h1+m2h2=3m2h3 or h3=2h1+h23

Substituting,

h3 = (268.18 + 293.38)/3 = 143.25 kJ/kg

At 1 MPa, hf = 107.32 kJ/kg and hg = 270.99 kJ/kg.  Thus the exit stream is a saturated mixture since

hf < h3 < hg.  Therefore,

T3 = Tsat @ 1 MPa = 39.37C

and

x3=h3−hfhfg=143.25−107.32163.67=0.220

6-81 EES Problem 6-80 is reconsidered. The effect of the mass flow rate of the cold stream of R-134a on the temperature and the quality of the exit stream as the ratio of the mass flow rate of the cold stream to that of the hot stream varies from 1 to 4 is to be investigated. The mixture temperature and quality are to be plotted against the cold-to-hot mass flow rate ratio.

Analysis The problem is solved using EES, and the solution is given below.

"Input Data"

"m_frac = 2"  "m_frac =m_dot_cold/m_dot_hot= m_dot_1/m_dot_2"

T[1]=12 [C]

P[1]=1000 [kPa]

T[2]=60 [C]

P[2]=1000 [kPa]

m_dot_1=m_frac*m_dot_2

P[3]=1000 [kPa]

m_dot_1=1

"Conservation of mass for the R134a: Sum of m_dot_in=m_dot_out"

m_dot_1+ m_dot_2 =m_dot_3

"Conservation of Energy for steady-flow: neglect changes in KE and PE"

"We assume no heat transfer and no work occur across the control surface."

E_dot_in - E_dot_out = DELTAE_dot_cv

DELTAE_dot_cv=0   "Steady-flow requirement"

E_dot_in=m_dot_1*h[1] + m_dot_2*h[2]

E_dot_out=m_dot_3*h[3]

"Property data are given by:"

h[1] =enthalpy(R134a,T=T[1],P=P[1])

h[2] =enthalpy(R134a,T=T[2],P=P[2])

T[3] =temperature(R134a,P=P[3],h=h[3])

x_3=QUALITY(R134a,h=h[3],P=P[3])

| mfrac | T3 [C] | x3 |
| --- | --- | --- |
| 1 | 39.37 | 0.4491 |
| 1.333 | 39.37 | 0.3509 |
| 1.667 | 39.37 | 0.2772 |
| 2 | 39.37 | 0.2199 |
| 2.333 | 39.37 | 0.174 |
| 2.667 | 39.37 | 0.1365 |
| 3 | 39.37 | 0.1053 |
| 3.333 | 39.37 | 0.07881 |
| 3.667 | 39.37 | 0.05613 |
| 4 | 39.37 | 0.03649 |

6-82 Refrigerant-134a is condensed in a water-cooled condenser. The mass flow rate of the cooling water  required is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 There are no work interactions. 4 Heat loss from the device to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid.

Properties  The enthalpies of R-134a at the inlet and the exit states are (Tables A-11 through A-13)

Water

R-134a

3

4

1

2

P3=700  kPa  h3=308.33  kJ/kgP4=700  kPa  h4=hf @700  kPa=88.82  kJ/kg

Water exists as compressed liquid at both states, and thus (Table A-4)

h1    hf @ 15C   =   62.98 kJ/kg

h2    hf @ 25C   = 104.83 kJ/kg

Analysis We take the heat exchanger as the system, which is a control volume. The mass and energy balances for this steady-flow system can be expressed in the rate form as

Mass balance (for each fluid stream):

min−mout=Δmsystemä0  (steady)=0 → min=mout → m1=m2=mw and m3=m4=mR

Energy balance (for the heat exchanger):

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout m1h1+m3h3=m2h2+m4h4   (since  {Q=W=Δke≃Δpe≃0)

Combining the two,        mwh2−h1=mRh3−h4

Solving for :

Substituting,

mw=(308.33−88.82) kJ/kg(104.83−62.98) kJ/kg(8  kg/min)=42.0  kg/min

6-83E CD EES Air is heated in a steam heating system. For specified flow rates, the volume flow rate of air at the inlet is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 There are no work interactions. 4 Heat loss from the device to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 5 Air is an ideal gas with constant specific heats at room temperature.

Properties The gas constant of air is 0.3704 psia.ft3/lbm.R (Table A-1E). The constant pressure specific heat of air is cp = 0.240 Btu/lbm·°F (Table A-2E). The enthalpies of steam at the inlet and the exit states are (Tables A-4E through A-6E)

P3=30  psia  h3=1237.9  Btu/lbmP4=25  psia  h4≃hf @ 212F=180.21  Btu/lbm

Analysis  We take the entire heat exchanger as the system, which is a control volume. The mass and energy balances for this steady-flow system can be expressed in the rate form as

AIR

Steam

3

4

1

2

Mass balance ( for each fluid stream):

min−mout=ΔmsystemÃ0  (steady)=0  min=mout  m1=m2=ma   and    m3=m4=ms

Energy balance (for the entire heat exchanger):

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout m1h1+m3h3=m2h2+m4h4   (since   {Q=W=Δke≃Δpe≃0)

Combining the two,	     mah2−h1=msh3−h4

Solving for :

ma=h3−h4h2−h1ms≃h3−h4cpT2−T1ms

Substituting,

ma=(1237.9−180.21) Btu/lbm(0.240  Btu/lbm⋅°F)(130−80)°F(15  lbm/min)=1322  lbm/min=22.04  lbm/s

Also,

v1=RT1P1=(0.3704  psia⋅ft3/lbm⋅R)(540  R)14.7  psia=13.61  ft3/lbm

Then the volume flow rate of air at the inlet becomes

{ dot 1 { dot a1

6-84 Steam is condensed by cooling water in the condenser of a power plant. If the temperature rise of the cooling water is not to exceed 10C, the minimum mass flow rate of the cooling water required is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 There are no work interactions. 4 Heat loss from the device to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 5 Liquid water is an incompressible substance with constant specific heats at room temperature.

Properties The cooling water exists as compressed liquid at both states, and its specific heat at room temperature is c = 4.18 kJ/kg·°C (Table A-3). The enthalpies of the steam at the inlet and the exit states are (Tables A-5 and A-6)

P3=20  kPa  h3=hf+x3hfg=251.42+0.95×2357.5=2491.1  kJ/kgP4=20  kPa  h4≃hf @ 20  kPa=251.42  kJ/kg

Analysis We take the heat exchanger as the system, which is a control volume. The mass and energy balances for this steady-flow system can be expressed in the rate form as

Steam

20 kPa

Water

Mass balance (for each fluid stream):

min−mout=Δmsystemä0  (steady)=0 min=mout  m1=m2=mw   and    m3=m4=ms

Energy balance (for the heat exchanger):

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout m1h1+m3h3=m2h2+m4h4   (since   {Q=W=Δke≃Δpe≃0)

Combining the two,

mwh2−h1=msh3−h4

Solving for :

mw=h3−h4h2−h1ms≃h3−h4cpT2−T1ms

Substituting,

mw=(2491.1−251.42) kJ/kg(4.18  kJ/kg⋅C)(10°C)(20,000/3600  kg/s)=297.7 kg/s

6-85 Ethylene glycol is cooled by water in a heat exchanger. The rate of heat transfer in the heat exchanger and the mass flow rate of water are to be determined.

Assumptions 1 Steady operating conditions exist. 2 The heat exchanger is well-insulated so that heat loss to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 3 Changes in the kinetic and potential energies of fluid streams are negligible. 4 Fluid properties are constant.

Hot Glycol

80C

2 kg/s

Cold Water

20C

40C

Properties The specific heats of water and ethylene glycol are given to be 4.18 and 2.56 kJ/kg.C, respectively.

Analysis (a) We take the ethylene glycol tubes as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutmh1=Qout+mh2   (since Δke≃Δpe≃0)Qout=mcp(T1−T2)

Then the rate of heat transfer becomes

Q=[mcp(Tin−Tout)]glycol=(2 kg/s)(2.56 kJ/kg.°C)(80°C−40°C)=204.8 kW

(b) The rate of heat transfer from glycol must be equal to the rate of heat transfer to the water. Then,

Q=[mcp(Tout−Tin)]watermwater=Qcp(Tout−Tin)=204.8 kJ/s(4.18 kJ/kg.°C)(55°C−20°C)=1.4 kg/s

6-86 EES Problem 6-85 is reconsidered. The effect of the inlet temperature of cooling water on the mass flow rate of water as the inlet temperature varies from 10°C to 40°C at constant exit temperature) is to be investigated. The mass flow rate of water is to be plotted against the inlet temperature.

Analysis The problem is solved using EES, and the solution is given below.

"Input Data"

{T_w[1]=20 [C]}

T_w[2]=55 [C]  "w: water"

m_dot_eg=2 [kg/s] "eg: ethylene glycol"

T_eg[1]=80 [C]

T_eg[2]=40 [C]

C_p_w=4.18 [kJ/kg-K]

C_p_eg=2.56 [kJ/kg-K]

"Conservation of mass for the water: m_dot_w_in=m_dot_w_out=m_dot_w"

"Conservation of mass for the ethylene glycol: m_dot_eg_in=m_dot_eg_out=m_dot_eg"

"Conservation of Energy for steady-flow: neglect changes in KE and PE in each mass steam"

"We assume no heat transfer and no work occur across the control surface."

E_dot_in - E_dot_out = DELTAE_dot_cv

DELTAE_dot_cv=0 "Steady-flow requirement"

E_dot_in=m_dot_w*h_w[1] + m_dot_eg*h_eg[1]

E_dot_out=m_dot_w*h_w[2] + m_dot_eg*h_eg[2]

Q_exchanged =m_dot_eg*h_eg[1] - m_dot_eg*h_eg[2]

"Property data are given by:"

h_w[1] =C_p_w*T_w[1]  "liquid approximation applied for water and ethylene glycol"

h_w[2] =C_p_w*T_w[2]

h_eg[1] =C_p_eg*T_eg[1]

h_eg[2] =C_p_eg*T_eg[2]

| mw [kg/s] | Tw,1 [C] |
| --- | --- |
| 1.089 | 10 |
| 1.225 | 15 |
| 1.4 | 20 |
| 1.633 | 25 |
| 1.96 | 30 |
| 2.45 | 35 |
| 3.266 | 40 |

6-87 Oil is to be cooled by water in a thin-walled heat exchanger. The rate of heat transfer in the heat exchanger and the exit temperature of water is to be determined.

Assumptions 1 Steady operating conditions exist. 2 The heat exchanger is well-insulated so that heat loss to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 3 Changes in the kinetic and potential energies of fluid streams are negligible.  4 Fluid properties are constant.

Cold

water

22C

1.5 kg/s

Hot oil

150C

2 kg/s

40C

Properties The specific heats of water and oil are given to be 4.18 and 2.20 kJ/kg.C, respectively.

Analysis We take the oil tubes as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutmh1=Qout+mh2   (since  Δke≃Δpe≃0)Qout=mcp(T1−T2)

Then the rate of heat transfer from the oil becomes

Q=[mcp(Tin−Tout)]oil=(2 kg/s)(2.2 kJ/kg.°C)(150°C−40°C)=484 kW

Noting that the heat lost by the oil is gained by the water, the outlet temperature of the water is determined from

Q=[mcp(Tout−Tin)]waterTout=Tin+Qmwatercp=22°C+484 kJ/s(1.5 kg/s)(4.18 kJ/kg.°C)=99.2°C

6-88 Cold water is heated by hot water in a heat exchanger. The rate of heat transfer and the exit temperature of hot water are to be determined.

Assumptions 1 Steady operating conditions exist. 2 The heat exchanger is well-insulated so that heat loss to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 3 Changes in the kinetic and potential energies of fluid streams are negligible.  4 Fluid properties are constant.

Properties The specific heats of cold and hot water are given to be 4.18 and 4.19 kJ/kg.C, respectively.

Hot water

100C

3 kg/s

Cold Water

15C

0.60 kg/s

Analysis We take the cold water tubes as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since  Δke≃Δpe≃0)Qin=mcp(T2−T1)

Then the rate of heat transfer to the cold water in this heat exchanger becomes

Q=[mcp(Tout−Tin)]cold water=(0.60 kg/s)(4.18 kJ/kg.°C)(45°C−15°C)=75.24 kW

Noting that heat gain by the cold water is equal to the heat loss by the hot water, the outlet temperature of the hot water is determined to be

Q=[mcp(Tin−Tout)]hot waterTout=Tin−Qmcp=100°C−75.24 kW(3 kg/s)(4.19 kJ/kg.°C)=94.0°C

6-89 Air is preheated by hot exhaust gases in a cross-flow heat exchanger. The rate of heat transfer and the outlet temperature of the air are to be determined.

Assumptions 1 Steady operating conditions exist. 2 The heat exchanger is well-insulated so that heat loss to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 3 Changes in the kinetic and potential energies of fluid streams are negligible.  4 Fluid properties are constant.

Properties The specific heats of air and combustion gases are given to be 1.005 and 1.10 kJ/kg.C, respectively.

Analysis We take the exhaust pipes as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Air

95 kPa

20C

0.8 m3/s

Exhaust gases

1.1 kg/s, 95C

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0→Ein=Eoutmh1=Qout+mh2   (since  Δke≃Δpe≃0)Qout=mcp(T1−T2)

Then the rate of heat transfer from the exhaust gases becomes

Q=[mcp(Tin−Tout)]gas(1.1 kg/s)(1.1 kJ/kg.°C)(180°C−95°C)102.85 kW

The mass flow rate of air is

m=PVRT=(95 kPa)(0.8 m3/s)(0.287 kPa.m3/kg.K)×293 K=0.904 kg/s

Noting that heat loss by the exhaust gases is equal to the heat gain by the air, the outlet temperature of the air becomes

Q=mcp(Tc,out−Tc,in)Tc,out=Tc,in+Qmcp=20°C+102.85 kW(0.904 kg/s)(1.005 kJ/kg.°C)=133.2°C

6-90 An adiabatic open feedwater heater mixes steam with feedwater. The outlet mass flow rate and the outlet velocity are to be determined.

Assumptions Steady operating conditions exist.

Steam

100 kPa

160C

0.2 kg/s

Water

100 kPa

50C

10 kg/s

100 kPa

60C

2

1

3

Properties From a mass balance

m3=m1+m2=0.2+10=10.2 kg/s

The specific volume at the exit is (Table A-4)

P3=100 kPa     v3≃vf@60°C=0.001017 m3/kg

The exit velocity is then

V3=m3v3A3=4m3v3πD24(10.2 kg/s)(0.001017 m3/kg)π(0.03 m)214.68 m/s

6-91E An adiabatic open feedwater heater mixes steam with feedwater. The outlet mass flow rate and the outlet velocity are to be determined for two exit temperatures.

Assumptions Steady operating conditions exist.

Properties From a mass balance

Steam

10 psia

200F

0.1 lbm/s

Water

10 psia

100F

2 lbm/s

10 psia

120F

2

1

3

m3=m1+m2=0.1+2=2.1 lbm/s

The specific volume at the exit is (Table A-4E)

P3=10 psia     v3≃vf@120°F=0.01620 ft3/lbm

The exit velocity is then

V3=m3v3A3=4m3v3πD24(2.1 lbm/s)(0.01620 ft3/lbm)π(0.5 ft)2=0.1733 ft/s

When the temperature at the exit is 180F, we have

m3=m1+m2=0.1+2=2.1 lbm/s

P3=10 psia     v3≃vf@180°F=0.01651 ft3/lbm

V3=m3v3A3=4m3v3πD2=4(2.1 lbm/s)(0.01651 ft3/lbm)π(0.5 ft)2=0.1766 ft/s

The mass flow rate at the exit is same while the exit velocity slightly increases when the exit temperature is 180F instead of 120F.

6-92 Water is heated by hot oil in a heat exchanger. The rate of heat transfer in the heat exchanger and the outlet temperature of oil are to be determined.

Assumptions 1 Steady operating conditions exist. 2 The heat exchanger is well-insulated so that heat loss to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 3 Changes in the kinetic and potential energies of fluid streams are negligible.  4 Fluid properties are constant.

Properties The specific heats of  water and oil are given to be 4.18 and 2.3 kJ/kg.C, respectively.

Oil

170C

10 kg/s

Water

20C

4.5 kg/s

70C

Analysis We take the cold water tubes as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since Δke≃Δpe≃0)Qin=mcp(T2−T1)

Then the rate of heat transfer to the cold water in this heat exchanger becomes

Q=[mcp(Tout−Tin)]water=(4.5 kg/s)(4.18 kJ/kg.°C)(70°C−20°C)=940.5 kW

Noting that heat gain by the water is equal to the heat loss by the oil, the outlet temperature of oil is determined from

Q=[mcp(Tin−Tout)]oilTout=Tin−Qmcp=170°C−940.5 kW(10 kg/s)(2.3 kJ/kg.°C)=129.1°C

6-93E Steam is condensed by cooling water in a condenser. The rate of heat transfer in the heat exchanger and the rate of condensation of steam are to be determined.

Assumptions 1 Steady operating conditions exist. 2 The heat exchanger is well-insulated so that heat loss to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 3 Changes in the kinetic and potential energies of fluid streams are negligible. 4 Fluid properties are constant.

Steam

85F

60F

Water

73F

85F

Properties The specific heat of water is 1.0 Btu/lbm.F (Table A-3E). The enthalpy of vaporization of water at 85F is 1045.2 Btu/lbm (Table A-4E).

Analysis We take the tube-side of the heat exchanger where cold water is flowing as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since  Δke≃Δpe≃0)Qin=mcp(T2−T1)

Then the rate of heat transfer to the cold water in this heat exchanger becomes

Q=[mcp(Tout−Tin)]water=(138 lbm/s)(1.0 Btu/lbm.°F)(73°F−60°F)=1794 Btu/s

Noting that heat gain by the water is equal to the heat loss by the condensing steam, the rate of condensation of the steam in the heat exchanger is determined from

Q=(mhfg)steam=msteam=Qhfg=1794 Btu/s1045.2 Btu/lbm=1.72  lbm/s

6-94 Two streams of cold and warm air are mixed in a chamber. If the ratio of hot to cold air is 1.6, the mixture temperature and the rate of heat gain of the room are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 There are no work interactions. 4 The device is adiabatic and thus heat transfer is negligible.

Cold

air

5C

24C

Room

Warm

air

34C

Properties The gas constant of air is      R = 0.287 kPa.m3/kg.K. The enthalpies of air are obtained from air table (Table A-21) as

h1  = h @278 K  = 278.13 kJ/kg

h2  =  h @ 307 K = 307.23 kJ/kg

hroom  =  h @ 297 K = 297.18 kJ/kg

Analysis (a) We take the mixing chamber as the system, which is a control volume since mass crosses the boundary. The mass and energy balances for this steady-flow system can be expressed in the rate form as

Mass balance:

min−mout=Δmsystemä0  (steady)=0 → min=mout→ m1+1.6m1=m3=2.6m1   since    {m2=1.6m1

Energy balance:

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutm1h1+m2h2=m3h3     (since   {Q≃W≃Δke≃Δpe≃0)

Combining the two gives	   m1h1+1.6m1h2=2.6m1h3 or h3=h1+1.6h22.6

Substituting,

h3 = (278.13 +1.6 307.23)/2.6 = 296.04 kJ/kg

From air table at this enthalpy, the mixture temperature is

T3  = T @ h = 296.04 kJ/kg  = 295.9 K = 22.9C

(b) The mass flow rates are determined as follows

v1=RT1P=(0.287  kPa⋅m3/kg⋅K)(5+273  K)105  kPa=0.7599 m3 kgm1=V1v1=1.25 m3/s0.7599 m3/kg=1.645 kg/sm3=2.6m1=2.6(1.645 kg/s)=4.277 kg/s

The rate of heat gain of the room is determined from

Qcool=m3(hroom−h3)=(4.277 kg/s)(297.18−296.04) kJ/kg=4.88 kW

6-95 A heat exchanger that is not insulated is used to produce steam from the heat given up by the exhaust gases of an internal combustion engine. The temperature of exhaust gases at the heat exchanger exit and the rate of heat transfer to the water are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 There are no work interactions. 4 Exhaust gases are assumed to have air properties with constant specific heats.

Properties The constant pressure specific heat of the exhaust gases is taken to be cp = 1.045 kJ/kg·°C (Table A-2). The inlet and exit enthalpies of water are (Tables A-4 and A-5)

Water

15C

Q

Exh. gas

400C

Heat exchanger

2 MPa

sat. vap.

Tw,in=15°C hw,in=62.98 kJ/kgPw,out=2 MPa hw,out=2798.3 kJ/kg

Analysis We take the entire heat exchanger as the system, which is a control volume. The mass and energy balances for this steady-flow system can be expressed in the rate form as

Mass balance (for each fluid stream):

min−mout=Δmsystemä0  (steady)=0  min=mout

Energy balance (for the entire heat exchanger):

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout mexhhexh,in+mwhw,in= mexhhexh,out+mwhw,out+Qout  (since   {W=Δke≃Δpe≃0)

or	 mexhcpTexh,in+mwhw,in= mexhcpTexh,out+mwhw,out+Qout

Noting that the mass flow rate of exhaust gases is 15 times that of the water, substituting gives

15 {mw(1.045 kJ/kg.°C)(400°C)+mw(62.98 kJ/kg)                               =15 {mw(1.045 kJ/kg.°C)Texh,out+mw(2798.3 kJ/kg)+Qout	(1)

The heat given up by the exhaust gases and heat picked up by the water are

Qexh=mexhcp(Texh,in−Texh,out)=15 {mw(1.045 kJ/kg.°C)(400−Texh,out)°C	(2)

Qw=mw(hw,out−hw,in)=mw(2798.3−62.98)kJ/kg				(3)

The heat loss is

Qout=fheat lossQexh=0.1Qexh						(4)

The solution may be obtained by a trial-error approach. Or, solving the above equations simultaneously using EES software, we obtain

Texh,out=206.1°C,  {Qw=97.26 kW,   {mw=0.03556 kg/s,   {mexh=0.5333 kg/s

6-96 Two streams of water are mixed in an insulated chamber. The temperature of the exit stream is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 There are no work interactions. 4 The device is adiabatic and thus heat transfer is negligible.

Properties From the steam tables (Table A-4),

h1    hf @ 90C = 377.04 kJ/kg

h2   hf @ 50C = 209.34 kJ/kg

Analysis  We take the mixing chamber as the system, which is a control volume since mass crosses the boundary. The mass and energy balances for this steady-flow system can be expressed in the rate form as

Water

90C

30 kg/s

Water

50C

200 kg/s

1

2

3

Mass balance:  min−mout=Δmsystemä0  (steady)=0  min=mout m1+m2=m3

Energy balance:

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutm1h1+m2h2=m3h3   (since  {Q=W=Δke≃Δpe≃0)

Solving for the exit enthalpy,

h3=m1h1+m2h2m1+m2=(30)(377.04)+(200)(209.34)30+200=231.21 kJ/kg

The temperature corresponding to this enthalpy is

T3 =55.2C	(Table A-4)

6-97 A chilled-water heat-exchange unit is designed to cool air by water. The maximum water outlet temperature is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 There are no work interactions. 4 Heat loss from the device to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 5 Air is an ideal gas with constant specific heats at room temperature.

Properties The gas constant of air is 0.287 kPa.m3/kg.K (Table A-1). The constant pressure specific heat of air at room temperature is cp = 1.005 kJ/kg·°C (Table A-2a). The specific heat of water is 4.18 kJ/kgK (Table A-3).

Analysis The water temperature at the heat exchanger exit will be maximum when all the heat released by the air is picked up by the water. First, the inlet specific volume and the mass flow rate of air are

v1=RT1P1=(0.287  kPa⋅m3/kg⋅K)(303 K)100  kPa=0.8696 m3/kg { dot aV1v15  m3/s0.8696 m3/kg

We take the entire heat exchanger as the system, which is a control volume. The mass and energy balances for this steady-flow system can be expressed in the rate form as

Mass balance ( for each fluid stream):

min−mout=Δmsystemä0  (steady)=0 → min=mout → m1=m3=ma and m2=m4=mw

Energy balance (for the entire heat exchanger):

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout m1h1+m2h2=m3h3+m4h4   (since   {Q=W=Δke≃Δpe≃0)

Combining the two,

ma(h1−h3)=mw(h4−h2)macp,a(T1−T3)=mwcp,w(T4−T2)

Solving for the exit temperature of water,

T4=T2+macp,a(T1−T3)mwcp,w=8°C+(5.750 kg/s)(1.005 kJ/kg⋅°C)(30−18)°C(2 kg/s)(4.18 kJ/kg⋅°C)=16.3°C

# Pipe and duct Flow

6-98 Saturated liquid water is heated in a steam boiler at a specified rate. The rate of heat transfer in the boiler is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 There are no work interactions.

Analysis We take the pipe in which the water is heated as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

5 MPa

350C

Water

5 MPa, sat. liq.

10 kg/s

Qin

.

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout mh1+Qin=mh2Qin=m(h2−h1)

The enthalpies of water at the inlet and exit of the boiler are (Table A-5, A-6).

P1=5 MPa  h1≃hf@5 MPa=1154.5 kJ/kgP2=5 MPa  h2=3069.3 kJ/kg

Substituting,

Qin=(10 kg/s)(3069.3−1154.5)kJ/kg=19,150 kW

6-99 Air at a specified rate is heated by an electrical heater. The current is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The heat losses from the air is negligible.

Properties The gas constant of air is 0.287 kPa.m3/kg.K (Table A-1). The constant pressure specific heat of air at room temperature is cp = 1.005 kJ/kg·°C (Table A-2a).

Analysis We take the pipe in which the air is heated as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

100 kPa

30C

Air

100 kPa, 15C

0.3 m3/s

We,in

.

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout mh1+We,in=mh2We,in=m(h2−h1)VI=mcp(T2−T1)

The inlet specific volume and the mass flow rate of air are

v1=RT1P1=(0.287  kPa⋅m3/kg⋅K)(288 K)100  kPa=0.8266 m3/kg { dot V1v10.3  m3/s0.8266 m3/kg

Substituting into the energy balance equation and solving for the current gives

I=mcp(T2−T1)V=(0.3629 kg/s)(1.005 kJ/kg⋅K)(30−15)K110 V1000 VI1 kJ/s=49.7 Amperes

6-100E The cooling fan of a computer draws air, which is heated in the computer by absorbing the heat of PC circuits. The electrical power dissipated by the circuits is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 All the heat dissipated by the circuits are picked up by the air drawn by the fan.

Properties The gas constant of air is 0.3704 psia·ft3/lbm·R (Table A-1E). The constant pressure specific heat of air at room temperature is cp = 0.240 Btu/lbm·°F (Table A-2Ea).

Analysis We take the pipe in which the air is heated as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

14.7 psia

80F

Air

14.7 psia, 70F

0.5 ft3/s

We,in

.

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout mh1+We,in=mh2We,in=m(h2−h1)We,in=mcp(T2−T1)

The inlet specific volume and the mass flow rate of air are

v1=RT1P1=(0.3704 psia⋅ft3/lbm⋅R)(530 R)14.7 psia=13.35 ft3/lbm { dot V1v10.5  ft3/s13.35 ft3/lbm

Substituting,

We,out=(0.03745 lbm/s)(0.240 Btu/lbm⋅R)(80−70)Btu/lbm1 kW0.94782 Btu=0.0948 kW

6-101 A desktop computer is to be cooled safely by a fan in hot environments and high elevations. The air flow rate of the fan and the diameter of the casing are to be determined.

Assumptions 1 Steady operation under worst conditions is considered. 2 Air is an ideal gas with constant specific heats. 3 Kinetic and potential energy changes are negligible.

Properties The specific heat of air at the average temperature of Tavg =  (45+60)/2 = 52.5C = 325.5 K is cp = 1.0065 kJ/kg.C. The gas constant for air is R = 0.287 kJ/kg.K (Table A-2).

Analysis The fan selected must be able to meet the cooling requirements of the computer at worst conditions. Therefore, we assume air to enter the computer at 66.63 kPa and 45C, and leave at 60C.

We take the air space in the computer as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since  Δke≃Δpe≃0)Qin=mcp(T2−T1)

Then the required mass flow rate of air to absorb heat at a rate of 60 W is determined to be

Q=mcp(Tout−Tin)→m=Qcp(Tout−Tin)=60 W(1006.5 J/kg.°C)(60-45)°C                                                                    =0.00397 kg/s=0.238 kg/min

The density of air entering the fan at the exit and its volume flow rate are

ρ=PRT=66.63 kPa(0.287 kPa.m3/kg.K)(60+273)K=0.6972 kg/m3V=mρ=0.238 kg/min0.6972 kg/m3=0.341 m3/min

For an average exit velocity of 110 m/min, the diameter of the casing of the fan is determined from

V=AcV=πD24V→D=4VπV=(4)(0.341 m3/min)π(110 m/min)=0.063 m=6.3 cm

6-102 A desktop computer is to be cooled safely by a fan in hot environments and high elevations. The air flow rate of the fan and the diameter of the casing are to be determined.

Assumptions 1 Steady operation under worst conditions is considered. 2 Air is an ideal gas with constant specific heats. 3 Kinetic and potential energy changes are negligible.

Properties The specific heat of air at the average temperature of Tave =  (45+60)/2 = 52.5C is cp = 1.0065 kJ/kg.C The gas constant for air is R = 0.287 kJ/kg.K (Table A-2).

Analysis The fan selected must be able to meet the cooling requirements of the computer at worst conditions. Therefore, we assume air to enter the computer at 66.63 kPa and 45C, and leave at 60C.

We take the air space in the computer as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since  Δke≃Δpe≃0)Qin=mcp(T2−T1)

Then the required mass flow rate of air to absorb heat at a rate of 100 W is determined to be

Q=mcp(Tout−Tin)m=Qcp(Tout−Tin)=100 W(1006.5 J/kg.°C)(60-45)°C=0.006624 kg/s=0.397 kg/min

The density of air entering the fan at the exit and its volume flow rate are

ρ=PRT=66.63 kPa(0.287 kPa.m3/kg.K)(60+273)K=0.6972 kg/m3V=mρ=0.397 kg/min0.6972 kg/m3=0.57 m3/min

For an average exit velocity of 110 m/min, the diameter of the casing of the fan is determined from

V=AcV=πD24VD=4VπV=(4)(0.57 m3/min)π(110 m/min)=0.081 m=8.1 cm

6-103E Electronic devices mounted on a cold plate are cooled by water. The amount of heat generated by the electronic devices is to be determined.

Assumptions 1 Steady operating conditions exist. 2 About 15 percent of the heat generated is dissipated from the components to the surroundings by convection and radiation. 3 Kinetic and potential energy changes are negligible.

Properties The properties of water at room temperature are  = 62.1 lbm/ft3 and cp = 1.00 Btu/lbm.F (Table A-3E).

Analysis We take the tubes of the cold plate to be the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Water

inlet

Cold plate

1

2

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since  Δke≃Δpe≃0)Qin=mcp(T2−T1)

Then mass flow rate of water and the rate of heat removal by the water are determined to be

m=ρAV=ρπD24V=(62.1 lbm/ft3)π(0.2512 ft)24(60 ft/min)=1.270 lbm/min=76.2 lbm/hQ=mcp(Tout−Tin)=(76.2 lbm/h)(1.00 Btu/lbm.°F)(105-95)°F=762 Btu/h

which is 85 percent of the heat generated by the electronic devices. Then the total amount of heat generated by the electronic devices becomes

Q=762 Btu/h0.85=896 Btu/h=263 W

6-104 CD EES The components of an electronic device located in a horizontal duct of rectangular cross section are cooled by forced air. The heat transfer from the outer surfaces of the duct is to be determined.

Assumptions 1 Steady operating conditions exist. 2 Air is an ideal gas with constant specific heats at room temperature. 3 Kinetic and potential energy changes are negligible

Properties The gas constant of air is R  = 0.287 kJ/kg.C (Table A-1). The specific heat of air at room temperature is cp = 1.005 kJ/kg.C (Table A-2).

Air

30C

0.6 m3/min

180 W

25C

40C

Analysis  The density of air entering the duct and the mass flow rate are

ρ=PRT=101.325 kPa(0.287 kPa.m3/kg.K)(30+273)K=1.165 kg/m3m=ρV=(1.165 kg/m3)(0.6 m3/min)=0.700kgmin

We take the channel, excluding the electronic components, to be the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since Δke≃Δpe≃0)Qin=mcp(T2−T1)

Then the rate of heat transfer to the air passing through the duct becomes

Qair=[mcp(Tout−Tin)]air=(0.70060 kg/s)(1.005 kJ/kg.°C)(40−30)°C=0.117 kW=117 W

The rest of the 180 W heat generated must be dissipated through the outer surfaces of the duct by natural convection and radiation,

6-105 The components of an electronic device located in a horizontal duct of circular cross section is cooled by forced air. The heat transfer from the outer surfaces of the duct is to be determined.

Assumptions 1 Steady operating conditions exist. 2 Air is an ideal gas with constant specific heats at room temperature. 3 Kinetic and potential energy changes are negligible

Properties The gas constant of air is R  = 0.287 kJ/kg.C (Table A-1). The specific heat of air at room temperature is cp = 1.005 kJ/kg.C (Table A-2).

Analysis  The density of air entering the duct and the mass flow rate are

ρ=PRT=101.325 kPa(0.287 kPa.m3/kg.K)(30+273)K=1.165 kg/m3m=ρV=(1.165 kg/m3)(0.6 m3/min)=0.700kgmin

We take the channel, excluding the electronic components, to be the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

1

Air

30C

0.6 m3/s

2

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since Δke≃Δpe≃0)Qin=mcp(T2−T1)

Then the rate of heat transfer to the air passing through the duct becomes

Qair=[mcp(Tout−Tin)]air=(0.70060 kg/s)(1.005 kJ/kg.°C)(40−30)°C=0.117 kW=117 W

The rest of the 180 W heat generated must be dissipated through the outer surfaces of the duct by natural convection and radiation,

6-106E Water is heated in a parabolic solar collector. The required length of parabolic collector is to be determined.

Assumptions 1 Steady operating conditions exist. 2 Heat loss from the tube is negligible so that the entire solar energy incident on the tube is transferred to the water. 3 Kinetic and potential energy changes are negligible

Properties The specific heat of water at room temperature is cp = 1.00 Btu/lbm.F (Table A-2E).

Analysis We take the thin aluminum tube to be the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

1

Water

55F

4 lbm/s

2

180F

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since Δke≃Δpe≃0)Qin=mwatercp(T2−T1)

Then the total rate of heat transfer to the water flowing through the tube becomes

Qtotal=mcp(Te−Ti)=(4 lbm/s)(1.00 Btu/lbm.°F)(180−55)°F=500 Btu/s=1,800,000 Btu/h

The length of the tube required is

L=QtotalQ=1,800,000 Btu/h400 Btu/h.ft=4500 ft

6-107 Air enters a hollow-core printed circuit board. The exit temperature of the air is to be determined.

Assumptions 1 Steady operating conditions exist. 2 Air is an ideal gas with constant specific heats at room temperature. 3 The local atmospheric pressure is 1 atm. 4 Kinetic and potential energy changes are negligible.

2

1

Air

32C

0.8 L/s

Properties The gas constant of air is R  = 0.287 kJ/kg.C (Table A-1). The specific heat of air at room temperature is cp = 1.005 kJ/kg.C (Table A-2).

Analysis The density of air entering the duct and the mass flow rate are

ρ=PRT=101.325 kPa(0.287 kPa.m3/kg.K)(32+273)K=1.16 kg/m3m=ρV=(1.16 kg/m3)(0.0008 m3s)=0.000928 kgs

We take the hollow core to be the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since Δke≃Δpe≃0)Qin=mcp(T2−T1)

Then the exit temperature of air leaving the hollow core becomes

Qin=mcp(T2−T1)→T2=T1+Qinmcp=32°C+20 J/s(0.000928 kg/s)(1005 J/kg.°C)=53.4°C

6-108 A computer is cooled by a fan blowing air through the case of the computer. The required flow rate of the air and the fraction of the temperature rise of air that is due to heat generated by the fan are to be determined.

Assumptions 1 Steady flow conditions exist. 2 Air is an ideal gas with constant specific heats. 3 The pressure of air is 1 atm. 4 Kinetic and potential energy changes are negligible

Properties   The specific heat of air at room temperature is cp = 1.005 kJ/kg.C (Table A-2).

Analysis   (a) We take the air space in the computer as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+Win+mh1=mh2   (since Δke≃Δpe≃0)Qin+Win=mcp(T2−T1)

Noting that the fan power is 25 W and the 8 PCBs transfer a total of 80 W of heat to air, the mass flow rate of air is determined to be

Qin+Win=mcp(Te−Ti)→m=Qin+Wincp(Te−Ti)=(8×10) W+25 W(1005 J/kg.°C)(10°C)=0.0104 kg/s

(b) The fraction of temperature rise of air that is due to the heat generated by the fan and its motor can be determined from

Q=mcpΔT→ΔT=Qmcp=25 W(0.0104 kg/s)(1005 J/kg.°C)=2.4°Cf=2.4°C10°C=0.24=24%

6-109 Hot water enters a pipe whose outer surface is exposed to cold air in a basement. The rate of heat loss from the water is to be determined.

Assumptions 1 Steady flow conditions exist. 2 Water is an incompressible substance with constant specific heats. 3 The changes in kinetic and potential energies are negligible.

Properties  The properties of water at the average temperature of (90+88)/2 = 89C are ρ=965 kg/m3 and cp = 4.21 kJ/kg.C (Table A-3).

Analysis  The mass flow rate of water is

m=ρAcV=(965 kg/m3)π(0.04 m)24(0.8m/s)=0.970 kg/s

1

Water

90C

0.8 m/s

2

88C

We take the section of the pipe in the basement to be the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutmh1=Qout+mh2   (since Δke≃Δpe≃0)Qout=mcp(T1−T2)

Then the rate of heat transfer from the hot water to the surrounding air becomes

Qout=mcp[Tin−Tout]water=(0.970 kg/s)(4.21 kJ/kg.°C)(90−88)°C=8.17 kW

6-110 EES Problem 6-109 is reconsidered. The effect of the inner pipe diameter on the rate of heat loss as the pipe diameter varies from 1.5 cm to 7.5 cm is to be investigated. The rate of heat loss is to be plotted against the diameter.

Analysis The problem is solved using EES, and the solution is given below.

"Knowns:"

{D = 0.04 [m]}

rho = 965 [kg/m^3]

Vel = 0.8 [m/s]

T_1 = 90 [C]

T_2 = 88 [C]

C_P = 4.21[kJ/kg-C]

"Analysis:"

"The mass flow rate of water is:"

Area = pi*D^2/4

m_dot = rho*Area*Vel

"We take the section of the pipe in the basement to be the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as"

E_dot_in - E_dot_out = DELTAE_dot_sys

DELTAE_dot_sys = 0 "Steady-flow assumption"

E_dot_in = m_dot*h_in

E_dot_out =Q_dot_out+m_dot*h_out

h_in = C_P * T_1

h_out = C_P * T_2

| D [m] | Qout [kW] |
| --- | --- |
| 0.015 | 1.149 |
| 0.025 | 3.191 |
| 0.035 | 6.254 |
| 0.045 | 10.34 |
| 0.055 | 15.44 |
| 0.065 | 21.57 |
| 0.075 | 28.72 |

6-111 A room is to be heated by an electric resistance heater placed in a duct in the room. The power rating of the electric heater and the temperature rise of air as it passes through the heater are to be determined.

Assumptions 1 Steady operating conditions exist. 2 Air is an ideal gas with constant specific heats at room temperature. 3 Kinetic and potential energy changes are negligible. 4 The heating duct is adiabatic, and thus heat transfer through it is negligible. 5 No air leaks in and out of the room.

Properties The gas constant of air is 0.287 kPa.m3/kg.K (Table A-1). The specific heats of air at room temperature are cp = 1.005 and cv = 0.718 kJ/kg·K  (Table A-2).

Analysis (a) The total mass of air in the room is

568 m3

We

200 W

200 kJ/min

V=5×6×8  m3=240  m3m=P1VRT1=(98  kPa)(240  m3)(0.287  kPa⋅m3/kg⋅K)(288  K)=284.6  kg

We first take the entire room as our system, which is a closed system since no mass leaks in or out. The power rating of the electric heater is determined by applying the conservation of energy relation to this constant volume closed system:

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesWe,in+Wfan,in−Qout=ΔU    (since ΔKE=ΔPE=0) ΔtWe,in+Wfan,in−Qout=mcv,avgT2−T1

Solving for the electrical work input gives

We,in=Qout−Wfan,in+mcv(T2−T1)Δt=(200/60  kJ/s)−(0.2  kJ/s)+(284.6  kg)(0.718  kJ/kg⋅C)(25−15)C/(15×60  s)5.40   kW

(b) We now take the heating duct as the system, which is a control volume since mass crosses the boundary. There is only one inlet and one exit, and thus . The energy balance for this adiabatic steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutWe,in+Wfan,in+mh1=mh2   (since   {Q=Δke≃Δpe≃0)We,in+Wfan,in=m(h2−h1)=mcp(T2−T1)

Thus,

ΔT=T2−T1=We,in+Wfan,inmcp=(5.40+0.2) kJ/s50/60  kg/s1.005  kJ/kg⋅K=6.7C

6-112 A house is heated by an electric resistance heater placed in a duct. The power rating of the electric heater is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Air is an ideal gas with constant specific heats at room temperature. 3 Kinetic and potential energy changes are negligible.

Properties The constant pressure specific heat of air at room temperature is cp = 1.005 kJ/kg·K (Table A-2)

We

300 W

300 W

Analysis We take the heating duct as the system, which is a control volume since mass crosses the boundary. There is only one inlet and one exit, and thus . The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutWe,in+Wfan,in+mh1=Qout+mh2   (since  Δke≃Δpe≃0)We,in+Wfan,in=Qout+m(h2−h1)=Qout+mcp(T2−T1)

Substituting, the power rating of the heating element is determined to be

We,in=Qout+mcpΔT−Wfan,in=(0.3  kJ/s)+(0.6  kg/s)(1.005  kJ/kg⋅°C)(7°C)−0.3  kW=4.22   kW

6-113 A hair dryer consumes 1200 W of electric power when running. The inlet volume flow rate and the exit velocity of air are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Air is an ideal gas with constant specific heats at room temperature. 3 Kinetic and potential energy changes are negligible. 4 The power consumed by the fan and the heat losses are negligible.

Properties The gas constant of air is 0.287 kPa.m3/kg.K (Table A-1). The constant pressure specific heat of air at room temperature is cp = 1.005 kJ/kg·K (Table A-2)

Analysis We take the hair dryer as the system, which is a control volume since mass crosses the boundary. There is only one inlet and one exit, and thus . The energy balance for this steady-flow system can be expressed in the rate form as

T2 = 47C

A2 = 60 cm2

P1 = 100 kPa

T1 = 22C

We = 1200 W

·

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutWe,in+mh1=mh2   (since   {Qout≃Δke≃Δpe≃0)We,in=m(h2−h1)=mcp(T2−T1)

Substituting, the mass and volume flow rates of air are determined to be

m=We,incpT2−T1=1.2 kJ/s1.005  kJ/kg⋅C47−22C=0.04776  kg/s

v1=RT1P1=0.287  kPa⋅m3/kg⋅K295  K100  kPa=0.8467  m3/kgV1=mv1=0.04776  kg/s0.8467  m3/kg=0.0404 m3/s

(b) The exit velocity of air is determined from the mass balance  to be

v2=RT2P2=(0.287  kPa⋅m3/kg⋅K)(320 K)(100  kPa)=0.9184  m3/kgm=1v2A2V2  V2=mv2A2=(0.04776  kg/s)(0.9184  m3/kg)60×10−4 m2=7.31   m/s

6-114 EES Problem 6-113 is reconsidered. The effect of the exit cross-sectional area of the hair drier on the exit velocity as the exit area varies from 25 cm2 to 75 cm2 is to be investigated. The exit velocity is to be plotted against the exit cross-sectional area.

Analysis The problem is solved using EES, and the results are tabulated and plotted below.

"Knowns:"

R=0.287 [kPa-m^3/kg-K]

P= 100 [kPa]

T_1 = 22 [C]

T_2= 47 [C]

{A_2 = 60 [cm^2]}

A_1 = 53.35 [cm^2]

W_dot_ele=1200 [W]

"Analysis:

We take the hair dryer as the system, which is a control volume since mass crosses the boundary. There is only one inlet and one exit.  Thus, the energy balance for this steady-flow system can be expressed in the rate form as:"

E_dot_in = E_dot_out

E_dot_in = W_dot_ele*convert(W,kW) + m_dot_1*(h_1+Vel_1^2/2*convert(m^2/s^2,kJ/kg))

E_dot_out = m_dot_2*(h_2+Vel_2^2/2*convert(m^2/s^2,kJ/kg))

h_2  = enthalpy(air, T = T_2)

h_1= enthalpy(air, T = T_1)

"The volume flow rates of air are determined to be:"

V_dot_1 = m_dot_1*v_1

P*v_1=R*(T_1+273)

V_dot_2 = m_dot_2*v_2

P*v_2=R*(T_2+273)

m_dot_1 = m_dot_2

Vel_1=V_dot_1/(A_1*convert(cm^2,m^2))

"(b) The exit velocity of air is determined from the mass balance  to be"

Vel_2=V_dot_2/(A_2*convert(cm^2,m^2))

| A2 [cm2] | Vel2 [m/s] |
| --- | --- |
| 25 | 16 |
| 30 | 13.75 |
| 35 | 12.03 |
| 40 | 10.68 |
| 45 | 9.583 |
| 50 | 8.688 |
| 55 | 7.941 |
| 60 | 7.31 |
| 65 | 6.77 |
| 70 | 6.303 |
| 75 | 5.896 |

6-115 The ducts of a heating system pass through an unheated area. The rate of heat loss from the air in the ducts is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Air is an ideal gas with constant specific heats at room temperature. 3 Kinetic and potential energy changes are negligible. 4 There are no work interactions involved.

Properties The constant pressure specific heat of air at room temperature is cp = 1.005 kJ/kg·K (Table A-2)

Analysis We take the heating duct as the system, which is a control volume since mass crosses the boundary. There is only one inlet and one exit, and thus . The energy balance for this steady-flow system can be expressed in the rate form as

120 kg/min

AIR

Q

·

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutmh1=Qout+mh2     (since   {W≃Δke≃Δpe≃0)Qout=m(h1−h2)=mcp(T1−T2)

Substituting,

Qout=(120  kg/min)(1.005  kJ/kg⋅C)(4C)=482 kJ/min

6-116 Steam pipes pass through an unheated area, and the temperature of steam drops as a result of heat losses. The mass flow rate of steam and the rate of heat loss from are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time.  2 Kinetic and potential energy changes are negligible. 4 There are no work interactions involved.

Properties From the steam tables (Table A-6),

1 MPa

300C

STEAM

Q

·

800 kPa

250C

P1=1  MPa  v1=0.25799  m3/kgh1=3051.6  kJ/kgP2=800  kPa  h2=2950.4  kJ/kg

Analysis (a) The mass flow rate of steam is determined directly from

m=1v1A1V1=10.25799  m3/kgπ0.06  m22  m/s=0.0877  kg/s

(b) We take the steam pipe as the system, which is a control volume since mass crosses the boundary. There is only one inlet and one exit, and thus . The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutmh1=Qout+mh2     (since   {W≃Δke≃Δpe≃0)Qout=m(h1−h2)

Substituting, the rate of heat loss is determined to be

Qloss=(0.0877  kg/s)(3051.6-2950.4)  kJ/kg=8.87  kJ/s

6-117 Steam flows through a non-constant cross-section pipe. The inlet and exit velocities of the steam are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy change is negligible. 3 There are no work interactions. 4 The device is adiabatic and thus heat transfer is negligible.

D1

200 kPa

200C

V1

D2

150 kPa

150C

V2

Steam

Analysis We take the pipe as the system, which is a control volume since mass crosses the boundary. The mass and energy balances for this steady-flow system can be expressed in the rate form as

Mass balance:

min−mout=Δmsystemä0  (steady)=0  min=moutA1V1v1=A1V1v1πD124V1v1=πD224V2v2

Energy balance:

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eouth1+V122=h2+V222    (since   {Q≃W≃Δke≃Δpe≃0)

The properties of steam at the inlet and exit are (Table A-6)

P1=200  kPa  v1=1.0805 m3/kgh1=2870.7 kJ/kg

P2=150  kPa  v2=1.2855 m3/kgh2=2772.9 kJ/kg

Assuming inlet diameter to be 1.8 m and the exit diameter to be 1.0 m, and substituting,

π(1.8 m)24V1(1.0805 m3/kg)=π(1.0 m)24V2(1.2855 m3/kg)			(1)

2870.7 kJ/kg+V1221  kJ/kg1000  m2/s2=2772.9  kJ/kg+V2221  kJ/kg1000  m2/s2	(2)

There are two equations and two unknowns. Solving equations (1) and (2) simultaneously using an equation solver such as EES, the velocities are determined to be

V1 = 118.8 m/s

V2 = 458.0 m/s

6-118E Saturated liquid water is heated in a steam boiler. The heat transfer per unit mass is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 There are no work interactions.

Analysis We take the pipe in which the water is heated as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

500 psia

600F

Water

500 psia,

sat. liq.

qin

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout mh1+Qin=mh2Qin=m(h2−h1)qin=h2−h1

The enthalpies of water at the inlet and exit of the boiler are (Table A-5E, A-6E).

P1=500 psia  h1≃hf@500 psia=449.51 Btu/lbmP2=500 psia  h2=1298.6 Btu/lbm

Substituting,

qin=1298.6−449.51=849.1 Btu/lbm

6-119 R-134a is condensed in a condenser. The heat transfer per unit mass is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 There are no work interactions.

Analysis We take the pipe in which R-134a is condensed as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

1200 kPa

sat. liq.

R134a

1200 kPa

80C

qout

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout mh1=mh2+QoutQout=m(h1−h2)qout=h1−h2

The enthalpies of R-134a at the inlet and exit of the condenser are (Table A-12, A-13).

P1=1200 kPa  h1=311.39 kJ/kgP2=1200 kPa  h2≃hf@1200 kPa=117.77 kJ/kg

Substituting,

qout=311.39−117.77=193.6 kJ/kg

6-120 Water is heated at constant pressure so that it changes a state from saturated liquid to saturated vapor. The heat transfer per unit mass is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 There are no work interactions.

Analysis We take the pipe in which water is heated as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

800 kPa

sat. vap.

Water

800 kPa

Sat. Liq.

qin

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout mh1+Qin=mh2Qin=m(h2−h1)qin=h2−h1=hfg

where	hfg@800 kPa=2047.5 kJ/kg	(Table A-5)

Thus,

qin=2047.5 kJ/kg

Charging and Discharging Processes

6-121 Helium flows from a supply line to an initially evacuated tank. The flow work of the helium in the supply line and the final temperature of the helium in the tank are to be determined.

Properties The properties of helium are R = 2.0769 kJ/kg.K, cp = 5.1926 kJ/kg.K, cv = 3.1156 kJ/kg.K (Table A-2a).

Helium

Initially

evacuated

200 kPa, 120C

Analysis The flow work is determined from its definition but we first determine the specific volume

v=RTlineP=(2.0769 kJ/kg.K)(120+273 K)(200 kPa)=4.0811 m3/kg

wflow=Pv=(200 kPa)(4.0811  m3/kg)=816.2 kJ/kg

Noting that the flow work in the supply line is converted to sensible internal energy in the tank, the final helium temperature in the tank is determined as follows

utank=hlinehline=cpTline=(5.1926 kJ/kg.K)(120+273 K)=2040.7 kJ/kgu-tank=cvTtank2040.7 kJ/kg=(3.1156 kJ/kg.K)TtankTtank=655.0 K

Alternative Solution: Noting the definition of specific heat ratio, the final temperature in the tank can also be determined from

Ttank=kTline=1.667(120+273 K)=655.1 K

which is practically the same result.

6-122 An evacuated bottle is surrounded by atmospheric air. A valve is opened, and air is allowed to fill the bottle. The amount of heat transfer through the wall of the bottle when thermal and mechanical equilibrium is established is to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid at the inlet remains constant. 2 Air is an ideal gas with variable specific heats. 3 Kinetic and potential energies are negligible. 4 There are no work interactions involved. 5 The direction of heat transfer is to the air in the bottle (will be verified).

Properties The gas constant of air is 0.287 kPa.m3/kg.K (Table A-1).

Analysis We take the bottle as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

8 L

Evacuated

100 kPa

17C

Mass balance:

min−mout=Δmsystem   →    mi=m2    (since mout=minitial=0)

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin+mihi=m2u2  (since W≃Eout=Einitial=ke≃pe≃0)

Combining the two balances:

Qin=m2u2−hi

where

m2=P2VRT2=(100  kPa)(0.008  m3)(0.287  kPa⋅m3/kg⋅K)(290  K)=0.0096  kgTi=T2=290  K Table A-17 hi=290.16  kJ/kgu2=206.91  kJ/kg

Substituting,

Qin = (0.0096 kg)(206.91 - 290.16) kJ/kg = - 0.8 kJ

or

Qout = 0.8 kJ

Discussion The negative sign for heat transfer indicates that the assumed direction is wrong. Therefore, we reverse the direction.

6-123 An insulated rigid tank is evacuated. A valve is opened, and air is allowed to fill the tank until mechanical equilibrium is established. The final temperature in the tank is to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid at the inlet remains constant. 2 Air is an ideal gas with constant specific heats. 3 Kinetic and potential energies are negligible. 4 There are no work interactions involved. 5 The device is adiabatic and thus heat transfer is negligible.

Properties The specific heat ratio for air at room temperature is  k = 1.4 (Table A-2).

initially evacuated

Air

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:

min−mout=Δmsystem   →    mi=m2    (since mout=minitial=0)

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesmihi=m2u2  (since Q≃W≃Eout=Einitial=ke≃pe≃0)

Combining the two balances:

u2=hi → cvT2=cpTi → T2=(cpcv)Ti=kTi

Substituting,

6-124 A rigid tank initially contains air at atmospheric conditions. The tank is connected to a supply line, and air is allowed to enter the tank until mechanical equilibrium is established. The mass of air that entered and the amount of heat transfer are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid at the inlet remains constant. 2 Air is an ideal gas with variable specific heats. 3 Kinetic and potential energies are negligible. 4 There are no work interactions involved. 5 The direction of heat transfer is to the tank (will be verified).

Properties The gas constant of air is 0.287 kPa.m3/kg.K (Table A-1). The properties of air are (Table A-21)

Ti=295 K  hi=295.17 kJ/kg1122

Analysis (a) We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Pi = 600 kPa

Ti = 22C

V1 = 2 m3

P1 = 100 kPa

T1 = 22C

Q

·

Mass balance:

min−mout=Δmsystem   →    mi=m2−m1

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin+mihi=m2u2−m1u1  (since  W≃ke≃pe≃0)

The initial and the final masses in the tank are

m1=P1VRT1=(100  kPa)(2  m3)(0.287  kPa⋅m3/kg⋅K)(295  K)=2.362  kgm2=P2VRT2=(600  kPa)(2  m3)(0.287  kPa⋅m3/kg⋅K)(350  K)=11.946  kg

Then from the mass balance,

(b) The heat transfer during this process is determined from

Qin=−mihi+m2u2−m1u1−9.584  kg295.17  kJ/kg+11.946  kg250.02  kJ/kg−2.362  kg210.49  kJ/kg−339  kJ   →   Qout=339 kJ

Discussion The negative sign for heat transfer indicates that the assumed direction is wrong. Therefore, we reversed the direction.

6-125 A rigid tank initially contains saturated R-134a liquid-vapor mixture. The tank is connected to a supply line, and R-134a is allowed to enter the tank. The final temperature in the tank, the mass of R-134a that entered, and the heat transfer are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid at the inlet remains constant.  2 Kinetic and potential energies are negligible. 3 There are no work interactions involved. 4 The direction of heat transfer is to the tank (will be verified).

Properties The properties of refrigerant are (Tables A-11 through A-13)

1 MPa

100C

0.2 m3

R-134a

R-134a

T1=8°C   v1=vf+x1vfg=0.0007887+0.7×0.052762−0.0007887=0.03717 m3/kgu1=uf+x1ufg=62.39+0.7×172.19=182.92  kJ/kg2sat.  vaporv2=vg @ 800  kPa=0.02562  m3/kgu2=ug @ 800  kPa=246.79  kJ/kgPi=1.0  MPa  hi=335.06  kJ/kg

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:        min−mout=Δmsystem   →    mi=m2−m1

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin+mihi=m2u2−m1u1  (since  W≃ke≃pe≃0)

(a) The tank contains saturated vapor at the final state at 800 kPa, and thus the final temperature is the saturation temperature at this pressure,

T2=Tsat  @  800  kPa=31.31ºC

(b)  The initial and the final masses in the tank are

m1=Vv1=0.2  m30.03717  m3/kg=5.38  kgm2=Vv2=0.2  m30.02562  m3/kg=7.81  kg

Then from the mass balance

mi=m2−m1=7.81−5.38=2.43 kg

(c)  The heat transfer during this process is determined from the energy balance to be

Qin=−mihi+m2u2−m1u1−2.43  kg335.06  kJ/kg+7.81  kg246.79  kJ/kg −5.38  kg182.92  kJ/kg130  kJ

6-126 A cylinder initially contains saturated liquid-vapor mixture of water. The cylinder is connected to a supply line, and the steam is allowed to enter the cylinder until all the liquid is vaporized. The final temperature in the cylinder and the mass of the steam that entered are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid at the inlet remains constant.  2 The expansion process is quasi-equilibrium.  3 Kinetic and potential energies are negligible. 3 There are no work interactions involved other than boundary work. 4 The device is insulated and thus heat transfer is negligible.

(P = 200 kPa)

m1 = 10 kg

H2O

Pi = 0.5 MPa

Ti = 350C

Properties The properties of steam are (Tables A-4 through A-6)

P1=200  kPa   h1=hf+x1hfg=504.71+0.6×2201.6=1825.6  kJ/kg2sat.  vapor2g @ 200  kPaPi=0.5  MPa  hi=3168.1  kJ/kg

Analysis (a) The cylinder contains saturated vapor at the final state at a pressure of 200 kPa, thus the final temperature in the cylinder must be

T2 = Tsat @ 200 kPa = 120.2°C

(b) We take the cylinder as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:        min−mout=Δmsystem   →    mi=m2−m1

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesmihi=Wb,out+m2u2−m1u1  (since  Q≃ke≃pe≃0)

Combining the two relations gives	0=Wb,out−m2−m1hi+m2u2−m1u1

or,	                                           0=−m2−m1hi+m2h2−m1h1

since the boundary work and U combine into H for constant pressure expansion and compression processes. Solving for m2 and substituting,

m2=hi−h1hi−h2m1=3168.1−1825.6 kJ/kg3168.1−2706.3  kJ/kg10  kg=29.07  kg

Thus,

mi = m2 - m1 = 29.07 - 10 = 19.07 kg

6-127E A scuba diver's air tank is to be filled with air from a compressed air line. The temperature and mass in the tank at the final state are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid at the inlet remains constant. 2 Air is an ideal gas with constant specific heats. 3 Kinetic and potential energies are negligible. 4 There are no work interactions involved. 5 The tank is well-insulated, and thus there is no heat transfer.

Properties The gas constant of air is 0.3704 psia·ft3/lbm·R (Table A-1E). The specific heats of air at room temperature are cp = 0.240 Btu/lbm·R and cv = 0.171 Btu/lbm·R (Table A-2Ea).

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:

Air

120 psia, 100F

20 psia 70F

2 ft3

min−mout=Δmsystem   →    mi=m2−m1

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesmihi=m2u2−m1u1micpTi=m2cvT2−m1cvT1

Combining the two balances:

(m2−m1)cpTi=m2cvT2−m1cvT1

The initial and final masses are given by

m1=P1VRT1=(20 psia)(2 ft3)(0.3704 psia⋅ft3/lbm⋅R)(70+460 R)=0.2038 lbm

m2=P2VRT2=(120 psia)(2 ft3)(0.3704 psia⋅ft3/lbm⋅R)T2=647.9T2

Substituting,

647.9T2−0.2038(0.24)(560)=647.9T2(0.171)T2−(0.2038)(0.171)(530)

whose solution is

T2=727.4 R=267.4°F

The final mass is then

m2=647.9T2=647.9727.4=0.890 lbm

6-128 R-134a from a tank is discharged to an air-conditioning line in an isothermal process. The final quality of the R-134a in the tank and the total heat transfer are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid at the exit remains constant. 2 Kinetic and potential energies are negligible. 3 There are no work interactions involved.

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:

A-C line

Liquid R-134a

5 kg

24C

min−mout=Δmsystem    −me=m2−m1me=m1−m2

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin−mehe=m2u2−m1u1Qin=m2u2−m1u1+mehe

Combining the two balances:

Qin=m2u2−m1u1+(m1−m2)he

The initial state properties of R-134a in the tank are

T1=24°C  v1=0.0008261 m3/kgu1=84.44 kJ/kghe=84.98 kJ/kg	(Table A-11)

Note that we assumed that the refrigerant leaving the tank is at saturated liquid state, and found the exiting enthalpy accordingly. The volume of the tank is

V=m1v1=(5 kg)(0.0008261 m3/kg)=0.004131 m3

The final specific volume in the container is

v2=Vm2=0.004131 m30.25 kg=0.01652 m3/kg

The final state is now fixed. The properties at this state are (Table A-11)

T2=24°C  x2=v2−vfvfg=0.01652−0.00082610.031834−0.0008261=0.5061u2=uf+x2ufg=84.44 kJ/kg+(0.5061)(158.65 kJ/kg)=164.73 kJ/kg

Substituting into the energy balance equation,

Qin=m2u2−m1u1+(m1−m2)he(0.25 kg)(164.73 kJ/kg)−(5 kg)(84.44 kJ/kg)+(4.75 kg)(84.98 kJ/kg)22.64 kJ

6-129E Oxygen is supplied to a medical facility from 10 compressed oxygen tanks in an isothermal process. The mass of oxygen used and the total heat transfer to the tanks are to be determined.

Assumptions 1 This is an unsteady process but it can be analyzed as a uniform-flow process. 2 Oxygen is an ideal gas with constant specific heats. 3 Kinetic and potential energies are negligible. 4 There are no work interactions involved.

Properties The gas constant of oxygen is 0.3353 psia·ft3/lbm·R (Table A-1E). The specific heats of oxygen at room temperature are cp = 0.219 Btu/lbm·R and cv = 0.157 Btu/lbm·R (Table A-2Ea).

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:

Oxygen

2000 psia 80F, 30 ft3

min−mout=Δmsystem−me=m2−m1me=m1−m2

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin−mehe=m2u2−m1u1Qin=m2u2−m1u1+meheQin=m2cvT2−m1cvT1+mecpTe

Combining the two balances:

Qin=m2cvT2−m1cvT1+(m1−m2)cpTe

The initial and final masses, and the mass used are

m1=P1VRT1=(2000 psia)(30 ft3)(0.3353 psia⋅ft3/lbm⋅R)(80+460 R)=331.4 lbm

m2=P2VRT2=(100 psia)(30 ft3)(0.3353 psia⋅ft3/lbm⋅R)(80+460 R)=16.57 lbm

me=m1−m2=331.4−16.57=314.8 lbm

Substituting into the energy balance equation,

Qin=m2cvT2−m1cvT1+mecpTe(16.57)(0.157)(540)−(331.4)(0.157)(540)+(314.8)(0.219)(540)10,540 Btu

6-130 The air in an insulated, rigid compressed-air tank is released until the pressure in the tank reduces to a specified value. The final temperature of the air in the tank is to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process. 2 Air is an ideal gas with constant specific heats. 3 Kinetic and potential energies are negligible. 4 There are no work interactions involved. 5 The tank is well-insulated, and thus there is no heat transfer.

Properties The gas constant of air is 0.287 kPa·m3/kg·K (Table A-1). The specific heats of air at room temperature are cp = 1.005 kJ/kg·K and cv = 0.718 kJ/kg·K (Table A-2a).

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:

Air

4000 kPa 20C, 0.5 m3

min−mout=Δmsystem−me=m2−m1me=m1−m2

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energies−mehe=m2u2−m1u10=m2u2−m1u1+mehe0=m2cvT2−m1cvT1+mecpTe

Combining the two balances:

0=m2cvT2−m1cvT1+(m1−m2)cpTe

The initial and final masses are given by

m1=P1VRT1=(4000 kPa)(0.5 m3)(0.287 kPa⋅m3/kg⋅K)(20+273 K)=23.78 kg

m2=P2VRT2=(2000 kPa)(0.5 m3)(0.287 kPa⋅m3/kg⋅K)T2=3484T2

The temperature of air leaving the tank changes from the initial temperature in the tank to the final temperature during the discharging process. We assume that the temperature of the air leaving the tank is the average of initial and final temperatures in the tank. Substituting into the energy balance equation gives

0=m2cvT2−m1cvT1+(m1−m2)cpTe0=3484T2(0.718)T2−(23.78)(0.718)(293)+23.78−3484T2(1.005)293+T22

whose solution by trial-error or by an equation solver such as EES is

T2=241 K=−32°C

6-131E Steam is supplied from a line to a weighted piston-cylinder device. The final temperature (and quality if appropriate) of the steam in the piston cylinder and the total work produced as the device is filled are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid at the inlet remains constant. 2 Kinetic and potential energies are negligible. 3 The process is adiabatic.

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, and also noting that the initial mass in the system is zero, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:

min−mout=Δmsystem    mi=m2

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesmihi−Wb,out=m2u2Wb,out=mihi−m2u2

Combining the two balances:

Wb,out=m2(hi−u2)

The boundary work is determined from

Wb,out=P(V2−V1)=P(m2v2−m1v1)=Pm2v2

Substituting, the energy balance equation simplifies into

Pm2v2=m2(hi−u2)Pv2=hi−u2

The enthalpy of steam at the inlet is

P1=300 psia  hi=1226.4 Btu/lbm    (Table A-6E)

Substituting this value into the energy balance equation and using an iterative solution of this equation gives (or better yet using EES software)

T2=425.1°F u2=1135.5 Btu/lbmv2=2.4575 ft3/lbm

The final mass is

m2=V2v2=10 ft32.4575 ft3/lbm=4.069 lbm

and the work produced is

Wb,out=PV2=(200 psia)(10 ft3)1 Btu5.404 psia⋅ft3=370.1 Btu

6-132E Oxygen is supplied from a line to a weighted piston-cylinder device. The final temperature of the oxygen in the piston cylinder and the total work produced as the device is filled are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid at the inlet remains constant. 2 Kinetic and potential energies are negligible. 3 The process is adiabatic. 4 Oxygen is an ideal gas with constant specific heats.

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, and also noting that the initial mass in the system is zero, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:

min−mout=Δmsystem    mi=m2

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesmihi−Wb,out=m2u2Wb,out=mihi−m2u2

Combining the two balances:

Wb,out=m2(hi−u2)

The boundary work is determined from

Wb,out=P(V2−V1)=P(m2v2−m1v1)=Pm2v2

Substituting, the energy balance equation simplifies into

Pm2v2=m2(hi−u2)Pv2=hi−u2RT2=cpTi−cvT2

Solving for the final temperature,

RT2=cpTi−cvT2T2=cpR+cvTi=cpcpTi=Ti=450°F

The work produced is

Wb,out=PV2=(300 psia)(10 ft3)1 Btu5.404 psia⋅ft3=555 Btu

6-133 A rigid tank initially contains saturated R-134a vapor. The tank is connected to a supply line, and R-134a is allowed to enter the tank. The mass of the R-134a that entered and the heat transfer are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid at the inlet remains constant.  2 Kinetic and potential energies are negligible. 3 There are no work interactions involved. 4 The direction of heat transfer is to the tank (will be verified).

1.2 MPa

36C

R-134a

0.12 m3

1 MPa

Sat. vapor

R-134a

Q

Properties The properties of refrigerant are (Tables A-11 through A-13)

P1=1  MPa  v1=vg @ 1  MPa=0.02031  m3/kgu1=ug @ 1  MPa=250.68  kJ/kgP2=1.2  MPa  v2=vf @ 1.2  MPa=0.0008934  m3/kgu2=uf @ 1.2  MPa=116.70  kJ/kgPi=1.2  MPa  hi=hf @ 36°C=102.30  kJ/kg

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:

min−mout=Δmsystem   →    mi=m2−m1

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin+mihi=m2u2−m1u1  (since  W≃ke≃pe≃0)

(a)  The initial and the final masses in the tank are

m1=V1v1=0.12  m30.02031  m3/kg=5.91 kgm2=V2v2=0.12  m30.0008934  m3/kg=134.31  kg

Then from the mass balance

mi=m2−m1=134.31−5.91=128.4  kg

(c)  The heat transfer during this process is determined from the energy balance to be

Qin=−mihi+m2u2−m1u1−128.4  kg102.30  kJ/kg+134.31  kg116.70  kJ/kg−5.91  kg250.68  kJ/kg1057  kJ

6-134 A rigid tank initially contains saturated liquid water. A valve at the bottom of the tank is opened, and half of the mass in liquid form is withdrawn from the tank. The temperature in the tank is maintained constant.  The amount of heat transfer is to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid leaving the device remains constant.  2 Kinetic and potential energies are negligible. 3 There are no work interactions involved. 4 The direction of heat transfer is to the tank (will be verified).

Properties The properties of water are (Tables A-4 through A-6)

T1=200°C  v1=vf @ 200C=0.001157  m3/kgu1=uf @ 200C=850.46  kJ/kgesat. liquidef @ 200C

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

H2O

Sat. liquid

T = 200C

V = 0.3 m3

Q

Mass balance:        min−mout=Δmsystem   →    me=m1−m2

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin=mehe+m2u2−m1u1  (since  W≃ke≃pe≃0)

The initial and the final masses in the tank are

m1=V1v1=0.3  m30.001157 m3/kg=259.4  kgm2=12m1=12259.4  kg=129.7  kg

Then from the mass balance,

me=m1−m2=259.4−129.7=129.7  kg

Now we determine the final internal energy,

v2=Vm2=0.3  m3129.7  kg=0.002313  m3/kgx2=v2−vfvfg=0.002313−0.0011570.12721−0.001157=0.009171T2=200°C u2=uf+x2ufg=850.46+0.0091711743.7=866.46  kJ/kg

Then the heat transfer during this process is determined from the energy balance by substitution to be

Q=129.7  kg852.26  kJ/kg+129.7  kg866.46  kJ/kg−259.4  kg850.46  kJ/kg=2308  kJ

6-135 A rigid tank initially contains saturated liquid-vapor mixture of refrigerant-134a. A valve at the bottom of the tank is opened, and liquid is withdrawn from the tank at constant pressure until no liquid remains inside. The amount of heat transfer is to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid leaving the device remains constant.  2 Kinetic and potential energies are negligible. 3 There are no work interactions involved. 4 The direction of heat transfer is to the tank (will be verified).

R-134a

Sat. vapor

P = 800 kPa

V = 0.12 m3

Q

Properties The properties of R-134a are (Tables A-11 through A-13)

P1=800  kPa  → vf= 0.0008458  m3/kg, vg=0.025621  m3/kg                                uf= 94.79 kJ/kg,   ug=246.79 kJ/kgP2=800  kPa  v2=vg @ 800  kPa=0.025621  m3/kgu2=ug @ 800  kPa=246.79  kJ/kgPe=800  kPa  he=hf @ 800  kPa=95.47  kJ/kg

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:

min−mout=Δmsystem   →    me=m1−m2

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin=mehe+m2u2−m1u1  (since  W≃ke≃pe≃0)

The initial mass, initial internal energy, and final mass in the tank are

m1=mf+mg=Vfvf+Vgvg=0.12×0.25  m30.0008458  m3/kg+0.12×0.75  m30.025621  m3/kg=35.47+3.513=38.98  kgU1=m1u1=mfuf+mgug=35.4794.79+3.513246.79=4229.2  kJm2=Vv2=0.1 2 m30.025621  m3/kg=4.684 kg

Then from the mass and energy balances,

me=m1−m2=38.98−4.684=34.30  kg

Qin=34.30  kg95.47  kJ/kg+4.684  kg246.79  kJ/kg−4229  kJ=201.2   kJ

6-136E A rigid tank initially contains saturated liquid-vapor mixture of R-134a. A valve at the top of the tank is opened, and vapor is allowed to escape at constant pressure until all the liquid in the tank disappears.  The amount of heat transfer is to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid leaving the device remains constant.  2 Kinetic and potential energies are negligible. 3 There are no work interactions involved.

R-134a

Sat. vapor

P = 100 psia

V = 4 ft3

Q

Properties The properties of R-134a are (Tables A-11E through A-13E)

P1=100  psia  → vf=0.01332  ft3/lbm,  vg=0.4776 ft3/lbm                               uf=37.623 Btu/lbm,  ug=104.99 Btu/lbmP2=100  psia  v2=vg @ 100  psia=0.4776  ft3/lbmu2=ug @ 100  psia=104.99  Btu/lbmPe=100  psia  he=hg @ 100  psia=113.83  Btu/lbm

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:

min−mout=Δmsystem   →    me=m1−m2

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin−mehe=m2u2−m1u1  (since  W≃ke≃pe≃0)

The initial mass, initial internal energy, and final mass in the tank are

m1=mf+mg=Vfvf+Vgvg=4×0.2  ft30.01332  ft3/lbm+4×0.8  ft30.4776  ft3/lbm=60.04+6.70=66.74  lbmU1=m1u1=mfuf+mgug=60.0437.623+6.70104.99=2962  Btum2=Vv2=4  ft30.4776 ft3/lbm=8.375  lbm

Then from the mass and energy balances,

me=m1−m2=66.74−8.375=58.37  lbm

Qin=mehe+m2u2−m1u158.37  lbm113.83  Btu/lbm+8.375  lbm104.99  Btu/lbm−2962  Btu4561  Btu

6-137 A rigid tank initially contains superheated steam. A valve at the top of the tank is opened, and vapor is allowed to escape at constant pressure until the temperature rises to 500C.  The amount of heat transfer is to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process by using constant average properties for the steam leaving the tank.  2 Kinetic and potential energies are negligible. 3 There are no work interactions involved. 4 The direction of heat transfer is to the tank (will be verified).

STEAM

2 MPa

Q

Properties The properties of water are (Tables A-4 through A-6)

P1=2  MPa  v1=0.12551  m3/kgu1=2773.2  kJ/kg,h1=3024.2  kJ/kgP2=2  MPa  v2=0.17568  m3/kgu2=3116.9  kJ/kg,h2=3468.3  kJ/kg

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:

min−mout=Δmsystem   →    me=m1−m2

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin−mehe=m2u2−m1u1  (since  W≃ke≃pe≃0)

The state and thus the enthalpy of the steam leaving the tank is changing during this process.  But for simplicity, we assume constant properties for the exiting steam at the average values. Thus,

he≃h1+h22=3024.2+3468.3  kJ/kg2=3246.2  kJ/kg

The initial and the final masses in the tank are

m1=V1v1=0.2  m30.12551  m3/kg=1.594  kgm2=V2v2=0.2  m30.17568  m3/kg=1.138 kg

Then from the mass and energy balance relations,

me=m1−m2=1.594−1.138=0.456  kg

Qin=mehe+m2u2−m1u10.456  kg3246.2  kJ/kg+1.138  kg3116.9  kJ/kg−1.594  kg2773.2  kJ/kg606.8   kJ

6-138 Steam is supplied from a line to a piston-cylinder device equipped with a spring. The final temperature (and quality if appropriate) of the steam in the cylinder and the total work produced as the device is filled are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid at the inlet remains constant. 2 Kinetic and potential energies are negligible. 3 The process is adiabatic.

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, and also noting that the initial mass in the system is zero, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:  	min−mout=Δmsystem          mi=m2

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesmihi−Wb,out=m2u2Wb,out=mihi−m2u2

Combining the two balances:

Wb,out=m2(hi−u2)

Because of the spring, the relation between the pressure and volume is a linear relation. According to the data in the problem statement,

P−300=27005V

The final vapor volume is then

V2=52700(1500−300)=2.222 m3

The work needed to compress the spring is

Wb,out=PdV=0V227005V+300dV=27002×5V22+300V2=270×2.2222+300×2.222=2000 kJ

The enthalpy of steam at the inlet is

P1=1500 kPa  hi=2796.0 kJ/kg    (Table A-6)

Substituting the information found into the energy balance equation gives

Wb,out=m2(hi−u2)Wb,out=V2v2(hi−u2)2000=2.222v2(2796.0−u2)

Using an iterative solution of this equation with steam tables gives (or better yet using EES software)

T2=233.2°C u2=2664.8 kJ/kgv2=0.1458 m3/kg

6-139 Air is supplied from a line to a piston-cylinder device equipped with a spring. The final temperature of the steam in the cylinder and the total work produced as the device is filled are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid at the inlet remains constant. 2 Kinetic and potential energies are negligible. 3 The process is adiabatic. 4 Air is an ideal gas with constant specific heats.

Properties The gas constant of air is 0.287 kPa·m3/kg·K (Table A-1). The specific heats of air at room temperature are cp = 1.005 kJ/kg·K and cv = 0.718 kJ/kg·K (Table A-2a).

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, and also noting that the initial mass in the system is zero, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:  	min−mout=Δmsystem          mi=m2

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesmihi−Wb,out=m2u2Wb,out=mihi−m2u2

Combining the two balances:

Wb,out=m2(hi−u2)

Because of the spring, the relation between the pressure and volume is a linear relation. According to the data in the problem statement,

P−300=27005V

The final air volume is then

V2=52700(2000−300)=3.148 m3

The work needed to compress the spring is

Wb,out=PdV=0V227005V+300dV=27002×5V22+300V2=270×3.1482+300×3.148=3620 kJ

Substituting the information found into the energy balance equation gives

Wb,out=m2(hi−u2)Wb,out=P2V2RT2(cpTi−cvT2)3620=2000×3.148(0.287)T2(1.005×523−0.718×T2)

The final temperature is then

T2=595.2 K=322.2°C

6-140 A hot-air balloon is considered. The final volume of the balloon and work produced by the air inside the balloon as it expands the balloon skin are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process. 2 Air is an ideal gas with constant specific heats. 3 Kinetic and potential energies are negligible. 4 There is no heat transfer.

Properties The gas constant of air is 0.287 kPa·m3/kg·K (Table A-1).

Analysis The specific volume of the air at the entrance and exit, and in the balloon is

v=RTP=(0.287 kPa⋅m3/kg⋅K)(35+273 K)100 kPa=0.8840 m3/kg

The mass flow rate at the entrance is then

mi=AiViv=(1 m2)(2 m/s)0.8840 m3/kg=2.262 kg/s

while that at the outlet is

me=AeVev=(0.5 m2)(1 m/s)0.8840 m3/kg=0.5656 kg/s

Applying a mass balance to the balloon,

min−mout=Δmsystemmi−me=m2−m1m2−m1=(mi−me)Δt=[(2.262−0.5656) kg/s](2×60 s)=203.6 kg

The volume in the balloon then changes by the amount

ΔV=(m2−m1)v=(203.6 kg)(0.8840 m3/kg)=180 m3

and the final volume of the balloon is

V2=V1+ΔV=75+180=255 m3

In order to push back the boundary of the balloon against the surrounding atmosphere, the amount of work that must be done is

Wb,out=PΔV=(100 kPa)(180 m3)1 kJ1 kPa⋅m3=18,000 kJ

6-141 An insulated rigid tank initially contains helium gas at high pressure. A valve is opened, and half of the mass of helium is allowed to escape. The final temperature and pressure in the tank are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process by using constant average properties for the helium leaving the tank.  2 Kinetic and potential energies are negligible. 3 There are no work interactions involved. 4 The tank is insulated and thus heat transfer is negligible. 5 Helium is an ideal gas with constant specific heats.

Properties The specific heat ratio of helium is k =1.667 (Table A-2).

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

He

0.08 m3

2 MPa

80C

Mass balance:        min−mout=Δmsystem   →    me=m1−m2

m2=12m1 (given)  me=m2=12m1

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energies−mehe=m2u2−m1u1  (since  W≃Q≃ke≃pe≃0)

Note that the state and thus the enthalpy of helium leaving the tank is changing during this process.  But for simplicity, we assume constant properties for the exiting steam at the average values.

Combining the mass and energy balances:

Dividing by m1/2		0=he+u2−2u1 or 0=cpT1+T22+cvT2−2cvT1

Dividing by cv:		0=kT1+T2+2T2−4T1 since k=cpcv

Solving for T2:		T2=4−k2+kT1=4−1.6672+1.667353  K=225  K

The final pressure in the tank is

P1VP2V=m1RT1m2RT2  P2=m2T2m1T2P1=122253532000  kPa=637  kPa

6-142E An insulated rigid tank equipped with an electric heater initially contains pressurized air. A valve is opened, and air is allowed to escape at constant temperature until the pressure inside drops to 30 psia. The  amount of electrical work transferred is to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the exit temperature (and enthalpy) of air remains constant.  2 Kinetic and potential energies are negligible. 3 The tank is insulated and thus heat transfer is negligible. 4 Air is an ideal gas with variable specific heats.

Properties The gas constant of air is R =0.3704 psia.ft3/lbm.R (Table A-1E). The properties of air are (Table A-21E)

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

AIR

60 ft3

75 psia

120F

We

Mass balance:

min−mout=Δmsystem   →    me=m1−m2

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesWe,in−mehe=m2u2−m1u1  (since Q≃ke≃pe≃0)

The initial and the final masses of air in the tank are

m1=P1VRT1=75  psia60  ft30.3704  psia⋅ft3/lbm⋅R580  R=20.95  lbmm2=P2VRT2=30  psia60  ft30.3704  psia⋅ft3/lbm⋅R580  R=8.38  lbm

Then from the mass and energy balances,

We,in=mehe+m2u2−m1u112.57  lbm138.66  Btu/lbm+8.38  lbm98.90  Btu/lbm−20.95  lbm98.90  Btu/lbm500   Btu

6-143 A vertical cylinder initially contains air at room temperature. Now a valve is opened, and air is allowed to escape at constant pressure and temperature until the volume of the cylinder goes down by half. The  amount air that left the cylinder and the amount of heat transfer are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the exit temperature (and enthalpy) of air remains constant.  2 Kinetic and potential energies are negligible. 3 There are no work interactions other than boundary work. 4 Air is an ideal gas with constant specific heats. 5 The direction of heat transfer is to the cylinder (will be verified).

Properties The gas constant of air is R = 0.287 kPa.m3/kg.K (Table A-1).

Analysis (a) We take the cylinder as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:

AIR

300 kPa

0.2 m3

20C

min−mout=Δmsystem   →    me=m1−m2

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin+Wb,in−mehe=m2u2−m1u1  (since  ke≃pe≃0)

The initial and the final masses of air in the cylinder are

m1=P1V1RT1=300  kPa0.2  m30.287  kPa⋅m3/kg⋅K293  K=0.714  kgm2=P2V2RT2=300  kPa0.1  m30.287  kPa⋅m3/kg⋅K293  K=0.357  kg=12m1

Then from the mass balance,

(b) This is a constant pressure process, and thus the Wb and the U terms can be combined into H to yield

Noting that the temperature of the air remains constant during this process, we have

hi = h1 = h2 = h.

Also,

me=m2=12m1.

Thus,

Q=12m1+12m1−m1h=0

6-144 A balloon is initially filled with helium gas at atmospheric conditions. The tank is connected to a supply line, and helium is allowed to enter the balloon until the pressure rises from 100 to 150 kPa. The final temperature in the balloon is to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid at the inlet remains constant. 2 Helium is an ideal gas with constant specific heats. 3 The expansion process is quasi-equilibrium.  4 Kinetic and potential energies are negligible. 5 There are no work interactions involved other than boundary work. 6 Heat transfer is negligible.

Properties The gas constant of helium is R = 2.0769 kJ/kg·K (Table A-1). The specific heats of helium are cp = 5.1926 and cv = 3.1156 kJ/kg·K (Table A-2a).

Analysis  We take the cylinder as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:

min−mout=Δmsystem   →    mi=m2−m1

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesmihi=Wb,out+m2u2−m1u1  (since Q≃ke≃pe≃0)

m1=P1V1RT1=100  kPa65  m32.0769  kPa⋅m3/kg⋅K295  K=10.61 kgP1P2=V1V2  V2=P2P1V1=150  kPa100  kPa65  m3=97.5  m3m2=P2V2RT2=150  kPa97.5  m32.0769  kPa⋅m3/kg⋅KT2 K=7041.74T2 kg

Then from the mass balance,

Noting that P varies linearly with V, the boundary work done during this process is

Wb=P1+P22V2−V1=100+150 kPa297.5−65 m3=4062.5  kJ

Using specific heats, the energy balance relation reduces to

Wb,out=micpTi−m2cvT2+m1cvT1

Substituting,

4062.5=7041.74T2−10.615.1926298−7041.74T23.1156T2+10.613.1156295

It yields		T2 = 333.6 K

6-145 An insulated piston-cylinder device with a linear spring is applying force to the piston. A valve at the bottom of the cylinder is opened, and refrigerant is allowed to escape. The amount of refrigerant that escapes and the final temperature of the refrigerant are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process assuming that the state of fluid leaving the device remains constant. 2 Kinetic and potential energies are negligible.

Properties The initial properties of R-134a are (Tables A-11 through A-13)

P1=1.2 MPa  v1=0.02423 m3/kgu1=325.03 kJ/kgh1=354.11 kJ/kg

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

R-134a

0.8 m3

1.2 MPa

120C

Mass balance:        min−mout=Δmsystem   →    me=m1−m2

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesWb,in−mehe=m2u2−m1u1  (since  Q≃ke≃pe≃0)

The initial mass and the relations for the final and exiting masses are

m1=V1v1=0.8  m30.02423  m3/kg=33.02 kgm2=V2v2=0.5  m3v2me=m1−m2=33.02−0.5  m3v2

Noting that the spring is linear, the boundary work can be determined from

Wb,in=P1+P22(V1−V2)=(1200+600) kPa2(0.8-0.5) m3=270 kJ

Substituting the energy balance,

270−33.02−0.5 m3v2he=0.5 m3v2u2−(33.02 kg)(325.03 kJ/kg)	(Eq. 1)

where the enthalpy of exiting fluid is assumed to be the average of initial and final enthalpies of the refrigerant in the cylinder. That is,

he=h1+h22=(354.11 kJ/kg)+h22

Final state properties of the refrigerant (h2, u2, and v2) are all functions of final pressure (known) and temperature (unknown). The solution may be obtained by a trial-error approach by trying different final state temperatures until Eq. (1) is satisfied. Or solving the above equations simultaneously using an equation solver with built-in thermodynamic functions such as EES, we obtain

T2 = 96.8C,  me = 22.47 kg, h2 = 336.20 kJ/kg,

u2 = 307.77 kJ/kg, v2 = 0.04739 m3/kg, m2 = 10.55 kg

6-146 Steam at a specified state is allowed to enter a piston-cylinder device in which steam undergoes a constant pressure expansion process. The amount of mass that enters and the amount of heat transfer are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid entering the device remains constant. 2 Kinetic and potential energies are negligible.

Properties The properties of steam at various states are (Tables A-4 through A-6)

Steam

0.6 kg

0.1 m3

800 kPa

Steam

5 MPa

500C

Q

v1=V1m1=0.1  m30.6 kg= 0.16667 m3/kgP2=P1

P1=800 kPa  u1=2004.4 kJ/kg

P2=800 kPa  v2=0.29321 m3/kgu2=2715.9 kJ/kg

Pi=5 MPa  hi=3434.7  kJ/kg

Analysis (a) We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:

min−mout=Δmsystem   →    mi=m2−m1

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin−Wb,out+mihi=m2u2−m1u1     (since ke≃pe≃0)

Noting that the pressure remains constant, the boundary work is determined from

Wb,out=P(V2−V1)=(800 kPa)(2×0.1−0.1) m3=80 kJ

The final mass and the mass that has entered are

m2=V2v2=0.2  m30.29321  m3/kg=0.682 kgmi=m2−m1=0.682−0.6=0.082 kg

(b) Finally, substituting into energy balance equation

Qin−80 kJ+(0.082 kg)(3434.7 kJ/kg)=(0.682 kg)(2715.9 kJ/kg)−(0.6 kg)(2004.4 kJ/kg)                                        Qin=447.9 kJ

Review Problems

6-147 A water tank open to the atmosphere is initially filled with water. The tank discharges to the atmosphere through a long pipe connected to a valve. The initial discharge velocity from the tank and the time required to empty the tank are to be determined.

Assumptions 1 The flow is incompressible. 2 The draining pipe is horizontal. 3 The tank is considered to be empty when the water level drops to the center of the valve.

Analysis (a) Substituting the known quantities, the discharge velocity can be expressed as

V=2gz1.5+fLD=2gz1.5+0.015(100 m)(0.10 m)=0.1212gz

D0

D

z

Then the initial discharge velocity becomes

V1=0.1212gz1=0.1212(9.81 m/s2)(2 m)=1.54  m/s

where z is the water height relative to the center of the orifice at that time.

(b) The flow rate of water from the tank can be obtained by multiplying the discharge velocity by the pipe cross-sectional area,

V=ApipeV2=πD240.1212gz

Then the amount of water that flows through the pipe during a differential time interval dt is

dV=Vdt=πD240.1212gzdt         		(1)

which, from conservation of mass, must be equal to the decrease in the volume of water in the tank,

dV=Atank(−dz)=−πD024dz			(2)

where dz is the change in the water level in the tank during dt. (Note that dz is a negative quantity since the positive direction of z is upwards. Therefore, we used –dz to get a positive quantity for the amount of water discharged). Setting Eqs. (1) and (2) equal to each other and rearranging,

πD240.1212gzdt=−πD024dz  →  dt=−D02D2dz0.1212gz=−D02D20.1212gz−12dz

The last relation can be integrated easily since the variables are separated. Letting tf be the discharge time and integrating it from t = 0 when z = z1 to t = tf when z = 0 (completely drained tank) gives

dt=−D02D20.1212gz−12dz  →   tf= -D02D20.1212gz1212z10=2D02D20.1212gz112

Simplifying and substituting the values given, the draining time is determined to be

tf=2D02D2z10.1212g=2(10 m)2(0.1 m)22 m0.1212(9.81 m/s2)=25,940 s = 7.21 h

Discussion The draining time can be shortened considerably by installing a pump in the pipe.

6-148 The rate of accumulation of water in a pool and the rate of discharge are given. The rate supply of water to the pool is to be determined.

Assumptions 1 Water is supplied and discharged steadily. 2 The rate of evaporation of water is negligible. 3 No water is supplied or removed through other means.

Analysis The conservation of mass principle applied to the pool requires that the rate of increase in the amount of water in the pool be equal to the difference between the rate of supply of water and the rate of discharge. That is,

dmpooldt=mi−me         →        {mi=dmpooldt+me          →           Vi=dVpooldt+Ve

since the density of water is constant and thus the conservation of mass is equivalent to conservation of volume. The rate of discharge of water is

Ve=AeVe=(πD2/4)Ve=[π(0.05 m)2/4](5 m/s)=0.00982 m3/s

The rate of accumulation of water in the pool is equal to the cross-section of the pool times the rate at which the water level rises,

dVpooldt=Across-sectionVlevel=(3 m×4 m)(0.015 m/min)=0.18 m3/min = 0.00300 m3/s

Substituting, the rate at which water is supplied to the pool is determined to be

Vi=dVpooldt+Ve=0.003+0.00982=0.01282 m3/s

Therefore, water is supplied at a rate of 0.01282 m3/s = 12.82 L/s.

6-149  A fluid is flowing in a circular pipe. A relation is to be obtained for the average fluid velocity in therms of V(r), R, and r.

Analysis Choosing a circular ring of area dA = 2rdr as our differential area, the mass flow rate through a cross-sectional area can be expressed as

R

r

dr

m=AρVr dA=ρVr2πr dr

Solving for Vavg,

Vavg=2R2Vrr dr

6-150 Air is accelerated in a nozzle. The density of air at the nozzle exit is to be determined.

Assumptions Flow through the nozzle is steady.

Properties The density of air is given to be 4.18 kg/m3 at the inlet.

AIR

1

2

Analysis  There is only one inlet and one exit, and thus . Then,

m1=m2ρ1A1V1=ρ2A2V2ρ2=A1A2V1V2ρ1=2120 m/s380 m/s(4.18 kg/m3)=2.64  kg/m3

Discussion Note that the density of air decreases considerably despite a decrease in the cross-sectional area of the nozzle.

6-151 An air compressor consumes 4.5 kW of power to compress a specified rate of air. The flow work required by the compressor is to be compared to the power used to increase the pressure of the air.

Assumptions 1 Flow through the compressor is steady. 2 Air is an ideal gas.

Compressor

1 MPa

300°C

120 kPa

20°C

Properties The gas constant of air is 0.287 kPa·m3/kg·K (Table A-1).

Analysis The specific volume of the air at the inlet is

v1=RT1P1=(0.287 kPa⋅m3/kg⋅K)(20+273 K)120 kPa=0.7008 m3/kg

The mass flow rate of the air is

m=V1v1=0.010 m3/s0.7008 m3/kg=0.01427 kg/s

Combining the flow work expression with the ideal gas equation of state gives the flow work as

wflow=P2v2−P1v1=R(T2−T1)=(0.287 kJ/kg⋅K)(300−20)K=80.36 kJ/kg

The flow power is

Wflow=mwflow=(0.01427 kg/s)(80.36 kJ/kg)=1.147 kW

The remainder of compressor power input is used to increase the pressure of the air:

W=Wtotal,in−Wflow=4.5−1.147=3.353 kW

6-152 Steam expands in a turbine whose power production is 9000 kW. The rate of heat lost from the turbine is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible.

Properties From the steam tables (Tables A-6 and A-4)

Turbine

1.6 MPa

350°C

16 kg/s

30°C

sat. vap.

Heat

P1=1.6 MPa  h1=3146.0 kJ/kgT2=30°C  h2=2555.6 kJkg

Analysis We take the turbine as the system, which is a control volume since mass crosses the boundary. Noting that there is one inlet and one exiti the energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m1h1=m2h2+Wout+QoutQout=m(h1−h2)−Wout

Substituting,

Qout=(16 kg/s)(3146.0−2555.6) kJ/kg−9000 kW=446.4 kW

6-153E Nitrogen gas flows through a long, constant-diameter adiabatic pipe. The velocities at the inlet and exit are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Nitrogen is an ideal gas with constant specific heats. 3 Potential energy changes are negligible. 4 There are no work interactions. 5 The re is no heat transfer from the nitrogen.

Properties The specific heat of nitrogen at the room temperature iss cp = 0.248 Btu/lbmR (Table A-2Ea).

Analysis There is only one inlet and one exit, and thus m1=m2=m. We take the pipe as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

50 psia

70F

N2

100 psia

120F

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=m(h2+V22/2)h1+V122=h2+V22/2V12−V222=cp(T2−T1)

Combining the mass balance and ideal gas equation of state yields

m1=m2A1V1v1=A2V2v2V2=A1A2v2v1V1=v2v1V1=T2T1P1P2V1

Substituting this expression for V2 into the energy balance equation gives

V1=2cp(T2−T1)1−T2T1P1P220.5=2(0.248)(70−120)1−53058010050225,037 ft2/s21 Btu/lbm0.5=515 ft/s

The velocity at the exit is

V2=T2T1P1P2V1=53058010050515=941 ft/s

6-154 Water at a specified rate is heated by an electrical heater. The current is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The heat losses from the water is negligible.

Properties The specific heat and the density of water are taken to be cp = 4.18 kJ/kg·°C and  = 1 kg/L (Table A-3).

Analysis We take the pipe in which water is heated as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

20C

water

15C

0.1 L/s

We,in

.

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout mh1+We,in=mh2We,in=m(h2−h1)VI=mcp(T2−T1)

The mass flow rate of the water is

m=ρV=(1 kg/L)(0.1 L/s)=0.1 kg/s

Substituting into the energy balance equation and solving for the current gives

I=mcp(T2−T1)V=(0.1 kg/s)(4.18 kJ/kg⋅K)(20−15)K110 V1000 VI1 kJ/s=19.0 A

6-155 Steam flows in an insulated pipe. The mass flow rate of the steam and the speed of the steam at the pipe outlet are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 There are no work and heat interactions.

Analysis We take the pipe in which steam flows as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

1000 kPa

D=0.1 m

Water

1400 kPa, 350C

D=0.15 m, 10 m/s

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout mh1=mh2h1=h2

The properties of the steam at the inlet and exit are (Table A-6)

P=1400 kPa   v1=0.20029 m3/kgh1=3150.1 kJ/kg

P2=1000 kPa     v2=0.28064 m3/kg

The mass flow rate is

m=A1V1v1=πD124V1v1=π(0.15 m)2410 m/s0.20029 m3/kg=0.8823 kg/s

The outlet velocity will then be

V2=mv2A2=4mv2πD22=4(0.8823 kg/s)(0.28064 m3/kg)π(0.10 m)2=31.53 m/s

6-156 The mass flow rate of a compressed air line is divided into two equal streams by a T-fitting in the line. The velocity of the air at the outlets and the rate of change of flow energy (flow power) across the T-fitting are to be determined.

Assumptions 1 Air is an ideal gas with constant specific heats. 2 The flow is steady. 3 Since the outlets are identical, it is presumed that the flow divides evenly between the two.

1.4 MPa

36°C

1.6 MPa

40°C

50 m/s

1.4 MPa

36°C

Properties The gas constant of air is R = 0.287 kPam3/kgK (Table A-1).

Analysis  The specific volumes of air at the inlet and outlets are

v1=RT1P1=(0.287 kPa⋅m3/kg⋅K)(40+273 K)1600 kPa=0.05614 m3/kg

v2=v3=RT2P2=(0.287 kPa⋅m3/kg⋅K)(36+273 K)1400 kPa=0.06335 m3/kg

Assuming an even division of the inlet flow rate, the mass balance can be written as

A1V1v1=2A2V2v2V2=V3=A1A2v2v1V12=0.063350.05614502=28.21 m/s

The mass flow rate at the inlet is

m1=A1V1v1=πD124V1v1=π(0.025 m)2450 m/s0.05614 m3/kg=0.4372 kg/s

while that at the outlets is

m2=m3=m12=0.4372 kg/s2=0.2186 kg/s

Substituting the above results into the flow power expression produces

Wflow=2m2P2v2−m1P1v12(0.2186 kg/s)(1400 kPa)(0.06335 m3/kg)−(0.4372 kg/s)(1600 kPa)(0.05614 m3/kg)−0.496 kW

6-157 Air flows through a non-constant cross-section pipe. The inlet and exit velocities of the air are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy change is negligible. 3 There are no work interactions. 4 The device is adiabatic and thus heat transfer is negligible. 5 Air is an ideal gas with constant specific heats.

Properties The gas constant of air is R = 0.287 kPa.m3/kg.K. Also, cp = 1.005 kJ/kg.K for air at room temperature (Table A-2)

D1

200 kPa

50C

V1

D2

150 kPa

40C

V2

Air

q

Analysis We take the pipe as the system, which is a control volume since mass crosses the boundary. The mass and energy balances for this steady-flow system can be expressed in the rate form as

Mass balance:

min−mout=Δmsystemä0  (steady)=0  min=moutρ1A1V1=ρ2A2V2P1RT1πD124V1=P2RT2πD224V2P1T1D12V1=P2T2D22V2

Energy balance:

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0     since  {W≃Δpe≃0)Ein=Eouth1+V122=h2+V222+qout

or	cpT1+V122=cpT2+V222+qout

Assuming inlet diameter to be 1.8 m and the exit diameter to be 1.0 m, and substituting given values into mass and energy balance equations

200 kPa323 K(1.8 m)2V1=150 kPa313 K(1.0 m)2V2		(1)

(1.005 kJ/kg.K)(323 K)+V1221 kJ/kg1000 m2/s2=(1.005 kJ/kg.K)(313 K)+V2221 kJ/kg1000 m2/s2+3.3 kJ/kg  (2)

There are two equations and two unknowns. Solving equations (1) and (2) simultaneously using an equation solver such as EES, the velocities are determined to be

V1 = 28.6 m/s

V2 = 120 m/s

6-158  Cold water enters a steam generator at 20C, and leaves as saturated vapor at Tsat = 150°C. The fraction of heat used to preheat the liquid water from 20C to saturation temperature of 150C is to be determined.

Assumptions  1 Steady operating conditions exist. 2 Heat losses from the steam generator are negligible. 3 The specific heat of water is constant at the average temperature.

Properties   The heat of vaporization of water at 150°C is hfg = 2113.8 kJ/kg (Table A-4), and the specific heat of liquid water is c = 4.18 kJ/kg.C (Table A-3).

Water

150C

Heater

Steam

Cold water

20C

Analysis The heat of vaporization of water represents the amount of heat needed to vaporize a unit mass of liquid at a specified temperature. Using the average specific heat, the amount of heat transfer needed to preheat a unit mass of water from 20C to 150C is

qpreheating=cΔT(4.18 kJ/kg⋅°C)(150−20)°C=543.4 kJ/kg

and

qtotal=qboiling+qpreheating2113.8+543.4=2657.2 kJ/kg

Therefore, the fraction of heat used to preheat the water is

Fraction to preheat=qpreheatingqtotal=543.42657.2=0.205 (or 20.5%)

6-159  Cold water enters a steam generator at 20C and is boiled, and leaves as saturated vapor at boiler pressure. The boiler pressure at which the amount of heat needed to preheat the water to saturation temperature that is equal to the heat of vaporization is to be determined.

Assumptions  Heat losses from the steam generator are negligible.

Properties  The enthalpy of liquid water at 20C is 83.91 kJ/kg. Other properties needed to solve this problem are the heat of vaporization hfg and the enthalpy of saturated liquid at the specified temperatures, and they can be obtained from Table A-4.

Analysis The heat of vaporization of water represents the amount of heat needed to vaporize a unit mass of liquid at a specified temperature, and h represents the amount of heat needed to preheat a unit mass of water from 20C to the saturation temperature. Therefore,

qpreheating=qboiling(hf@Tsat−hf@20°C)=hfg@Tsathf@Tsat−83.91 kJ/kg=hfg@Tsat  →   hf@Tsat−hfg@Tsat=83.91 kJ/kg

Water

Heater

Steam

Cold water

20C

The solution of this problem requires choosing a boiling temperature, reading hf and hfg at that temperature, and substituting the values into the relation above to see if it is satisfied. By trial and error,  (Table A-4)

At 310C:    1402.0 – 1325.9 = 76.1 kJ/kg

At 315C:    1431.6 – 1283.4 = 148.2 kJ/kg

The temperature that satisfies this condition is determined from the two values above by interpolation to be 310.6C. The saturation pressure corresponding to this temperature is 9.94 MPa.

6-160E A refrigeration system is to cool eggs by chilled air at a rate of 10,000 eggs per hour. The rate of heat removal from the eggs, the required volume flow rate of air, and the size of the compressor of the refrigeration system are to be determined.

Assumptions 1 Steady operating conditions exist. 2 The eggs are at uniform temperatures before and after cooling. 3 The cooling section is well-insulated. 4 The properties of eggs are constant. 5 The local atmospheric pressure is 1 atm.

Properties The properties of the eggs are given to  = 67.4 lbm/ft3 and cp = 0.80 Btu/lbm.F. The specific heat of air at room temperature cp = 0.24 Btu/lbm. F (Table A-2E). The gas constant of air is R = 0.3704 psia.ft3/lbm.R (Table A-1E).

Analysis   (a) Noting that eggs are cooled at a rate of 10,000 eggs per hour, eggs can be considered to flow steadily through the cooling section at a mass flow rate of

megg=(10,000  eggs/h)(0.14  lbm/egg)=1400  lbm/h = 0.3889  lbm/s

Air

34F

Egg

0.14 lbm

Taking the egg flow stream in the cooler as the system, the energy balance for steadily flowing eggs can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutmh1=Qout+mh2   (since Δke≃Δpe≃0)Qout=Qegg=meggcp(T1−T2)

Then the rate of heat removal from the eggs as they are cooled from 90F to 50ºF at this rate becomes

Qegg=(mcpΔT)egg=(1400 lbm/h)(0.80 Btu/lbm.°F)(90−50)°F=44,800Btu/h

(b) All the heat released by the eggs is absorbed by the refrigerated air since heat transfer through he walls of cooler is negligible, and the temperature rise of air is not to exceed 10F.  The minimum mass flow and volume flow rates of air are determined to be

mair=Qair(cpΔT)air=44,800 Btu/h(0.24Btu/lbm.°F)(10°F)=18,667 lbm/h

ρair=PRT=14.7 psia(0.3704 psia.ft3/lbm.R)(34+460)R=0.0803 lbm/ft3Vair=mairρair=18,667 lbm/h0.0803lbm/ft³=232,500ft³/h

6-161  Glass bottles are washed in hot water in an uncovered rectangular glass washing bath. The   rates of heat and water mass that need to be supplied to the water are to be determined.

Assumptions 1 Steady operating conditions exist. 2 The entire water body is maintained at a uniform temperature of 55C. 3 Heat losses from the outer surfaces of the bath are negligible. 4 Water is an incompressible substance with constant properties.

Properties The specific heat of water at room temperature is cp = 4.18 kJ/kg.C. Also, the specific heat of glass is 0.80 kJ/kg.C (Table A-3).

Analysis (a) The mass flow rate of glass bottles through the water bath in steady operation is

Water bath

55C

Taking the bottle flow section as the system, which is a steady-flow control volume, the energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since Δke≃Δpe≃0)Qin=Qbottle=mwatercp(T2−T1)

Then the rate of heat removal by the bottles as they are heated from 20 to 55C is

Qbottle=mbottlecpΔT=2kg/s0.8kJ/kg.ºC55−20ºC=56,000 W

The amount of water removed by the bottles is

mwater,out=Flow rate ofbottlesWater removed per bottle=800bottles/min0.2 g/bottle=160g/min=2.67×10−3kg/s

Noting that the water removed by the bottles is made up by fresh water entering

at 15C, the rate of heat removal by the water that sticks to the bottles is

Qwater removed=mwater removedcpΔT=(2.67×10−3kg/s)(4180 J/kg⋅ºC)(55−15)ºC=446W

Therefore, the total amount of heat removed by the wet bottles is

Qtotal, removed=Qglass removed+Qwater removed=56,000+446=56,446W

Discussion In practice, the rates of heat and water removal will be much larger since the heat losses from the tank and the moisture loss from the open surface are not considered.

6-162  Glass bottles are washed in hot water in an uncovered rectangular glass washing bath. The   rates of heat and water mass that need to be supplied to the water are to be determined.

Assumptions 1 Steady operating conditions exist. 2 The entire water body is maintained at a uniform temperature of 50C. 3 Heat losses from the outer surfaces of the bath are negligible. 4 Water is an incompressible substance with constant properties.

Properties The specific heat of water at room temperature is cp = 4.18 kJ/kg.C. Also, the specific heat of glass is 0.80 kJ/kg.C (Table A-3).

Analysis (a) The mass flow rate of glass bottles through the water bath in steady operation is

Water bath

50C

Taking the bottle flow section as the system, which is a steady-flow control volume, the energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since Δke≃Δpe≃0)Qin=Qbottle=mwatercp(T2−T1)

Then the rate of heat removal by the bottles as they are heated from 20 to 50C is

Qbottle=mbottlecpΔT=2kg/s0.8kJ/kg.ºC50−20ºC=48,000W

The amount of water removed by the bottles is

mwater,out=Flow rate ofbottlesWater removed perbottle=800bottles/min0.2g/bottle=160 g/min=2.67×10−3kg/s

Noting that the water removed by the bottles is made up by fresh water entering at 15C, the rate of heat removal by the water that sticks to the bottles is

Qwater removed=mwater removedcpΔT=(2.67×10−3kg/s)(4180 J/kg⋅ºC)(50−15)ºC=391W

Therefore, the total amount of heat removed by the wet bottles is

Qtotal, removed=Qglass removed+Qwater removed=48,000+391=48,391 W

Discussion In practice, the rates of heat and water removal will be much larger since the heat losses from the tank and the moisture loss from the open surface are not considered.

6-163 Long aluminum wires are extruded at a velocity of 10 m/min, and are exposed to atmospheric air. The rate of heat transfer from the wire is to be determined.

Assumptions 1 Steady operating conditions exist.  2 The thermal properties of the wire are constant.

Properties The properties of aluminum are given to be  = 2702 kg/m3 and cp = 0.896 kJ/kg.C.

Analysis  The mass flow rate of the extruded wire through the air is

m=ρV=ρ(πr02)V=(2702 kg/m3)π(0.0015 m)2(10 m/min)=0.191 kg/min

Taking the volume occupied by the extruded wire as the system, which is a steady-flow control volume, the energy balance for this steady-flow system can be expressed in the rate form as

350C

Aluminum wire

10 m/min

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutmh1=Qout+mh2   (since Δke≃Δpe≃0)Qout=Qwire=mwirecp(T1−T2)

Then the rate of heat transfer from the wire to the air becomes

Q=mcp[T(t)−T∞]=(0.191 kg/min)(0.896 kJ/kg.°C)(350−50)°C=51.3 kJ/min= 0.856 kW

6-164 Long copper wires are extruded at a velocity of 10 m/min, and are exposed to atmospheric air. The rate of heat transfer from the wire is to be determined.

Assumptions 1 Steady operating conditions exist.  2 The thermal properties of the wire are constant.

Properties The properties of copper are given to be  = 8950 kg/m3 and cp = 0.383 kJ/kg.C.

Analysis  The mass flow rate of the extruded wire through the air is

m=ρV=ρ(πr02)V=(8950 kg/m3)π(0.0015 m)2(10 m/min)=0.633 kg/min

Taking the volume occupied by the extruded wire as the system, which is a steady-flow control volume, the energy balance for this steady-flow system can be expressed in the rate form as

350C

Copper wire

10 m/min

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutmh1=Qout+mh2   (since Δke≃Δpe≃0)Qout=Qwire=mwirecp(T1−T2)

Then the rate of heat transfer from the wire to the air becomes

Q=mcp[T(t)−T∞]=(0.633 kg/min)(0.383 kJ/kg.°C)(350−50)°C=72.7 kJ/min= 1.21 kW

6-165 Steam at a saturation temperature of Tsat = 40°C condenses on the outside of a thin horizontal tube. Heat is transferred to the cooling water that enters the tube at 25°C and exits at 35°C. The rate of condensation of steam is to be determined.

Assumptions  1 Steady operating conditions exist. 2 Water is an incompressible substance with constant properties at room temperature. 3 The changes in kinetic and potential energies are negligible.

Properties   The properties of water at room temperature are  = 997 kg/m3 and cp = 4.18 kJ/kg.C (Table A-3). The enthalpy of vaporization of water at 40°C is hfg = 2406.0 kJ/kg (Table A-4).

Analysis The mass flow rate of water through the tube is

mwater=ρVAc=(997 kg/m3)(2 m/s)[π(0.03 m)24]=1.409 kg/s

Taking the volume occupied by the cold water in the tube as the system, which is a steady-flow control volume, the energy balance for this steady-flow system can be expressed in the rate form as

Cold Water

25C

Steam

40C

35C

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since Δke≃Δpe≃0)Qin=Qwater=mwatercp(T2−T1)

Then the rate of heat transfer to the water and the rate of condensation become

Q=mcp(Tout−Tin)=(1.409 kg/s)(4.18 kJ/kg⋅°C)(35−25)°C=58.9 kW

Q=mcondhfg  →   {mcond=Qhfg=58.9 kJ/s2406.0 kJ/kg=0.0245 kg/s

6-166 A fan is powered by a 0.5 hp motor, and delivers air at a rate of 85 m3/min. The highest possible air velocity at the fan exit is to be determined.

Assumptions  1 This is a steady-flow process since there is no change with time at any point and thus . 2 The inlet velocity and the change in potential energy are negligible, V1≃0  and  Δpe≃0.  3 There are no heat and work interactions other than the electrical power consumed by the fan motor. 4 The efficiencies of the motor and the fan are 100% since best possible operation is assumed. 5 Air is an ideal gas with constant specific heats at room temperature.

Properties  The density of air is given to be  = 1.18 kg/m3. The constant pressure specific heat of air at room temperature is cp = 1.005 kJ/kg.C (Table A-2).

Analysis We take the fan-motor assembly as the system. This is a control volume since mass crosses the system boundary during the process. We note that there is only one inlet and one exit, and thus .

The velocity of air leaving the fan will be highest when all of the entire electrical energy drawn by the motor is converted to kinetic energy, and the friction between the air layers is zero. In this best possible case, no energy will be converted to thermal energy, and thus the temperature change of air will be zero,  Then the energy balance for this steady-flow system can be expressed in the rate form as

0.5 hp

85 m3/min

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutWe,in+mh1=m(h2+V22/2)   (since V1≃0 and Δpe≃0)

Noting that the temperature and thus enthalpy remains constant, the relation above simplifies further to

We,in=mV22/2

where

m=ρV=(1.18 kg/m3)(85 m3/min)=100.3 kg/min=1.67 kg/s

Solving for V2 and substituting gives

V2=2We,inm=2(0.5 hp)1.67 kg/s745.7 W1 hp1 m2s21 W=21.1  m/s

Discussion In reality, the velocity will be less because of the inefficiencies of the motor and the fan.

6-167 A tank initially contains saturated mixture of R-134a. A valve is opened and R-134a vapor only is allowed to escape slowly such that temperature remains constant. The heat transfer necessary with the surroundings to maintain the temperature and pressure of the R-134a constant is to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid at the exit remains constant. 2 Kinetic and potential energies are negligible. 3 There are no work interactions involved.

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:

R-134a

0.4 kg, 1 L

25C

min−mout=Δmsystem    −me=m2−m1me=m1−m2

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin−mehe=m2u2−m1u1Qin=m2u2−m1u1+mehe

Combining the two balances:

Qin=m2u2−m1u1+(m1−m2)he

The specific volume at the initial state is

v1=Vm1=0.001 m30.4 kg=0.0025 m3/kg

The initial state properties of R-134a in the tank are

T1=26°C  x1=v1−vfvfg=0.0025−0.00083130.029976−0.0008313=0.05726u1=uf+x1ufg=87.26+(0.05726)(156.87)=96.24 kJ/kg  (Table A-11)

The enthalpy of saturated vapor refrigerant leaving the bottle is

he=hg@26°C=264.68 kJ/kg

The specific volume at the final state is

v2=Vm2=0.001 m30.1 kg=0.01 m3/kg

The internal energy at the final state is

T2=26°C  x2=v2−vfvfg=0.01−0.00083130.029976−0.0008313=0.3146u2=uf+x1ufg=87.26+(0.3146)(156.87)=136.61 kJ/kg  (Table A-11)

Substituting into the energy balance equation,

Qin=m2u2−m1u1+(m1−m2)he(0.1 kg)(136.61 kJ/kg)−(0.4 kg)(96.24 kJ/kg)+(0.4−0.1 kg)(264.68 kJ/kg)54.6 kJ

6-168 CD EES Steam expands in a turbine steadily. The mass flow rate of the steam, the exit velocity, and the power output are to be determined.

H2O

1

2

30 kJ/kg

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy changes are negligible.

Properties From the steam tables (Tables A-4 through 6)

and P1=10  MPa  v1=0.035655  m3/kgh1=3502.0  kJ/kgP2=25  kPa  v2=vf+x2vfg=0.00102+0.956.2034-0.00102=5.8933  m3/kgh2=hf+x2hfg=271.96+0.952345.5=2500.2  kJ/kg

Analysis  (a) The mass flow rate of the steam is

m=1v1V1A1=10.035655  m3/kg60  m/s0.015  m2=25.24  kg/s

(b) There is only one inlet and one exit, and thus . Then the exit velocity is determined from

m=1v2V2A2  V2=mv2A2=(25.24  kg/s)(5.8933  m3/kg)0.14  m2=1063  m/s

(c) We take the turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=Wout+Qout+m(h2+V22/2)   (since Δpe≃0)Wout=−Qout−mh2−h1+V22−V122

Then the power output of the turbine is determined by substituting to be

Wout=−25.24×30  kJ/s−25.24  kg/s2500.2−3502.0+1063  m/s2−60  m/s221  kJ/kg1000  m2/s210,330  kW

6-169 EES Problem 6-168 is reconsidered. The effects of turbine exit area and turbine exit pressure on the exit velocity and power output of the turbine as the exit pressure varies from 10 kPa to 50 kPa (with the same quality), and the exit area to varies from 1000 cm2 to 3000 cm2 is to be investigated. The exit velocity and the power output are to be plotted against the exit pressure for the exit areas of 1000, 2000, and 3000 cm2.

Analysis The problem is solved using EES, and the results are tabulated and plotted below.

Fluid$='Steam_IAPWS'

A[1]=150 [cm^2]

T[1]=550 [C]

P[1]=10000 [kPa]

Vel[1]= 60 [m/s]

A[2]=1400 [cm^2]

P[2]=25 [kPa]

q_out = 30 [kJ/kg]

m_dot = A[1]*Vel[1]/v[1]*convert(cm^2,m^2)

v[1]=volume(Fluid$, T=T[1], P=P[1]) "specific volume of steam at state 1"

Vel[2]=m_dot*v[2]/(A[2]*convert(cm^2,m^2))

v[2]=volume(Fluid$, x=0.95, P=P[2]) "specific volume of steam at state 2"

T[2]=temperature(Fluid$, P=P[2], v=v[2]) "[C]" "not required, but good to know"

"[conservation of Energy for steady-flow:"

"Ein_dot - Eout_dot = DeltaE_dot"  "For steady-flow, DeltaE_dot = 0"

DELTAE_dot=0 "[kW]"

"For the turbine as the control volume, neglecting the PE of each flow steam:"

E_dot_in=E_dot_out

h[1]=enthalpy(Fluid$,T=T[1], P=P[1])

E_dot_in=m_dot*(h[1]+ Vel[1]^2/2*Convert(m^2/s^2, kJ/kg))

h[2]=enthalpy(Fluid$,x=0.95, P=P[2])

E_dot_out=m_dot*(h[2]+ Vel[2]^2/2*Convert(m^2/s^2, kJ/kg))+ m_dot *q_out+ W_dot_out

Power=W_dot_out

Q_dot_out=m_dot*q_out

| Power [kW] | P2 [kPa] | Vel2 [m/s] |
| --- | --- | --- |
| -54208 | 10 | 2513 |
| -14781 | 14.44 | 1778 |
| 750.2 | 18.89 | 1382 |
| 8428 | 23.33 | 1134 |
| 12770 | 27.78 | 962.6 |
| 15452 | 32.22 | 837.6 |
| 17217 | 36.67 | 742.1 |
| 18432 | 41.11 | 666.7 |
| 19299 | 45.56 | 605.6 |
| 19935 | 50 | 555 |

6-170E Refrigerant-134a is compressed steadily by a compressor. The mass flow rate of the refrigerant and the exit temperature are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible.

Properties From the refrigerant tables (Tables A-11E through A-13E)

R-134a

2

1

P1=15  psia  v1=3.2551  ft3/lbmh1=107.52  Btu/lbm

Analysis  (a) The mass flow rate of refrigerant is

m=V1v1=10  ft3/s3.2551  ft3/lbm=3.072  lbm/s

(b) There is only one inlet and one exit, and thus . We take the compressor as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

Substituting,

(45 hp)0.7068  Btu/s1  hp=3.072 lbm/sh2−107.52 Btu/lbmh2=117.87  Btu/lbm

Then the exit temperature becomes

P2=100  psia T2=95.7°F

6-171 Air is preheated by the exhaust gases of a gas turbine in a regenerator. For a specified heat transfer rate, the exit temperature of air and the mass flow rate of exhaust gases are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 There are no work interactions. 4 Heat loss from the regenerator to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 5 Exhaust gases can be treated as air. 6 Air is an ideal gas with variable specific heats.

Properties The gas constant of air is 0.287 kPa.m3/kg.K (Table A-1). The enthalpies of air are (Table A-21)

AIR

Exhaust Gases

3

4

1

2

T1=550 K   →   h1=555.74 kJ/kgT3=800 K   →   h3=821.95 kJ/kgT4=600 K   →   h4=607.02 kJ/kg

Analysis (a) We take the air side of the heat exchanger as the system, which is a control volume since mass crosses the boundary. There is only one inlet and one exit, and thus . The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mairh1=mairh2   (since   {W=Δke≃Δpe≃0)Qin=mair(h2−h1)

Substituting,

3200  kJ/s=800/60  kg/sh2−554.71  kJ/kg   →  h2  =  794.71  kJ/kg

Then from Table A-21 we read

T2  =  775.1 K

(b)  Treating the exhaust gases as an ideal gas, the mass flow rate of the exhaust gases is determined from the steady-flow energy relation applied only to the exhaust gases,

Ein=Eoutmexhausth3=Qout+mexhausth4   (since   {W=Δke≃Δpe≃0)Qout=mexhaust(h3−h4)

3200  kJ/s=mexhaust821.95-607.02  kJ/kg

It yields

mexhaust=14.9  kg/s

6-172 Water is to be heated steadily from 20°C to 55°C by an electrical resistor inside an insulated pipe.  The power rating of the resistance heater and the average velocity of the water are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time at any point within the system and thus . 2 Water is an incompressible substance with constant specific heats. 3 The kinetic and potential energy changes are negligible, . 4 The pipe is insulated and thus the heat losses are negligible.

Properties The density and specific heat of water at room temperature are  = 1000 kg/m3 and  c = 4.18 kJ/kg·°C  (Table A-3).

Analysis (a) We take the pipe as the system. This is a control volume since mass crosses the system boundary during the process.  Also, there is only one inlet and one exit and thus . The energy balance for this steady-flow system can be expressed in the rate form as

We

·

30 L/min

D = 5 cm

WATER

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutWe,in+mh1=mh2   (since   {Qout≃Δke≃Δpe≃0)We,in=m(h2−h1)=m[c(T2−T1)+vΔPÃ0]=mcT2−T1

The mass flow rate of water through the pipe is

m=ρV1=(1000  kg/m3)(0.030  m3/min)=30  kg/min

Therefore,

We,in=mc(T2−T1)=(30/60  kg/s)(4.18 kJ/kg⋅C)(55−20)C=73.2 kW

(b)  The average velocity of water through the pipe is determined from

V=VA=Vπr2=0.030  m3/minπ (0.025  m)2=15.3 m/min

6-173 CD EES An insulated cylinder equipped with an external spring initially contains air. The tank is connected to a supply line, and air is allowed to enter the cylinder until its volume doubles. The mass of the air that entered and the final temperature in the cylinder are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid at the inlet remains constant.  2 The expansion process is quasi-equilibrium.  3 Kinetic and potential energies are negligible. 4 The spring is a linear spring. 5 The device is insulated and thus heat transfer is negligible. 6 Air is an ideal gas with constant specific heats.

Properties The gas constant of air is R = 0.287 kJ/kg·K (Table A-1). The specific heats of air at room temperature are cv = 0.718 and cp = 1.005 kJ/kg·K (Table A-2a). Also, u = cvT  and h = cpT.

Air

P = 200 kPa

T1 = 22C

V1 = 0.2 m3

Pi = 0.8 MPa

Ti = 22C

Fspring

Analysis We take the cylinder as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:

min−mout=Δmsystem   →    mi=m2−m1

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesmihi=Wb,out+m2u2−m1u1  (since  Q≃ke≃pe≃0)

Combining the two relations,	m2−m1hi=Wb,out+m2u2−m1u1

or,	                                           (m2−m1)cpTi=Wb,out+m2cvT2−m1cvT1

The initial and the final masses in the tank are

m1=P1V1RT1=200  kPa0.2  m30.287  kPa⋅m3/kg⋅K295  K=0.472  kgm2=P2V2RT2=600  kPa0.4  m30.287  kPa⋅m3/kg⋅KT2=836.2T2

Then from the mass balance becomes

The spring is a linear spring, and thus the boundary work for this process can be determined from

Wb=Area=P1+P22V2−V1=200+600 kPa20.4−0.2 m3=80  kJ

Substituting into the energy balance, the final temperature of air T2 is determined to be

−80=−836.2T2−0.4721.005295+836.2T20.718T2−0.4720.718295

It yields		T2  =  344.1 K

Thus,		m2=836.2T2=836.2344.1=2.430  kg

and		mi  =  m2  -  m1  =  2.430  -  0.472  =  1.96 kg

6-174E … 6-178  Design and Essay Problems

