Solutions Manual

for

Introduction to Thermodynamics and Heat Transfer

Yunus A. Cengel

2nd Edition, 2008

Chapter 8

ENTROPY

# PROPRIETARY AND CONFIDENTIAL

This Manual is the proprietary property of The McGraw-Hill Companies, Inc. (“McGraw-Hill”) and protected by copyright and other state and federal laws.  By opening and using this Manual the user agrees to the following restrictions, and if the recipient does not agree to these restrictions, the Manual should be promptly returned unopened to McGraw-Hill:  This Manual is being provided only to authorized professors and instructors for use in preparing for the classes using the affiliated textbook.  No other use or distribution of this Manual is permitted.  This Manual may not be sold and may not be distributed to or used by any student or other third party.  No part of this Manual may be reproduced, displayed or distributed in any form or by any means, electronic or otherwise, without the prior written permission of McGraw-Hill.

Entropy and the Increase of Entropy Principle

8-1C  No.  The δQ represents the net heat transfer during a cycle, which could be positive.

8-2C No.  A system may produce more (or less) work than it receives during a cycle.  A steam power plant, for example, produces more work than it receives during a cycle, the difference being the net work output.

8-3C  The entropy change will be the same for both cases since entropy is a property and it has a fixed value at a fixed state.

8-4C No.  In general, that integral will have a different value for different processes.  However, it will have the same value for all reversible processes.

8-5C  Yes.

8-6C  That integral should be performed along a reversible path to determine the entropy change.

8-7C No.  An isothermal process can be irreversible. Example: A system that involves paddle-wheel work while losing an equivalent amount of heat.

8-8C  The value of this integral is always larger for reversible processes.

8-9C  No.  Because the entropy of the surrounding air increases even more during that process, making the total entropy change positive.

8-10C  It is possible to create entropy, but it is not possible to destroy it.

8-11C If the system undergoes a reversible process, the entropy of the system cannot change without a heat transfer.  Otherwise, the entropy must increase since there are no offsetting entropy changes associated with reservoirs exchanging heat with the system.

8-12C The claim that work will not change the entropy of a fluid passing through an adiabatic steady-flow system with a single inlet and outlet is true only if the process is also reversible.  Since no real process is reversible, there will be an entropy increase in the fluid during the adiabatic process in devices such as pumps, compressors, and turbines.

8-13C Sometimes.

8-14C  Never.

8-15C  Always.

8-16C  Increase.

8-17C  Increases.

8-18C  Decreases.

8-19C  Sometimes.

8-20C  Yes.  This will happen when the system is losing heat, and the decrease in entropy as a result of this heat loss is equal to the increase in entropy as a result of irreversibilities.

8-21C  They are heat transfer, irreversibilities, and entropy transport with mass.

8-22C  Greater than.

8-23 A rigid tank contains an ideal gas that is being stirred by a paddle wheel. The temperature of the gas remains constant as a result of heat transfer out. The entropy change of the gas is to be determined.

IDEAL GAS

40C

Heat

30C

200 kJ

Assumptions  The gas in the tank is given to be an ideal gas.

Analysis  The temperature and the specific volume of the gas remain constant during this process. Therefore, the initial and the final states of the gas are the same. Then s2 = s1 since entropy is a property.  Therefore,

ΔSsys=0

8-24 Air is compressed steadily by a compressor. The air temperature is maintained constant by heat rejection to the surroundings. The rate of entropy change of air is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible.  3 Air is an ideal gas. 4 The process involves no internal irreversibilities such as friction, and thus it is an isothermal, internally reversible process.

Properties Noting that h =  h(T) for ideal gases, we have h1 = h2  since T1 = T2 = 25C.

Analysis  We take the compressor as the system. Noting that the enthalpy of air remains constant, the energy balance for this steady-flow system can be expressed in the rate form as

AIR

T = const.

P2

P1

Q

·

12 kW

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutWin=Qout

Therefore,

Qout=Win=12 kW

Noting that the process is assumed to be an isothermal and internally reversible process, the rate of entropy change of air is determined to be

ΔSair=−Qout,airTsys=−12 kW298  K=−0.0403 kW/K

8-25 Heat is transferred directly from an energy-source reservoir to an energy-sink. The entropy change of the two reservoirs is to be calculated and it is to be determined if the increase of entropy principle is satisfied.

Assumptions  The reservoirs operate steadily.

Analysis  The entropy change of the source and sink is given by

ΔS=QHTH+QLTL=−100 kJ1200 K+100 kJ600 K=0.0833 kJ/K

Since the entropy of everything involved in this process has increased, this transfer of heat is possible.

8-26 It is assumed that heat is transferred from a cold reservoir to the hot reservoir contrary to the Clausius statement of the second law. It is to be proven that this violates the increase in entropy principle.

Assumptions  The reservoirs operate steadily.

Q =100 kJ

TH

TL

Analysis  According to the definition of the entropy, the entropy change of the high-temperature reservoir shown below is

ΔSH=QTH=100 kJ1200 K=0.08333 kJ/K

and the entropy change of the low-temperature reservoir is

ΔSL=QTL=−100 kJ600 K=−0.1667 kJ/K

The total entropy change of everything involved with this system is then

ΔStotal=ΔSH+ΔSL=0.08333−0.1667=−0.0833 kJ/K

which violates the increase in entropy principle since the entropy is decreasing, not increasing or staying fixed.

8-27 Heat is transferred from a hot reservoir to a cold reservoir. The entropy change of the two reservoirs is to be calculated and it is to be determined if the second law is satisfied.

2 kW

TH

TL

Assumptions  The reservoirs operate steadily.

Analysis  The rate of entropy change of everything involved in this transfer of heat is given by

ΔS=QHTH+QLTL=−2 kW800 K+2 kW300 K=0.00417 kW/K

Since this rate is positive (i.e., the entropy increases as time passes), this transfer of heat is possible.

8-28E A reversible air conditioner with specified reservoir temperatures is considered. The entropy change of two reservoirs is to be calculated and it is to be determined if this air conditioner satisfies the increase in entropy principle.

Assumptions  The air conditioner operates steadily.

530 R

570 R

R

Analysis  According to the thermodynamic temperature scale,

QH=QLTHTL=(36,000 Btu/h)570 R530 R=38,720 Btu/h

The rate of entropy change of the hot reservoir is then

ΔSH=QHTH=−38,720 Btu/h570 R=−67.92 Btu/h⋅R

Similarly, the rate of entropy change of the cold reservoir is

ΔSL=QLTL=36,000 Btu/h530 R=67.92 Btu/h⋅R

The net rate of entropy change of everything in this system is

ΔStotal=ΔSH+ΔSL=−67.92+67.92=0 Btu/h⋅R

The net rate of entropy change is zero as it must be in order to satisfy the second law.

8-29 A reversible heat pump with specified reservoir temperatures is considered. The entropy change of two reservoirs is to be calculated and it is to be determined if this heat pump satisfies the increase in entropy principle.

Assumptions  The heat pump operates steadily.

10°C

21°C

HP

100 kW

Analysis  Since the heat pump is completely reversible, the combination of the coefficient of performance expression, first Law, and thermodynamic temperature scale gives

COPHP,rev=11−TLTH=11−(283 K)(294 K)=26.73

The power required to drive this heat pump, according to the coefficient of performance, is then

Wnet,in=QHCOPHP,rev=100 kW26.73=3.741 kW

According to the first law, the rate at which heat is removed from the low-temperature energy reservoir is

QL=QH−Wnet,in=100 kW−3.741 kW=96.26 kW

The rate at which the entropy of the high temperature reservoir changes, according to the definition of the entropy, is

ΔSH=QHTH=100 kW294 K=0.340 kW/K

and that of the low-temperature reservoir is

ΔSL=QLTL=−96.26 kW283 K=−0.340 kW/K

The net rate of entropy change of everything in this system is

ΔStotal=ΔSH+ΔSL=0.340−0.340=0 kW/K

as it must be since the heat pump is completely reversible.

8-30E Heat is transferred isothermally from the working fluid of a Carnot engine to a heat sink. The entropy change of the working fluid is given. The amount of heat transfer, the entropy change of the sink, and the total entropy change during the process are to be determined.

95F

Carnot heat engine

SINK

95F

Heat

Analysis (a) This is a reversible isothermal process, and the entropy change during such a process is given by

Noting that heat transferred from the working fluid is equal to the heat transferred to the sink, the heat transfer become

Qfluid=TfluidΔSfluid=555  R−0.7  Btu/R=−388.5  Btu   →   Qfluid,out=388.5 Btu

(b)  The entropy change of the sink is determined from

ΔSsink=Qsink,inTsink=388.5  Btu555  R=0.7 Btu/R

(c)  Thus the total entropy change of the process is

Sgen=ΔStotal=ΔSfluid+ΔSsink=−0.7+0.7=0

This is expected since all processes of the Carnot cycle are reversible processes, and no entropy is generated during a reversible process.

8-31 R-134a enters an evaporator as a saturated liquid-vapor at a specified pressure. Heat is transferred to the refrigerant from the cooled space, and the liquid is vaporized. The entropy change of the refrigerant, the entropy change of the cooled space, and the total entropy change for this process are to be determined.

Assumptions 1 Both the refrigerant and the cooled space involve no internal irreversibilities such as friction. 2 Any temperature change occurs within the wall of the tube, and thus both the refrigerant and the cooled space remain isothermal during this process. Thus it is an isothermal, internally reversible process.

Analysis Noting that both the refrigerant and the cooled space undergo reversible isothermal processes, the entropy change for them can be determined from

(a) The pressure of the refrigerant is maintained constant.  Therefore, the temperature of the refrigerant also remains constant at the saturation value,

T=Tsat @160  kPa=−15.6°C=257.4  K     (Table A-12)

R-134a

160 kPa

-5C

180 kJ

Then,

ΔSrefrigerant=Qrefrigerant,inTrefrigerant=180  kJ257.4  K=0.699  kJ/K

(b)  Similarly,

ΔSspace=−Qspace,outTspace=−180  kJ268  K=−0.672  kJ/K

(c)  The total entropy change of the process is

Sgen=ΔStotal=ΔSrefrigerant+ΔSspace=0.699−0.672=0.027  kJ/K

# Entropy Changes of Pure Substances

8-32C  Yes, because an internally reversible, adiabatic process involves no irreversibilities or heat transfer.

8-33C According to the conservation of mass principle,

dmCVdtinmoutmdmdt { dot in

An entropy balance adapted to this system becomes

dSsurrdt+d(ms)dt−minsin≥0

When this is combined with the mass balance, and the constant entropies are removed from the derivatives, it becomes

dSsurrdt+sdmdt−sindmdt≥0

Multiplying by dt and integrating the result yields

ΔSsurr+(s−sin)Δm≥0

or

ΔSsurr≥(s−sin)Δm

8-34C According to the conservation of mass principle,

dmCVdt=inm−outmdmdt { dot out

An entropy balance adapted to this system becomes

dSsurrdt+d(ms)dt+mouts≥0

When this is combined with the mass balance, it becomes

dSsurrdt+sdmdt−sdmdt≥0

Multiplying by dt and integrating the result yields

ΔSsurr+m2s2−m1s1−s(m2−m1)≥0

Since all the entropies are same, this reduces to

ΔSsurr≥0

Hence, the entropy of the surroundings can only increase or remain fixed.

8-35 R-134a is expanded in a turbine during which the entropy remains constant. The enthalpy difference is to be determined.

T

s

2

1

Analysis The initial state is superheated vapor and thus

11

The entropy is constant during the process. The final state is also superheated vapor and the enthalpy at this state is

221

Note that the properties at the inlet and exit states can also be determined from Table A-13E by interpolation but the values will not be as accurate as those by EES. The change in the enthalpy across the turbine is then

Δh=h2−h1=106.95−129.95=−23.0 Btu/lbm

8-36E A piston-cylinder device that is filled with water is heated. The total entropy change is to be determined.

Analysis The initial specific volume is

H2O

300 psia

2 lbm

2.5 ft3

1V1m2.5 ft32 lbm3

which is between vf and vg for 300 psia. The initial quality and the entropy are then (Table A-5E)

x1=v1−vfvfg=(1.25−0.01890) ft3/lbm(1.5435−0.01890) ft3/lbm=0.80751

P

v

2

1

The final state is superheated vapor and

T2=500°F21

Hence, the change in the total entropy is

ΔS=m(s2−s1)(2 lbm)(1.5706−1.3334) Btu/lbm⋅R0.4744 Btu/R

8-37 Water is compressed in a compressor during which the entropy remains constant. The final temperature and enthalpy are to be determined.

Analysis The initial state is superheated vapor and the entropy is

T

s

2

1

T1=160°C1

Note that the properties can also be determined from Table A-6 by interpolation but the values will not be as accurate as those by EES. The final state is superheated vapor and the properties are (Table A-6)

221

8-38E R-134a is expanded isentropically in a closed system. The heat transfer and work production are to be determined.

Assumptions 1 The system is stationary and thus the kinetic and potential energy changes are zero. 2 There are no work interactions involved other than the boundary work. 3 The thermal energy stored in the cylinder itself is negligible. 4 The compression or expansion process is quasi-equilibrium.

R-134a

1 lbm

100 psia

100F

Analysis As there is no area under the process line shown on the T-s diagram and this process is reversible,

The energy balance for this system can be expressed as

T

s

2

1

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energies−Wout=ΔU=m(u2−u1)Wout=m(u1−u2)

The initial state properties are

T1=100°F1

The final state properties for this isentropic process are (Table A-12E)

221

Substituting,

Wout=m(u1−u2)=(1 lbm)(109.45−90.40) Btu/lbm=19.05 Btu

8-39 An insulated rigid tank contains a saturated liquid-vapor mixture of water at a specified pressure. An electric heater inside is turned on and kept on until all the liquid vaporized. The entropy change of the water during this process is to be determined.

We

H2O

2 kg

100 kPa

Analysis From the steam tables (Tables A-4 through A-6)

P1=100  kPa  v1=vf+x1vfg=0.001+0.251.6941−0.001=0.4243  m3/kgs1=sf+x1sfg=1.3028+0.256.0562=2.8168  kJ/kg⋅Kv2=v12

Then the entropy change of the steam becomes

ΔS=ms2−s1=(2 kg)(6.8649−2.8168)  kJ/kg⋅K=8.10   kJ/K

8-40 CD EES A rigid tank is divided into two equal parts by a partition. One part is filled with compressed liquid water while the other side is evacuated. The partition is removed and water expands into the entire tank. The entropy change of the water during this process is to be determined.

1.5 kg

compressed liquid

300 kPa

60C

Vacuum

Analysis The properties of the water are (Table A-4)

P1=300  kPa  v1≃vf @ 60°C=0.001017 m3/kgs1=sf @ 60°C=0.8313  kJ/kg⋅K

Noting that

v2=2v1=20.001017=0.002034  m3/kg

P2=15  kPax2=v2−vfvfg=0.002034−0.00101410.02−0.001014=0.0001018s2=sf+x2sfg=0.7549+0.00010187.2522=0.7556  kJ/kg⋅K

Then the entropy change of the water becomes

ΔS=ms2−s1=1.5  kg0.7556−0.8313 kJ/kg⋅K=−0.114  kJ/K

8-41 EES Problem 8-40 is reconsidered. The entropy generated is to be evaluated and plotted as a function of surroundings temperature, and the values of the surroundings temperatures that are valid for this problem are to be determined. The surrounding temperature is to vary from 0°C to 100°C.

Analysis The problem is solved using EES, and the results are tabulated and plotted below.

"Input Data"

P[1]=300 [kPa]

T[1]=60 [C]

m=1.5 [kg]

P[2]=15 [kPa]

Fluid$='Steam_IAPWS'

V[1]=m*spv[1]

spv[1]=volume(Fluid$,T=T[1], P=P[1]) "specific volume of steam at state 1, m^3/kg"

s[1]=entropy(Fluid$,T=T[1],P=P[1]) "entropy of steam at state 1, kJ/kgK"

V[2]=2*V[1] "Steam expands to fill entire volume at state 2"

"State 2 is identified by P[2] and spv[2]"

spv[2]=V[2]/m "specific volume of steam at state 2, m^3/kg"

s[2]=entropy(Fluid$,P=P[2],v=spv[2])  "entropy of steam at state 2, kJ/kgK"

T[2]=temperature(Fluid$,P=P[2],v=spv[2])

DELTAS_sys=m*(s[2]-s[1])  "Total entopy change of steam, kJ/K"

"What does the first law tell us about this problem?"

"Conservation of Energy for the entire, closed system"

E_in - E_out = DELTAE_sys

"neglecting changes in KE and PE for the system:"

DELTAE_sys=m*(intenergy(Fluid$, P=P[2], v=spv[2]) - intenergy(Fluid$,T=T[1],P=P[1]))

E_in = 0

"How do you interpert the energy leaving the system, E_out?  Recall this is a constant volume

system."

Q_out = E_out

"What is the maximum value of the Surroundings temperature?"

"The maximum possible value for the surroundings temperature occurs when we set

S_gen = 0=Delta S_sys+sum(DeltaS_surr)"

Q_net_surr=Q_out

S_gen = 0

S_gen = DELTAS_sys+Q_net_surr/Tsurr

"Establish a parametric table for the variables S_gen, Q_net_surr, T_surr, and DELTAS_sys.  In

the Parametric Table window select T_surr and insert a range of values.  Then place '{' and '}'

about the S_gen = 0 line; press F3 to solve the table.  The results are shown in Plot Window 1.

What values of T_surr are valid for this problem?"

| Sgen [kJ/K] | Qnet,surr [kJ] | Tsurr [K] | Ssys [kJ/K] |
| --- | --- | --- | --- |
| 0.02533 | 37.44 | 270 | -0.1133 |
| 0.01146 | 37.44 | 300 | -0.1133 |
| 0.0001205 | 37.44 | 330 | -0.1133 |
| -0.009333 | 37.44 | 360 | -0.1133 |
| -0.01733 | 37.44 | 390 | -0.1133 |

8-42E A cylinder is initially filled with R-134a at a specified state. The refrigerant is cooled and condensed at constant pressure. The entropy change of refrigerant during this process is to be determined

Analysis   From the refrigerant tables (Tables A-11E through A-13E),

R-134a

120 psia

100F

Q

P1=120  psia s1=0.22361  Btu/lbm⋅RT2=50°F  s2≃sf @ 90F=0.06039  Btu/lbm⋅R

Then the entropy change of the refrigerant becomes

ΔS=ms2−s1=2  lbm0.06039−0.22361 Btu/lbm⋅R=−0.3264  Btu/R

8-43 An insulated cylinder is initially filled with saturated liquid water at a specified pressure. The water is heated electrically at constant pressure. The entropy change of the water during this process is to be determined.

H2O

150 kPa

Sat. liquid

2200 kJ

Assumptions 1 The kinetic and potential energy changes are negligible.  2 The cylinder is well-insulated and thus heat transfer is negligible. 3 The thermal energy stored in the cylinder itself is negligible. 4 The compression or expansion process is quasi-equilibrium.

Analysis   From the steam tables (Tables A-4 through A-6),

P1=150  kPa  v1=vf @ 150  kPa=0.001053  m3/kgh1=hf @ 150  kPa=467.13  kJ/kgs1=sf @ 150  kPa=1.4337  kJ/kg⋅K

Also,

m=Vv1=0.005  m30.001053  m3/kg=4.75  kg

We take the contents of the cylinder as the system. This is a closed system since no mass enters or leaves. The energy balance for this stationary closed system can be expressed as

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesWe,in−Wb,out=ΔUWe,in=m(h2−h1)

since U + Wb = H during a constant pressure quasi-equilibrium process.  Solving for h2,

h2=h1+We,inm=467.13+2200  kJ4.75  kg=930.33 kJ/kg

Thus,

P2=150  kPax2=h2−hfhfg=930.33−467.132226.0=0.2081s2=sf+x2sfg=1.4337+0.20815.7894=2.6384 kJ/kg⋅K

Then the entropy change of the water becomes

ΔS=ms2−s1=4.75  kg2.6384−1.4337 kJ/kg⋅K=5.72   kJ/K

8-44 An insulated cylinder is initially filled with saturated R-134a vapor at a specified pressure. The refrigerant expands in a reversible manner until the pressure drops to a specified value. The final temperature in the cylinder and the work done by the refrigerant are to be determined.

Assumptions 1 The kinetic and potential energy changes are negligible.  2 The cylinder is well-insulated and thus heat transfer is negligible. 3 The thermal energy stored in the cylinder itself is negligible. 4 The process is stated to be reversible.

Analysis (a) This is a reversible adiabatic (i.e., isentropic) process, and thus s2 = s1.  From the refrigerant tables (Tables A-11 through A-13),

R-134a

0.8 MPa

0.05 m3

P1=0.8  MPa  v1=vg @ 0.8  MPa=0.025621  m3/kgu1=ug @ 0.8  MPa=246.79  kJ/kgs1=sg @ 0.8  MPa=0.91835  kJ/kg⋅K

Also,

m=Vv1=0.05  m30.025621  m3/kg=1.952 kg

and

P2=0.4  MPa x2=s2−sfsfg=0.91835−0.247610.67929=0.9874u2=uf+x2ufg=63.62+0.9874171.45=232.91  kJ/kg

T2=Tsat @ 0.4  MPa=8.91°C

(b) We take the contents of the cylinder as the system. This is a closed system since no mass enters or leaves. The energy balance for this adiabatic closed system can be expressed as

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energies−Wb,out=ΔUWb,out=m(u1−u2)

Substituting, the work done during this isentropic process is determined to be

Wb,out=mu1−u2=(1.952  kg)(246.79−232.91)  kJ/kg=27.09  kJ

8-45 EES Problem 8-44 is reconsidered. The work done by the refrigerant is to be calculated and plotted as a function of final pressure as the pressure varies from 0.8 MPa to 0.4 MPa. The work done for this process is to be compared to one for which the temperature is constant over the same pressure range.

Analysis The problem is solved using EES, and the results are tabulated and plotted below.

Procedure IsothermWork(P_1,x_1,m_sys,P_2:Work_out_Isotherm,Q_isotherm,DELTAE_isotherm,T_isotherm)

T_isotherm=Temperature(R134a,P=P_1,x=x_1)

T=T_isotherm

u_1 = INTENERGY(R134a,P=P_1,x=x_1)

v_1 = volume(R134a,P=P_1,x=x_1)

s_1 = entropy(R134a,P=P_1,x=x_1)

u_2 = INTENERGY(R134a,P=P_2,T=T)

s_2 = entropy(R134a,P=P_2,T=T)

"The process is reversible and Isothermal thus the heat transfer is determined by:"

Q_isotherm = (T+273)*m_sys*(s_2 - s_1)

DELTAE_isotherm = m_sys*(u_2 - u_1)

E_in = Q_isotherm

E_out = DELTAE_isotherm+E_in

Work_out_isotherm=E_out

END

"Knowns:"

P_1 = 800 [kPa]

x_1 = 1.0

V_sys = 0.05[m^3]

"P_2 = 400 [kPa]"

"Analysis: "

" Treat the rigid tank as a closed system, with no heat transfer in, neglect

changes in KE and PE of the R134a."

"The isentropic work is determined from:"

E_in - E_out = DELTAE_sys

E_out = Work_out_isen

E_in = 0

DELTAE_sys = m_sys*(u_2 - u_1)

u_1 = INTENERGY(R134a,P=P_1,x=x_1)

v_1 = volume(R134a,P=P_1,x=x_1)

s_1 = entropy(R134a,P=P_1,x=x_1)

V_sys = m_sys*v_1

"Rigid Tank:  The process is reversible and adiabatic or isentropic.

Then P_2 and s_2 specify state 2."

s_2 = s_1

u_2 = INTENERGY(R134a,P=P_2,s=s_2)

T_2_isen = temperature(R134a,P=P_2,s=s_2)

Call IsothermWork(P_1,x_1,m_sys,P_2:Work_out_Isotherm,Q_isotherm,DELTAE_isotherm,T_isotherm)

| P2 [kPa] | Workout,isen [kJ] | Workout,isotherm [kJ] | Qisotherm [kJ] |
| --- | --- | --- | --- |
| 400 | 27.09 | 60.02 | 47.08 |
| 500 | 18.55 | 43.33 | 33.29 |
| 600 | 11.44 | 28.2 | 21.25 |
| 700 | 5.347 | 13.93 | 10.3 |
| 800 | 0 | 0 | 0 |

8-46 Saturated Refrigerant-134a vapor at 160 kPa is compressed steadily by an adiabatic compressor. The minimum power input to the compressor is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible.

Analysis The power input to an adiabatic compressor will be a minimum when the compression process is reversible.  For the reversible adiabatic process we have s2 = s1.  From the refrigerant tables (Tables A-11 through A-13),

R-134a

2

1

P1=160  kPa  v1=vg @ 160  kPa=0.12348  m3/kgh1=hg @ 160  kPa=241.11  kJ/kgs1=sg @ 160  kPa=0.9419  kJ/kg⋅KP2=900  kPa  h2=277.06  kJ/kg

Also,

m=V1v1=2  m3/min0.12348  m3/kg=16.20  kg/min=0.27  kg/s

There is only one inlet and one exit, and thus . We take the compressor as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

Substituting, the minimum power supplied to the compressor is determined to be

Win=0.27  kg/s277.06−241.11 kJ/kg=9.71   kW

8-47 An insulated cylinder is initially filled with superheated steam at a specified state. The steam is compressed in a reversible manner until the pressure drops to a specified value. The work input during this process is to be determined.

Assumptions 1 The kinetic and potential energy changes are negligible.  2 The cylinder is well-insulated and thus heat transfer is negligible. 3 The thermal energy stored in the cylinder itself is negligible. 4 The process is stated to be reversible.

Analysis This is a reversible adiabatic (i.e., isentropic) process, and thus s2 = s1.  From the steam tables (Tables A-4 through A-6),

H2O

300 kPa

150C

P1=300 kPa  v1=0.63402  m3/kgu1=2571.0  kJ/kgs1=7.0792  kJ/kg⋅KP2=1  MPa  u2=2773.8  kJ/kg

Also,

m=Vv1=0.05  m30.63402  m3/kg=0.0789  kg

We take the contents of the cylinder as the system. This is a closed system since no mass enters or leaves. The energy balance for this adiabatic closed system can be expressed as

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesWb,in=ΔU=m(u2−u1)

Substituting, the work input during this adiabatic process is determined to be

Wb,in=mu2−u1=0.0789  kg2773.8−2571.0  kJ/kg=16.0   kJ

8-48 EES Problem 8-47 is reconsidered. The work done on the steam is to be determined and plotted as a function of final pressure as the pressure varies from 300 kPa to 1 MPa.

Analysis The problem is solved using EES, and the results are tabulated and plotted below.

"Knowns:"

P_1 = 300 [kPa]

T_1 = 150 [C]

V_sys = 0.05 [m^3]

"P_2 = 1000 [kPa]"

"Analysis: "

Fluid$='Steam_IAPWS'

" Treat the piston-cylinder as a closed system, with no heat transfer in, neglect

changes in KE and PE of the Steam.  The process is reversible and adiabatic thus isentropic."

"The isentropic work is determined from:"

E_in - E_out = DELTAE_sys

E_out = 0 [kJ]

E_in = Work_in

DELTAE_sys = m_sys*(u_2 - u_1)

u_1 = INTENERGY(Fluid$,P=P_1,T=T_1)

v_1 = volume(Fluid$,P=P_1,T=T_1)

s_1 = entropy(Fluid$,P=P_1,T=T_1)

V_sys = m_sys*v_1

" The process is reversible and adiabatic or isentropic.

Then P_2 and s_2 specify state 2."

s_2 = s_1

u_2 = INTENERGY(Fluid$,P=P_2,s=s_2)

T_2_isen = temperature(Fluid$,P=P_2,s=s_2)

| P2 [kPa] | Workin [kJ] |
| --- | --- |
| 300 | 0 |
| 400 | 3.411 |
| 500 | 6.224 |
| 600 | 8.638 |
| 700 | 10.76 |
| 800 | 12.67 |
| 900 | 14.4 |
| 1000 | 16 |

8-49 A cylinder is initially filled with saturated water vapor at a specified temperature. Heat is transferred to the steam, and it expands in a reversible and isothermal manner until the pressure drops to a specified value. The heat transfer and the work output for this process are to be determined.

H2O

200C

sat. vapor

T = const

Q

Assumptions 1 The kinetic and potential energy changes are negligible.  2 The cylinder is well-insulated and thus heat transfer is negligible. 3 The thermal energy stored in the cylinder itself is negligible. 4 The process is stated to be reversible and isothermal.

Analysis From the steam tables (Tables A-4 through A-6),

T1=200°C u1=ug @ 200°C=2594.2  kJ/kgs1=sg @ 200°C=6.4302  kJ/kg⋅KP2=800  kPa  u2=2631.1  kJ/kgs2=6.8177  kJ/kg⋅K

The heat transfer for this reversible isothermal process can be determined from

Q=TΔS=Tms2−s1=(473  K)(1.2  kg)(6.8177−6.4302) kJ/kg⋅K=219.9   kJ

We take the contents of the cylinder as the system. This is a closed system since no mass enters or leaves. The energy balance for this closed system can be expressed as

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin−Wb,out=ΔU=m(u2−u1)Wb,out=Qin−m(u2−u1)

Substituting, the work done during this process is determined to be

Wb,out=219.9 kJ−(1.2  kg)(2631.1−2594.2) kJ/kg=175.6    kJ

8-50 EES Problem 8-49 is reconsidered. The heat transferred to the steam and the work done are to be determined and plotted as a function of final pressure as the pressure varies from the initial value to the final value of 800 kPa.

Analysis The problem is solved using EES, and the results are tabulated and plotted below.

"Knowns:"

T_1 = 200 [C]

x_1 = 1.0

m_sys = 1.2 [kg]

{P_2 = 800"[kPa]"}

"Analysis: "

Fluid$='Steam_IAPWS'

" Treat the piston-cylinder as a closed system,  neglect changes in KE and PE of the Steam.  The process is reversible and isothermal ."

T_2 = T_1

E_in - E_out = DELTAE_sys

E_in = Q_in

E_out = Work_out

DELTAE_sys = m_sys*(u_2 - u_1)

P_1 = pressure(Fluid$,T=T_1,x=1.0)

u_1 = INTENERGY(Fluid$,T=T_1,x=1.0)

v_1 = volume(Fluid$,T=T_1,x=1.0)

s_1 = entropy(Fluid$,T=T_1,x=1.0)

V_sys = m_sys*v_1

" The process is reversible and isothermal.

Then P_2 and T_2 specify state 2."

u_2 = INTENERGY(Fluid$,P=P_2,T=T_2)

s_2 = entropy(Fluid$,P=P_2,T=T_2)

Q_in= (T_1+273)*m_sys*(s_2-s_1)

| P2 [kPa] | Qin [kJ] | Workout [kJ] |
| --- | --- | --- |
| 800 | 219.9 | 175.7 |
| 900 | 183.7 | 144.7 |
| 1000 | 150.6 | 117 |
| 1100 | 120 | 91.84 |
| 1200 | 91.23 | 68.85 |
| 1300 | 64.08 | 47.65 |
| 1400 | 38.2 | 27.98 |
| 1500 | 13.32 | 9.605 |
| 1553 | 219.9 | 175.7 |

8-51 Water is compressed isentropically in a closed system. The work required is to be determined.

Assumptions 1 The system is stationary and thus the kinetic and potential energy changes are zero. 2 There are no work interactions involved other than the boundary work. 3 The thermal energy stored in the cylinder itself is negligible. 4 The compression or expansion process is quasi-equilibrium.

Water

10C

x = 0.814

Analysis The energy balance for this system can be expressed as

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesWin=ΔU=m(u2−u1)

The initial state properties are

T

s

2

1

u1=uf+x1ufg=42.020+(0.814)(2346.6)=1952.2 kJ/kg1

Since the entropy is constant during this process,

221

Substituting,

win=u2−u1=(3132.6−1952.2) kJ/kg=1180.4 kJ/kg

8-52 R-134a undergoes an isothermal process in a closed system. The work and heat transfer are to be determined.

Assumptions 1 The system is stationary and thus the kinetic and potential energy changes are zero. 2 There are no work interactions involved other than the boundary work. 3 The thermal energy stored in the cylinder itself is negligible. 4 The compression or expansion process is quasi-equilibrium.

T

s

2

1

R-134a

240 kPa

T1 =T2 =20C

Analysis The energy balance for this system can be expressed as

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesWin−Qout=ΔU=m(u2−u1)

The initial state properties are

11

For this isothermal process, the final state properties are (Table A-11)

212

The heat transfer is determined from

qin=T0(s2−s1)=(293 K)(0.42497−1.0134) kJ/kg⋅K=−172.4 kJ/kg

The negative sign shows that the heat is actually transferred from the system. That is,

qout=172.4 kJ/kg

The work required is determined from the energy balance to be

win=qout+(u2−u1)=172.4 kJ/kg+(111.29−246.74) kJ/kg=36.95 kJ/kg

8-53 The total heat transfer for the process 1-3 shown in the figure is to be determined.

T (C)

S (kJ/K)

2

1

3

360

55

1       2      3

Analysis For a reversible process, the area under the process line in T-s diagram is equal to the heat transfer during that process. Then,

Q1-3=Q1-2+Q2-312TdS+23TdST1+T22(S2−S1)+T2(S3−S2)(360+273)+(55+273) K2(3−1)kJ/K+(360+273 K)(2−3)kJ/K328 kJ

8-54 The total heat transfer for the process 1-2 shown in the figure is to be determined.

Analysis For a reversible process, the area under the process line in T-s diagram is equal to the heat transfer during that process. Then,

T (C)

S (kJ/K)

2

1

500

100

0.2           1.0

Q1-2=12TdST1+T22(S2−S1)(500+273)+(100+273) K2(1.0−0.2)kJ/K458.4 kJ

8-55 The heat transfer for the process 1-3 shown in the figure is to be determined.

Analysis For a reversible process, the area under the process line in T-s diagram is equal to the heat transfer during that process. Then,

T (C)

s (kJ/kgK)

2

1

120

100

30

0.02            1.0

3

q1-3=12Tds+23TdsT1+T22(s2−s1)+0(120+273)+(30+273) K2(1.0−0.02)kJ/kg⋅K341.0 kJ/kg

8-56E The total heat transfer for the process 1-2 shown in the figure is to be determined.

T (F)

s (Bu/lbmR)

2

1

500

100

0.2           1.0

Analysis For a reversible process, the area under the process line in T-s diagram is equal to the heat transfer during that process. Then,

Q1-2=12TdST1+T22(s2−s1)(500+460)+(100+460) R2(1.0−0.2) Btu/lbm⋅R608 Btu/lbm

8-57 The change in the entropy of R-134a as it is heated at constant pressure is to be calculated using the relation ds = (Q /T) int rev, and it is to be verified by using R-134a tables.

Analysis As R-134a is converted from a saturated liquid to a saturated vapor, both the pressure and temperature remains constant. Then, the relation ds = (Q /T) int rev reduces to

T

s

2

1

ds=dhT

When this result is integrated between the saturated liquid and saturated vapor states, the result is (Table A-12)

sg−sf=hg−hfT=hfg@200 kPaTsat @ 200 kPa206.03 kJ/kg(−10.09+273.15) K=0.78321 kJ/kg⋅K

Finding the result directly from the R-134a tables

sg−sf=sfg@200 kPa=0.78316 kJ/kg⋅K	(Table A-12)

The two results are practically identical.

8-58 Steam is expanded in an isentropic turbine. The work produced is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 The process is isentropic (i.e., reversible-adiabatic).

Analysis There is only one inlet and one exit, and thus m1=m2=m. We take the turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Turbine

2 MPa

360°C

100 kPa

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

T

s

2

1

2 MPa

100 kPa

mh1=mh2+WoutWout=m(h1−h2)

The inlet state properties are

11

For this isentropic process, the final state properties are (Table A-5)

221

Substituting,

wout=h1−h2=(3159.9−2538.9) kJ/kg=621.0 kJ/kg

8-59 R-134a is compressed in an isentropic compressor. The work required is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 The process is isentropic (i.e., reversible-adiabatic).

Analysis There is only one inlet and one exit, and thus m1=m2=m. We take the compressor as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Compressor

0°F

sat. vapor

200 psia

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

mh1+Win=mh2Win=m(h2−h1)

T

s

2

1

200 psia

0F

The inlet state properties are

11

For this isentropic process, the final state enthalpy is

221

Substituting,

win=h2−h1=(123.28−103.08) Btu/lbm=20.2 Btu/lbm

8-60 Steam is expanded in an isentropic turbine. The work produced is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 The process is isentropic (i.e., reversible-adiabatic).

Analysis There is one inlet and two exits. We take the turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Steam turbine

4 MPa

5 kg/s

50 kPa

100°C

700 kPa

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m1h1=m2h2+m3h3+WoutWout=m1h1−m2h2−m3h3

From a mass balance,

T

s

3

1

4 MPa

50 kPa

0.7 MPa

2

{ dot 2 { dot 1

Noting that the expansion process is isentropic, the enthalpies at three states are determined as follows:

33

113

223

Substituting,

Wout=m1h1−m2h2−m3h3(5 kg/s)(3979.3 kJ/kg)−(0.25 kg/s)(3309.1 kJ/kg)−(4.75 kg/s)(2682.4 kJ/kg)6328 kW

8-61 Heat is added to a pressure cooker that is maintained at a specified pressure. The minimum entropy change of the thermal-energy reservoir supplying this heat is to be determined.

Assumptions 1 Only water vapor escapes through the pressure relief valve.

Analysis According to the conservation of mass principle,

dmCVdt=inm−outmdmdt { dot out

An entropy balance adapted to this system becomes

dSsurrdt+d(ms)dt+mouts≥0

When this is combined with the mass balance, it becomes

dSsurrdt+d(ms)dt−sdmdt≥0

Multiplying by dt and integrating the result yields

ΔSsurr+m2s2−m1s1−sout(m2−m1)≥0

The properties at the initial and final states are (from Table A-5 at 200 kPa)

v1=vf+xvfg=0.001061+(0.10)(0.88578−0.001061)=0.08953 m3/kgs1=sf+xsfg=1.5302+(0.10)(5.5968)=2.0899 kJ/kg⋅Kv2=vf+xvfg=0.001061+(0.50)(0.88578−0.001061)=0.4434 m3/kgs2=sf+xsfg=1.5302+(0.50)(5.5968)=4.3286 kJ/kg⋅K

The initial and final masses are

m1=Vv1=Vvf+xvfg=0.1 m30.08953 m3/kg=1.117 kg

m2=Vv2=Vvf+xvfg=0.1 m30.4434 m3/kg=0.2255 kg

The entropy of escaping water vapor is

sout=sg @ 200 kPa=7.1270 kJ/kg⋅K

Substituting,

ΔSsurr+m2s2−m1s1−sout(m2−m1)≥0ΔSsurr+(0.2255)(4.3286)−(1.117)(2.0899)−(7.1270)(0.2255−1.117)≥0ΔSsurr+4.995≥0

The entropy change of the thermal energy reservoir must then satisfy

ΔSsurr≥−4.995 kJ/K

8-62 Heat is added to a pressure cooker that is maintained at a specified pressure. Work is also done on water. The minimum entropy change of the thermal-energy reservoir supplying this heat is to be determined.

Assumptions 1 Only water vapor escapes through the pressure relief valve.

Analysis According to the conservation of mass principle,

dmCVdt=inm−outmdmdt { dot out

An entropy balance adapted to this system becomes

dSsurrdt+d(ms)dt+mouts≥0

When this is combined with the mass balance, it becomes

dSsurrdt+d(ms)dt−sdmdt≥0

Multiplying by dt and integrating the result yields

ΔSsurr+m2s2−m1s1−sout(m2−m1)≥0

The properties at the initial and final states are (from Table A-5 at 200 kPa)

v1=vf+xvfg=0.001061+(0.10)(0.88578−0.001061)=0.08953 m3/kgs1=sf+xsfg=1.5302+(0.10)(5.5968)=2.0899 kJ/kg⋅Kv2=vf+xvfg=0.001061+(0.50)(0.88578−0.001061)=0.4434 m3/kgs2=sf+xsfg=1.5302+(0.50)(5.5968)=4.3286 kJ/kg⋅K

The initial and final masses are

m1=Vv1=Vvf+xvfg=0.1 m30.08953 m3/kg=1.117 kg

m2=Vv2=Vvf+xvfg=0.1 m30.4434 m3/kg=0.2255 kg

The entropy of escaping water vapor is

sout=sg @ 200 kPa=7.1270 kJ/kg⋅K

Substituting,

ΔSsurr+m2s2−m1s1−sout(m2−m1)≥0ΔSsurr+(0.2255)(4.3286)−(1.117)(2.0899)−(7.1270)(0.2255−1.117)≥0ΔSsurr+4.995≥0

The entropy change of the thermal energy reservoir must then satisfy

ΔSsurr≥−4.995 kJ/K

8-63 A cylinder is initially filled with saturated water vapor mixture at a specified temperature. Steam undergoes a reversible heat addition and an isentropic process. The processes are to be sketched and heat transfer for the first process and work done during the second process are to be determined.

Assumptions 1 The kinetic and potential energy changes are negligible.  2 The thermal energy stored in the cylinder itself is negligible. 3 Both processes are reversible.

H2O

100C

x = 0.5

Q

Analysis (b) From the steam tables (Tables A-4 through A-6),

T1=100°C h1=hf+xhfg=419.17+(0.5)(2256.4)=1547.4 kJ/kgT2=100°C h2=hg=2675.6  kJ/kgu2=ug=2506.0  kJ/kgs2=sg=7.3542 kJ/kg⋅K

We take the contents of the cylinder as the system. This is a closed system since no mass enters or leaves. The energy balance for this closed system can be expressed as

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin−Wb,out=ΔU=m(u2−u1)

For process 1-2, it reduces to

Q12,in=m(h2−h1)=(5 kg)(2675.6-1547.4)kJ/kg=5641 kJ

(c) For process 2-3, it reduces to

W23,b,out=m(u2−u3)=(5 kg)(2506.0-2247.9)kJ/kg=1291 kJ

8-64 Steam expands in an adiabatic turbine. Steam leaves the turbine at two different pressures. The process is to be sketched on a T-s diagram and the work done by the steam per unit mass of the steam at the inlet are to be determined.

Turbine

P1 = 6 MPa

T1 = 500C

P3 = 10 kPa

P2 = 1 MPa

Assumptions 1 The kinetic and potential energy changes are negligible.

Analysis (b) From the steam tables (Tables A-4 through A-6),

T1=500°C h1=3423.1 kJ/kgs1=6.8826 kJ/kg⋅K      P2=1 MPa h2s=2921.3 kJ/kgP3=10 kPa h3s=2179.6 kJ/kgx3s=0.831

A mass balance on the control volume gives

m1=m2+m3   where	m2=0.1m1m3=0.9m1

We take the turbine as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein=Eoutm1h1=Ws,out+m2h2+m3h3m1h1=Ws,out+0.1m1h2+0.9m1h3

or

h1=ws,out+0.1h2+0.9h3ws,out=h1−0.1h2−0.9h33423.1−(0.1)(2921.3)−(0.9)(2179.6)=1169.3 kJ/kg

The actual work output per unit mass of steam at the inlet is

wout=ηTws,out=(0.85)(1169.3 kJ/kg)=993.9 kJ/kg

8-65E An insulated rigid can initially contains R-134a at a specified state. A crack develops, and refrigerant escapes slowly. The final mass in the can is to be determined when the pressure inside drops to a specified value.

Assumptions 1 The can is well-insulated and thus heat transfer is negligible.  2 The refrigerant that remains in the can underwent a reversible adiabatic process.

R-134

140 psia

70°F

Leak

Analysis Noting that for a reversible adiabatic (i.e., isentropic) process, s1 = s2, the properties of the refrigerant in the can are (Tables A-11E through A-13E)

P1=140  psia  s1≃sf @ 70°F=0.07306  Btu/lbm⋅RP2=20  psia  x2=s2−sfsfg=0.07306−0.026050.19962=0.2355v2=vf+x2vfg=0.01182+0.23552.2772−0.01182=0.5453 ft3/lbm

Thus the final mass of the refrigerant in the can is

m=Vv2=1.2  ft30.5453  ft3/lbm=2.201   lbm

## Entropy Change of Incompressible Substances

8-66C  No, because entropy is not a conserved property.

8-67 A hot copper block is dropped into water in an insulated tank. The final equilibrium temperature of the tank and the total entropy change are to be determined.

Assumptions 1 Both the water and the copper block are incompressible substances with constant specific heats at room temperature. 2 The system is stationary and thus the kinetic and potential energies are negligible. 3 The tank is well-insulated and thus there is no heat transfer.

Properties The density and specific heat of water at 25C are  = 997 kg/m3 and cp = 4.18 kJ/kg.C. The specific heat of copper at 27C is  cp = 0.386 kJ/kg.C (Table A-3).

Analysis We take the entire contents of the tank, water + copper block, as the system. This is a closed system since no mass crosses the system boundary during the process. The energy balance for this system can be expressed as

WATER

Copper

50 kg

120 L

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energies0=ΔU

or,

ΔUCu+ΔUwater=0

[mc(T2−T1)]Cu+[mc(T2−T1)]water=0

where

mwater=ρV=(997  kg/m3)(0.120  m3)=119.6  kg

Using specific heat values for copper and liquid water at room temperature and substituting,

(50  kg)(0.386  kJ/kg⋅°C)(T2−80)°C+(119.6  kg)(4.18  kJ/kg⋅°C)(T2−25)°C=0

T2 = 27.0C

The entropy generated during this process is determined from

ΔScopper=mcavglnT2T1=50  kg0.386  kJ/kg⋅K ln300.0  K353   K=−3.140  kJ/KΔSwater=mcavglnT2T1=119.6  kg4.18  kJ/kg⋅K ln300.0  K298  K=3.344  kJ/K

Thus,

ΔStotal=ΔScopper+ΔSwater=−3.140+3.344=0.204  kJ/K

8-68 Computer chips are cooled by placing them in saturated liquid R-134a. The entropy changes of the chips, R-134a, and the entire system are to be determined.

Assumptions 1 The system is stationary and thus the kinetic and potential energy changes are zero. 2 There are no work interactions involved. 3 There is no heat transfer between the system and the surroundings.

Analysis (a) The energy balance for this system can be expressed as

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energies0=ΔU=m(u2−u1)chips+m(u2−u1)R-134am(u1−u2)chips=m(u2−u1)R-134a

The heat released by the chips is

Qchips=mc(T1−T2)=(0.010 kg)(0.3 kJ/kg⋅K)20−(−40) K=0.18 kJ

The mass of the refrigerant vaporized during this heat exchange process is

mg,2=QR−134aug−uf=QR−134aufg@−40°C=0.18 kJ207.40 kJ/kg=0.0008679 kg

Only a small fraction of R-134a is vaporized during the process. Therefore, the temperature of R-134a remains constant during the process. The change in the entropy of the R-134a is (at -40F from Table A-11)

ΔSR−134a=mg,2sg,2+mf,2sf,2−mf,1sf,1(0.0008679)(0.96866)+(0.005−0.0008679)(0)−(0.005)(0)0.000841 kJ/K

(b) The entropy change of the chips is

ΔSchips=mclnT2T1=(0.010 kg)(0.3 kJ/kg⋅K)ln(−40+273)K(20+273)K=−0.000687 kJ/K

(c) The total entropy change is

ΔStotal=Sgen=ΔSR-134a+ΔSchips=0.000841+(−0.000687)=0.000154 kJ/K

The positive result for the total entropy change (i.e., entropy generation) indicates that this process is possible.

8-69 A hot iron block is dropped into water in an insulated tank. The total entropy change during this process is to be determined.

Assumptions 1 Both the water and the iron block are incompressible substances with constant specific heats at room temperature. 2 The system is stationary and thus the kinetic and potential energies are negligible. 3 The tank is well-insulated and thus there is no heat transfer. 4 The water that evaporates, condenses back.

Properties The specific heat of water at 25C is cp = 4.18 kJ/kg.C. The specific heat of iron at room temperature is  cp = 0.45 kJ/kg.C (Table A-3).

Analysis We take the entire contents of the tank, water + iron block, as the system. This is a closed system since no mass crosses the system boundary during the process. The energy balance for this system can be expressed as

WATER

18C

Iron

350C

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energies0=ΔU

or,

ΔUiron+ΔUwater=0

[mc(T2−T1)]iron+[mc(T2−T1)]water=0

Substituting,

(25  kg)(0.45  kJ/kg⋅K)(T2−350C)+(100  kg)(4.18  kJ/kg⋅K)(T2−18C)=02

The entropy generated during this process is determined from

ΔSiron=mcavglnT2T1=25  kg0.45  kJ/kg⋅K ln299.7  K623  K=−8.232  kJ/KΔSwater=mcavglnT2T1=100  kg4.18  kJ/kg⋅K ln299.7  K291  K=12.314 kJ/K

Thus,

Sgen=ΔStotal=ΔSiron+ΔSwater=−8.232+12.314=4.08  kJ/K

Discussion The results can be improved somewhat by using specific heats at average temperature.

8-70 An aluminum block is brought into contact with an iron block in an insulated enclosure. The final equilibrium temperature and the total entropy change for this process  are to be determined.

Assumptions 1 Both the aluminum and the iron block are incompressible substances with constant specific heats. 2 The system is stationary and thus the kinetic and potential energies are negligible. 3 The system is well-insulated and thus there is no heat transfer.

Properties The specific heat of aluminum at the anticipated average temperature of 450 K is cp = 0.973 kJ/kg.C. The specific heat of iron at room temperature (the only value available in the tables) is  cp = 0.45 kJ/kg.C (Table A-3).

Analysis We take the iron+aluminum blocks as the system, which is a closed system. The energy balance for this system can be expressed as

Iron

20 kg

100C

Aluminum

20 kg

200C

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energies0=ΔU

or,

ΔUalum+ΔUiron=0[mc(T2−T1)]alum+[mc(T2−T1)]iron=0

Substituting,

(20  kg)(0.45  kJ/kg⋅K)(T2−100C)+(20  kg)(0.973 kJ/kg⋅K)(T2−200C)=02

The total entropy change for this process is determined from

ΔSiron=mcavglnT2T1=20  kg0.45  kJ/kg⋅K ln441.4  K373  K=1.515  kJ/KΔSalum=mcavglnT2T1=20  kg0.973  kJ/kg⋅K ln441.4  K473  K=−1.346  kJ/K

Thus,

ΔStotal=ΔSiron+ΔSalum=1.515−1.346=0.169 kJ/K

8-71 EES Problem 8-70 is reconsidered. The effect of the mass of the iron block on the final equilibrium temperature and the total entropy change for the process is to be studied. The mass of the iron is to vary from 1  to 10 kg. The equilibrium temperature and the total entropy change are to be plotted as a function of iron mass.

Analysis The problem is solved using EES, and the results are tabulated and plotted below.

"Knowns:"

T_1_iron = 100 [C]

{m_iron = 20 [kg]}

T_1_al = 200 [C]

m_al = 20 [kg]

C_al = 0.973 [kJ/kg-K]  "FromTable A-3 at the anticipated average temperature of 450 K."

C_iron= 0.45 [kJ/kg-K]  "FromTable A-3 at room temperature, the only value available."

"Analysis: "

" Treat the iron plus aluminum as a closed system, with no heat transfer in, no work out, neglect changes in KE and PE of the system.  "

"The final temperature is found from the energy balance."

E_in - E_out = DELTAE_sys

E_out = 0

E_in = 0

DELTAE_sys = m_iron*DELTAu_iron  + m_al*DELTAu_al

DELTAu_iron = C_iron*(T_2_iron - T_1_iron)

DELTAu_al = C_al*(T_2_al - T_1_al)

"the iron and aluminum reach thermal equilibrium:"

T_2_iron = T_2

T_2_al = T_2

DELTAS_iron = m_iron*C_iron*ln((T_2_iron+273) / (T_1_iron+273))

DELTAS_al = m_al*C_al*ln((T_2_al+273) / (T_1_al+273))

DELTAS_total = DELTAS_iron + DELTAS_al

| Stotal [kJ/kg] | miron [kg] | T2 [C] |
| --- | --- | --- |
| 0.01152 | 1 | 197.7 |
| 0.0226 | 2 | 195.6 |
| 0.03326 | 3 | 193.5 |
| 0.04353 | 4 | 191.5 |
| 0.05344 | 5 | 189.6 |
| 0.06299 | 6 | 187.8 |
| 0.07221 | 7 | 186.1 |
| 0.08112 | 8 | 184.4 |
| 0.08973 | 9 | 182.8 |
| 0.09805 | 10 | 181.2 |

8-72 An iron block and a copper block are dropped into a large lake. The total amount of entropy change when both blocks cool to the lake temperature is to be determined.

Assumptions 1 The water, the iron block and the copper block are incompressible substances with constant specific heats at room temperature. 2 Kinetic and potential energies are negligible.

Properties The specific heats of iron and copper at room temperature are ciron = 0.45 kJ/kg.C and ccopper = 0.386 kJ/kg.C (Table A-3).

Analysis The thermal-energy capacity of the lake is very large, and thus the temperatures of both the iron and the copper blocks will drop to the lake temperature (15C) when the thermal equilibrium is established.  Then the entropy changes of the blocks become

ΔSiron=mcavglnT2T1=50  kg0.45  kJ/kg⋅Kln288  K353  K=−4.579  kJ/KΔScopper=mcavglnT2T1=20  kg0.386  kJ/kg⋅Kln288  K353  K=−1.571  kJ/K

Iron

50 kg

80C

Copper

20 kg

80C

Lake

15C

We take both the iron and the copper blocks, as the system. This is a closed system since no mass crosses the system boundary during the process. The energy balance for this system can be expressed as

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energies−Qout=ΔU=ΔUiron+ΔUcopper

or,

Qout=[mc(T1−T2)]iron+[mc(T1−T2)]copper

Substituting,

Qout=50  kg0.45  kJ/kg⋅K353−288K+20  kg0.386  kJ/kg⋅K353−288K1964  kJ

Thus,

ΔSlake=Qlake,inTlake=1964  kJ288  K=6.820  kJ/K

Then the total entropy change for this process is

ΔStotal=ΔSiron+ΔScopper+ΔSlake=−4.579−1.571+6.820=0.670 kJ/K

8-73 An adiabatic pump is used to compress saturated liquid water in a reversible manner. The work input is to be determined by different approaches.

Assumptions 1 Steady operating conditions exist. 2 Kinetic and potential energy changes are negligible. 3 Heat transfer to or from the fluid is negligible.

Analysis The properties of water at the inlet and exit of the pump are (Tables A-4 through A-6)

10 kPa

15 MPa

pump

P1=10  kPa h1=191.81 kJ/kgs1=0.6492 kJ/kgv1=0.001010 m3/kgP2=15 MPa  h2=206.90 kJ/kgv2=0.001004 m3/kg

(a) Using the entropy data from the compressed liquid water table

wP=h2−h1=206.90−191.81=15.10 kJ/kg

(b) Using inlet specific volume and pressure values

wP=v1(P2−P1)=(0.001010 m3/kg)(15,000−10)kPa=15.14 kJ/kg

Error = 0.3%

(b) Using average specific volume and pressure values

wP=vavg(P2−P1)=12(0.001010+0.001004) m3/kg(15,000−10)kPa=15.10 kJ/kg

Error = 0%

Discussion The results show that any of the method may be used to calculate reversible pump work.

# Entropy Changes of Ideal Gases

8-74C  For ideal gases, cp = cv + R and

P2V2T2=P1V1T1  V2V1=T2P1T1P2

Thus,

s2−s1=cvlnT2T1+RlnV2V1vT2T1

8-75C  For an ideal gas, dh = cp dT and v = RT/P.  From the second Tds relation,

ds=dhT−vdPT=cpdPT−RTPdPT=cpdTT−RdPP

Integrating,

s2−s1=cplnT2T1−RlnP2P1

Since cp is assumed to be constant.

8-76C  No.  The entropy of an ideal gas depends on the pressure as well as the temperature.

8-77C  Setting s = 0 gives

cplnT2T1−RlnP2P1=0  lnT2T1=RcplnP2P1  T2T1=P2P1R/Cp

But

Rcp=cp−cvcp=1−1k=k−1k      since       k=cpcv. Thus,      T2T1=P2P1k−1/k

8-78C  The Pr and vr are called relative pressure and relative specific volume, respectively.  They are derived for isentropic processes of ideal gases, and thus their use is limited to isentropic processes only.

8-79C  The entropy of a gas can change during an isothermal process since entropy of an ideal gas depends on the pressure as well as the temperature.

8-80C  The entropy change relations of an ideal gas simplify to

s  =  cp ln(T2/T1) for a constant pressure process

and	s  =  cv ln(T2/T1) for a constant volume process.

Noting that cp  > cv, the entropy change will be larger for a constant pressure process.

8-81 The entropy difference between the two states of oxygen is to be determined.

Assumptions Oxygen is an ideal gas with constant specific heats.

Properties The specific heat of oxygen at the average temperature of (39+337)/2=188C=461 K is cp = 0.960 kJ/kgK (Table A-2b).

Analysis From the entropy change relation of an ideal gas,

Δsoxygen=cplnT2T1−RlnP2P1=(0.960 kJ/kg⋅K)ln(337+273)K(39+273)K−0=0.6436 kJ/kg⋅K

since the pressure is same at the initial and final states.

8-82 The entropy changes of helium and nitrogen is to be compared for the same initial and final states.

Assumptions Helium and nitrogen are ideal gases with constant specific heats.

Properties The properties of helium are cp = 5.1926 kJ/kgK, R = 2.0769 kJ/kgK (Table A-2a). The specific heat of nitrogen at the average temperature of (427+27)/2=227C=500 K is cp = 1.056 kJ/kgK (Table A-2b). The gas constant of nitrogen is R = 0.2968 kJ/kgK (Table A-2a).

Analysis From the entropy change relation of an ideal gas,

ΔsHe=cplnT2T1−RlnP2P1(5.1926 kJ/kg⋅K)ln(27+273)K(427+273)K−(2.0769 kJ/kg⋅K)ln200 kPa2000 kPa0.3826 kJ/kg⋅K

ΔsN2=cplnT2T1−RlnP2P1(1.056 kJ/kg⋅K)ln(27+273)K(427+273)K−(0.2968 kJ/kg⋅K)ln200 kPa2000 kPa−0.2113 kJ/kg⋅K

Hence, helium undergoes the largest change in entropy.

8-83E The entropy change of air during an expansion process is to be determined.

Assumptions Air is an ideal gas with constant specific heats.

Properties The specific heat of air at the average temperature of (500+50)/2=275F is cp = 0.243 Btu/lbmR (Table A-2Eb). The gas constant of air is R = 0.06855 Btu/lbmR (Table A-2Ea).

Analysis From the entropy change relation of an ideal gas,

Δsair=cplnT2T1−RlnP2P1(0.243 Btu/lbm⋅R)ln(50+460)R(500+460)R−(0.06855 Btu/lbm⋅R)ln100 psia200 psia−0.1062 Btu/lbm⋅R

8-84 The final temperature of air when it is expanded isentropically is to be determined.

Assumptions Air is an ideal gas with constant specific heats.

Properties The specific heat ratio of air at an anticipated average temperature of 550 K is k = 1.381 (Table A-2b).

Analysis From the isentropic relation of an ideal gas under constant specific heat assumption,

T2=T1P2P1(k−1)k=(477+273 K)100 kPa1000 kPa0.3811.381=397 K

Discussion The average air temperature is (750+397.4)/2=573.7 K, which is sufficiently close to the assumed average temperature of 550 K.

8-85E The final temperature of air when it is expanded isentropically is to be determined.

Assumptions Air is an ideal gas with constant specific heats.

Properties The specific heat ratio of air at an anticipated average temperature of 300F is k = 1.394 (Table A-2Eb).

Analysis From the isentropic relation of an ideal gas under constant specific heat assumption,

T2=T1P2P1(k−1)k=(500+460 R)20 psia100 psia0.3941.394=609 R

Discussion The average air temperature is (960+609)/2=785 R=325F, which is sufficiently close to the assumed average temperature of 300F.

8-86 The final temperatures of helium and nitrogen when they are compressed isentropically are to be compared.

Assumptions Helium and nitrogen are ideal gases with constant specific heats.

Properties The specific heat ratios of helium and nitrogen at room temperature are k = 1.667 and k = 1.4, respectively (Table A-2a).

Analysis From the isentropic relation of an ideal gas under constant specific heat assumption,

T2,He=T1P2P1(k−1)k=(298 K)1000 kPa100 kPa0.6671.667=749 K

T2,N2=T1P2P1(k−1)k=(298 K)1000 kPa100 kPa0.41.4=575 K

Hence, the helium produces the greater temperature when it is compressed.

8-87 The final temperatures of neon and air when they are expanded isentropically are to be compared.

Assumptions Neon and air are ideal gases with constant specific heats.

Properties The specific heat ratios of neon and air at room temperature are k = 1.667 and k = 1.4, respectively (Table A-2a).

Analysis From the isentropic relation of an ideal gas under constant specific heat assumption,

T2,Ne=T1P2P1(k−1)k=(773 K)100 kPa1000 kPa0.6671.667=308 K

T2,air=T1P2P1(k−1)k=(773 K)100 kPa1000 kPa0.41.4=400 K

Hence, the neon produces the smaller temperature when it is expanded.

8-88 An insulated cylinder initially contains air at a specified state. A resistance heater inside the cylinder is turned on, and air is heated for 15 min at constant pressure. The entropy change of air during this process is to be determined for the cases of constant and variable specific heats.

Assumptions At specified conditions, air can be treated as an ideal gas.

Properties The gas constant of air is R = 0.287 kJ/kg.K  (Table A-1).

Analysis  The mass of the air and the electrical work done during this process are

AIR

0.3 m3

120 kPa

17C

We

m=P1V1RT1=120  kPa0.3  m30.287  kPa⋅m3/kg⋅K290  K=0.4325  kge,in { dot e,in0.2  kJ/s

The energy balance for this stationary closed system can be expressed as

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesWe,in−Wb,out=ΔUWe,in=m(h2−h1)≃cp(T2−T1)

since U + Wb = H during a constant pressure quasi-equilibrium process.

(a) Using a constant cp value at the anticipated average temperature of 450 K, the final temperature becomes

Thus,	T2=T1+We,inmcp=290  K+180  kJ0.4325  kg1.02  kJ/kg⋅K=698  K

Then the entropy change becomes

ΔSsys=ms2−s1=mcp, avglnT2T1−RlnP2P1Ã0=mcp, avglnT2T10.4325  kg1.020  kJ/kg⋅K

(b)  Assuming variable specific heats,

We,in=mh2−h1  h2=h1+We,inm=290.16  kJ/kg+180  kJ0.4325  kg=706.34  kJ/kg

From the air table (Table A-21, we read  = 2.5628 kJ/kg·K corresponding to this h2 value.  Then,

ΔSsys=ms2∘−s1∘+RlnP2P1Ã0=ms2∘−s1∘=0.4325  kg2.5628−1.66802 kJ/kg⋅K=0.387 kJ/K

8-89 A cylinder contains N2 gas at a specified pressure and temperature. The gas is compressed polytropically until the volume is reduced by half. The entropy change of nitrogen during this process is to be determined.

Assumptions 1 At specified conditions, N2 can be treated as an ideal gas. 2 Nitrogen has constant specific heats at room temperature.

Properties The gas constant of nitrogen is R = 0.297 kJ/kg.K (Table A-1).  The constant volume specific heat of nitrogen at room temperature is cv = 0.743 kJ/kg.K (Table A-2).

N2

PV 1.3 = C

Analysis From the polytropic relation,

T2T1=v1v2n−1  T2=T1v1v2n−1=300  K21.3−1=369.3  K

Then the entropy change of nitrogen becomes

ΔSN2=mcv, avglnT2T1+RlnV2V1=1.2  kg0.743  kJ/kg⋅K ln369.3  K300  K+0.297  kJ/kg⋅K ln0.5=−0.0617  kJ/K

8-90 EES Problem 8-89 is reconsidered. The effect of varying the polytropic exponent from 1 to 1.4 on the entropy change of the nitrogen is to be investigated, and the processes are to be shown on a common P-v diagram.

Analysis The problem is solved using EES, and the results are tabulated and plotted below.

Function BoundWork(P[1],V[1],P[2],V[2],n)

"This function returns the Boundary Work for the polytropic process.  This function is required

since the expression for boundary work depens on whether n=1 or n<>1"

If n<>1 then

BoundWork:=(P[2]*V[2]-P[1]*V[1])/(1-n)"Use Equation 3-22 when n=1"

else

BoundWork:= P[1]*V[1]*ln(V[2]/V[1]) "Use Equation 3-20 when n=1"

endif

end

n=1

P[1] = 120 [kPa]

T[1] = 27 [C]

m = 1.2 [kg]

V[2]=V[1]/2

Gas$='N2'

MM=molarmass(Gas$)

R=R_u/MM

R_u=8.314 [kJ/kmol-K]

"System:  The gas enclosed in the piston-cylinder device."

"Process: Polytropic expansion or compression, P*V^n = C"

P[1]*V[1]=m*R*(T[1]+273)

P[2]*V[2]^n=P[1]*V[1]^n

W_b = BoundWork(P[1],V[1],P[2],V[2],n)

"Find the temperature at state 2 from the pressure and specific volume."

T[2]=temperature(gas$,P=P[2],v=V[2]/m)

"The entropy at states 1 and 2 is:"

s[1]=entropy(gas$,P=P[1],v=V[1]/m)

s[2]=entropy(gas$,P=P[2],v=V[2]/m)

DELTAS=m*(s[2] - s[1])

"Remove the {} to generate the P-v plot data"

{Nsteps = 10

VP[1]=V[1]

PP[1]=P[1]

Duplicate i=2,Nsteps

VP[i]=V[1]-i*(V[1]-V[2])/Nsteps

PP[i]=P[1]*(V[1]/VP[i])^n

END  }

| S [kJ/kg] | n | Wb [kJ] |
| --- | --- | --- |
| -0.2469 | 1 | -74.06 |
| -0.2159 | 1.05 | -75.36 |
| -0.1849 | 1.1 | -76.69 |
| -0.1539 | 1.15 | -78.05 |
| -0.1229 | 1.2 | -79.44 |
| -0.09191 | 1.25 | -80.86 |
| -0.06095 | 1.3 | -82.32 |
| -0.02999 | 1.35 | -83.82 |
| 0.0009849 | 1.4 | -85.34 |

8-91E A fixed mass of helium undergoes a process from one specified state to another specified state. The entropy change of helium is to be determined for the cases of reversible and irreversible processes.

Assumptions 1 At specified conditions, helium can be treated as an ideal gas. 2 Helium has constant specific heats at room temperature.

He

T1 = 540 R

T2 = 660 R

Properties The gas constant of helium is R = 0.4961 Btu/lbm.R (Table A-1E). The constant volume specific heat of helium is       cv = 0.753 Btu/lbm.R (Table A-2E).

Analysis From the ideal-gas entropy change relation,

ΔSHe=mcv, avelnT2T1+Rlnv2v1

The entropy change will be the same for both cases.

8-92 One side of a partitioned insulated rigid tank contains an ideal gas at a specified temperature and pressure while the other side is evacuated. The partition is removed, and the gas fills the entire tank. The total entropy change during this process is to be determined.

Assumptions The gas in the tank is given to be an ideal gas, and thus ideal gas relations apply.

Analysis Taking the entire rigid tank as the system, the energy balance can be expressed as

IDEAL GAS

5 kmol

40C

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energies0=ΔU=m(u2−u1)u2=u1T2=T1

since u = u(T) for an ideal gas. Then the entropy change of the gas becomes

ΔS=Ncv, avglnT2T1Ã0+RulnV2V1=NRulnV2V15  kmol8.314  kJ/kmol⋅K

This also represents the total entropy change since the tank does not contain anything else, and there are no interactions with the surroundings.

8-93 Air is compressed in a piston-cylinder device in a reversible and adiabatic manner. The final temperature and the work are to be determined for the cases of constant and variable specific heats.

Assumptions 1 At specified conditions, air can be treated as an ideal gas. 2 The process is given to be reversible and adiabatic, and thus isentropic. Therefore, isentropic relations of ideal gases apply.

Properties The gas constant of air is R = 0.287 kJ/kg.K  (Table A-1).  The specific heat ratio of air at low to moderately high temperatures is k = 1.4 (Table A-2).

Analysis (a) Assuming constant specific heats, the ideal gas isentropic relations give

AIR

Reversible

T2=T1P2P1k−1/k=290  K800  kPa100  kPa0.4/1.4=525.3 K

Then,

Tavg=290+525.3/2=407.7 K  cv, avg=0.727 kJ/kg⋅K

We take the air in the cylinder as the system. The energy balance for this stationary closed system can be expressed as

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesWin=ΔU=m(u2−u1)≃mcv(T2−T1)

Thus,

win=cv, avgT2−T1=0.727  kJ/kg⋅K525.3−290 K=171.1 kJ/kg

(b)  Assuming variable specific heats, the final temperature can be determined using the relative pressure data (Table A-21),

and	T1=290  K  Pr1=1.2311u1=206.91  kJ/kgr2P2P1r1800  kPa100  kPa1.2311T2=522.4 Ku2=376.16  kJ/kg

Then the work input becomes

win=u2−u1=376.16−206.91 kJ/kg=169.25 kJ/kg

8-94 EES Problem 8-93 is reconsidered. The work done and final temperature during the compression process are to be calculated and plotted as functions of the final pressure for the two cases as the final pressure varies from 100 kPa to 800 kPa.

Analysis The problem is solved using EES, and the results are tabulated and plotted below.

Procedure ConstPropSol(P_1,T_1,P_2,Gas$:Work_in_ConstProp,T2_ConstProp)

C_P=SPECHEAT(Gas$,T=27)

MM=MOLARMASS(Gas$)

R_u=8.314 [kJ/kmol-K]

R=R_u/MM

C_V = C_P - R

k = C_P/C_V

T2= (T_1+273)*(P_2/P_1)^((k-1)/k)

T2_ConstProp=T2-273 "[C]"

DELTAu = C_v*(T2-(T_1+273))

Work_in_ConstProp = DELTAu

End

"Knowns:"

P_1 = 100 [kPa]

T_1 = 17 [C]

P_2 = 800 [kPa]

"Analysis: "

" Treat the piston-cylinder as a closed system, with no heat transfer in, neglect

changes in KE and PE of the air.  The process is reversible and adiabatic thus isentropic."

"The isentropic work is determined from:"

e_in - e_out = DELTAe_sys

e_out = 0 [kJ/kg]

e_in = Work_in

DELTAE_sys = (u_2 - u_1)

u_1 = INTENERGY(air,T=T_1)

v_1 = volume(air,P=P_1,T=T_1)

s_1 = entropy(air,P=P_1,T=T_1)

" The process is reversible and adiabatic or isentropic.

Then P_2 and s_2 specify state 2."

s_2 = s_1

u_2 = INTENERGY(air,P=P_2,s=s_2)

T_2_isen=temperature(air,P=P_2,s=s_2)

Gas$ = 'air'

Call ConstPropSol(P_1,T_1,P_2,Gas$:

Work_in_ConstProp,T2_ConstProp)

| P2 [kPa] | Workin [kJ/kg] | Workin,ConstProp [kJ/kg] |
| --- | --- | --- |
| 100 | 0 | 0 |
| 200 | 45.63 | 45.6 |
| 300 | 76.84 | 76.77 |
| 400 | 101.3 | 101.2 |
| 500 | 121.7 | 121.5 |
| 600 | 139.4 | 139.1 |
| 700 | 155.2 | 154.8 |
| 800 | 169.3 | 168.9 |

8-95 An insulated rigid tank contains argon gas at a specified pressure and temperature. A valve is opened, and argon escapes until the pressure drops to a specified value. The final mass in the tank is to be determined.

Assumptions 1 At specified conditions, argon can be treated as an ideal gas. 2 The process is given to be reversible and adiabatic, and thus isentropic. Therefore, isentropic relations of ideal gases apply.

ARGON

4 kg

450 kPa

30C

Properties The specific heat ratio of argon is  k = 1.667 (Table A-2).

Analysis From the ideal gas isentropic relations,

T2=T1P2P1k−1/k=303  K200  kPa450  kPa0.667/1.667=219.0 K

The final mass in the tank is determined from the ideal gas relation,

P1VP2V=m1RT1m2RT2  m2=P2T1P1T2m1=200  kPa303  K450  kPa219  K4 kg=2.46  kg

8-96 EES Problem 8-95 is reconsidered. The effect of the final pressure on the final mass in the tank is to be investigated as the pressure varies from 450 kPa to 150 kPa, and the results are to be plotted.

Analysis The problem is solved using EES, and the results are tabulated and plotted below.

"Knowns:"

C_P = 0.5203"[kJ/kg-K ]"

C_V = 0.3122 "[kJ/kg-K ]"

R=0.2081 "[kPa-m^3/kg-K]"

P_1= 450"[kPa]"

T_1 = 30"[C]"

m_1 = 4"[kg]"

P_2= 150"[kPa]"

"Analysis:

We assume the mass that stays in the tank undergoes an isentropic expansion

process.  This allows us to determine the final temperature of that gas at the final

pressure in the tank by using the isentropic relation:"

k = C_P/C_V

T_2 = ((T_1+273)*(P_2/P_1)^((k-1)/k)-273)"[C]"

V_2 = V_1

P_1*V_1=m_1*R*(T_1+273)

P_2*V_2=m_2*R*(T_2+273)

| m2 [kg] | P2 [kPa] |
| --- | --- |
| 2.069 | 150 |
| 2.459 | 200 |
| 2.811 | 250 |
| 3.136 | 300 |
| 3.44 | 350 |
| 3.727 | 400 |
| 4 | 450 |

8-97E Air is accelerated in an adiabatic nozzle. Disregarding irreversibilities, the exit velocity of air is to be determined.

Assumptions 1 Air is an ideal gas with variable specific heats. 2 The process is given to be reversible and adiabatic, and thus isentropic. Therefore, isentropic relations of ideal gases apply. 2 The nozzle operates steadily.

Analysis Assuming variable specific heats, the inlet and exit properties are determined to be

AIR

1

2

and	T1=1000  R  Pr1=12.30h1=240.98  Btu/lbmr2P2P1r112  psia60  psia12.30T2=635.9  Rh2=152.11  Btu/lbm

We take the nozzle as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutm(h1+V122)=m(h2+V22/2)h2−h1+V22−V122=0

Therefore,

V2=2h1−h2+V12=2240.98−152.11 Btu/lbm25,037  ft2/s21  Btu/lbm+200  ft/s22119 ft/s

8-98E Air is expanded in an isentropic turbine. The exit temperature of the air and the power produced are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 The process is isentropic (i.e., reversible-adiabatic).  3 Air is an ideal gas with constant specific heats.

Properties The properties of air at an anticipated average temperature of 600°F are cp = 0.250 Btu/lbm·R and k = 1.377 (Table A-2Eb). The gas constant of air is R = 0.3704 psiaft3/lbmR (Table A-1E).

Analysis There is only one inlet and one exit, and thus m1=m2=m. We take the turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Turbine

150 psia

900°F

500 ft/s

15 psia

100 ft/s

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

T

s

2

1

150 psia

15 psia

mh1+V122=mh2+V222+WoutWout=mh1−h2+V12−V222mcp(T1−T2)+V12−V222

The exit temperature of the air for this isentropic process is

T2=T1P2P1(k−1)k=(900+460 R)15 psia150 psia0.3771.377=724 R

The specific volume of air at the inlet and the mass flow rate are

v1=RT1P1=(0.3704 psia⋅ft3/lbm⋅R)(900+460 R)150 psia=3.358 ft3/lbm

m=A1V1v1=(0.5 ft2)(500 ft/s)3.358 ft3/lbm=74.45 lbm/s

Substituting into the energy balance equation gives

Wout=mcp(T1−T2)+V12−V222(74.45 lbm/s)(0.250 Btu/lbm⋅R)(1360−724)R+(500 ft/s)2−(100 ft/s)221 Btu/lbm25,037 ft2/s212,194 Btu/s1 hp0.7068 Btu/s=17,250 hp

8-99 Nitrogen is compressed in an adiabatic compressor. The  minimum work input is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 The process is adiabatic, and thus there is no heat transfer.  3 Nitrogen is an ideal gas with constant specific heats.

Properties The properties of nitrogen at an anticipated average temperature of 400 K are cp = 1.044 kJ/kg·K and k = 1.397 (Table A-2b).

Analysis There is only one inlet and one exit, and thus m1=m2=m. We take the compressor as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Nitrogen

compressor

600 kPa

120 kPa

30°C

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

T

s

2

1

600 kPa

120 kPa

mh1+Win=mh2Win=m(h2−h1)

For the minimum work input to the compressor, the process must be reversible as well as adiabatic (i.e., isentropic). This being the case, the exit temperature will be

T2=T1P2P1(k−1)k=(303 K)600 kPa120 kPa0.3971.397=479 K

Substituting into the energy balance equation gives

win=h2−h1=cp(T2−T1)=(1.044 kJ/kg⋅K)(479−303)K=184 kJ/kg

8-100 Oxygen is expanded in an adiabatic nozzle. The maximum velocity at the exit is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 The process is adiabatic, and thus there is no heat transfer. 3 Oxygen is an ideal gas with constant specific heats.

Properties The properties of oxygen at room temperature are cp = 0.918 kJ/kg·K and k = 1.395 (Table A-2a).

Analysis For the maximum velocity at the exit, the process must be reversible as well as adiabatic (i.e., isentropic). This being the case, the exit temperature will be

T2=T1P2P1(k−1)k=(363 K)120 kPa300 kPa0.3951.395=280.0 K

There is only one inlet and one exit, and thus m1=m2=m. We take nozzle as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

O2

300 kPa

90C

3 m/s

120 kPa

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

T

s

2

1

300 kPa

120 kPa

mh1+V122=mh2+V222)h1+V122=h2+V222

Solving for the exit velocity,

V2=V12+2(h1−h2)0.5V12+2cp(T1−T2)0.5(3 m/s)2+2(0.918 kJ/kg⋅K)(363−280)K1000 m2/s21 kJ/kg0.5390 m/s

8-101 Air is expanded in an adiabatic nozzle by a polytropic process. The temperature and velocity at the exit are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 There is no heat transfer or shaft work associated with the process. 3 Air is an ideal gas with constant specific heats.

Properties The properties of air at room temperature are cp = 1.005 kJ/kg·K and k = 1.4 (Table A-2a).

Analysis For the polytropic process of an ideal gas, Pvn=Constant, and the exit temperature is given by

T2=T1P2P1(n−1)n=(373 K)200 kPa700 kPa0.31.3=279 K

There is only one inlet and one exit, and thus m1=m2=m. We take nozzle as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Air

700 kPa

100C

30 m/s

200 kPa

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

mh1+V122=mh2+V222)h1+V122=h2+V222

Solving for the exit velocity,

V2=V12+2(h1−h2)0.5V12+2cp(T1−T2)0.5(30 m/s)2+2(1.005 kJ/kg⋅K)(373−279)K1000 m2/s21 kJ/kg0.5436 m/s

8-102 Air is expanded in an adiabatic nozzle by a polytropic process. The temperature and velocity at the exit are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 There is no heat transfer or shaft work associated with the process. 3 Air is an ideal gas with constant specific heats.

Properties The properties of air at room temperature are cp = 1.005 kJ/kg·K and k = 1.4 (Table A-2a).

Analysis For the polytropic process of an ideal gas, Pvn=Constant, and the exit temperature is given by

T2=T1P2P1(n−1)n=(373 K)200 kPa700 kPa0.21.2=303 K

There is only one inlet and one exit, and thus m1=m2=m. We take nozzle as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Air

700 kPa

100C

30 m/s

200 kPa

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

mh1+V122=mh2+V222)h1+V122=h2+V222

Solving for the exit velocity,

V2=V12+2(h1−h2)0.5V12+2cp(T1−T2)0.5(30 m/s)2+2(1.005 kJ/kg⋅K)(373−303)K1000 m2/s21 kJ/kg0.5376 m/s

8-103E Air is charged to an initially evacuated container from a supply line. The minimum temperature of the air in the container after it is filled is to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid at the inlet remains constant. 2 Air is an ideal gas with constant specific heats. 3 Kinetic and potential energies are negligible. 4 There are no work interactions involved. 5 The tank is well-insulated, and thus there is no heat transfer.

Properties The specific heat of air at room temperature is cp = 0.240 Btu/lbm·R (Table A-2Ea).

Analysis We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and entropy balances for this uniform-flow system can be expressed as

Mass balance:

Air

200 psia, 100F

Evacuated

min−mout=Δmsystemmi=m2−m1mi=m2

Entropy balance:

m2s2−m1s1+mese−misi≥0m2s2−misi≥0

Combining the two balances,

m2s2−m2si≥0s2−si≥0

The minimum temperature will result when the equal sign applies. Noting that P2 = Pi, we have

s2−si=cplnT2Ti−RlnP2Pi=0cplnT2Ti=0

Then,

T2=Ti=100°F

8-104 A container filled with liquid water is placed in a room and heat transfer takes place between the container and the air in the room until the thermal equilibrium is established. The final temperature, the amount of heat transfer between the water and the air, and the entropy generation are to be determined.

Assumptions 1 Kinetic and potential energy changes are negligible. 2 Air is an ideal gas with constant specific heats. 3 The room is well-sealed and there is no heat transfer from the room to the surroundings. 4 Sea level atmospheric pressure is assumed. P = 101.3 kPa.

Properties The properties of air at room temperature are R = 0.287 kPa.m3/kg.K, cp = 1.005 kJ/kg.K, cv = 0.718 kJ/kg.K. The specific heat of water at room temperature is cw = 4.18 kJ/kg.K (Tables A-2, A-3).

Analysis (a) The mass of the air in the room is

Water

45 kg

95C

Room

90 m3

12C

ma=PVRTa1=(101.3 kPa)(90 m3)(0.287  kPa⋅m3/kg⋅K)(12+273  K)=111.5 kg

An energy balance on the system that consists of the water in the container and the air in the room gives the final equilibrium temperature

0=mwcw(T2−Tw1)+macv(T2−Ta1)0=(45 kg)(4.18 kJ/kg.K)(T2−95)+(111.5 kg)(0.718 kJ/kg.K)(T2−12)T2=70.2°C

(b) The heat transfer to the air is

Q=macv(T2−Ta1)=(111.5 kg)(0.718 kJ/kg.K)(70.2−12)=4660 kJ

(c) The entropy generation associated with this heat transfer process may be obtained by calculating total entropy change, which is the sum of the entropy changes of water and the air.

ΔSw=mwcwlnT2Tw1=(45 kg)(4.18 kJ/kg.K)ln(70.2+273) K(95+273) K=−13.11 kJ/K

P2=maRT2V=(111.5 kg)(0.287  kPa⋅m3/kg⋅K)(70.2+273  K)(90 m3)=122 kPa

ΔSa=macplnT2Ta1−RlnP2P1(111.5 kg)(1.005 kJ/kg.K)ln(70.2+273) K(12+273) K−(0.287 kJ/kg.K)ln122 kPa101.3 kPa=14.88 kJ/K	Sgen=ΔStotal=ΔSw+ΔSa=−13.11+14.88=1.77 kJ/K

8-105 Air is accelerated in an isentropic nozzle. The maximum velocity at the exit is to be determined.

Assumptions 1 Air is an ideal gas with constant specific heats. 2 The nozzle operates steadily.

Properties The properties of air at room temperature are cp = 1.005 kJ/kg.K, k = 1.4 (Table A-2a).

Analysis The exit temperature is determined from ideal gas isentropic relation to be,

T2=T1P2P1(k−1)k=400+273  K100  kPa800  kPa0.4/1.4=371.5 K

We take the nozzle as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

AIR

1

2

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutm(h1+V122)=m(h2+V22/2)0=h2−h1+V22−020=cp(T2−T1)+V222

Therefore,

V2=2cp(T1−T2)=2(1.005 kJ/kg.K)(673-371.5)K1000 m2/s21 kJ/kg=778.5 m/s

8-106 An ideal gas is compressed in an isentropic compressor. 10% of gas is compressed to 400 kPa and 90% is compressed to 600 kPa. The compression process is to be sketched, and the exit temperatures at the two exits, and the mass flow rate into the compressor are to be determined.

Assumptions 1 The compressor operates steadily. 2 The process is reversible-adiabatic (isentropic)

Properties The properties of ideal gas are given to be cp = 1.1 kJ/kg.K and cv = 0.8 kJ/kg.K.

Analysis (b) The specific heat ratio of the gas is

COMPRESSOR

P3 = 600 kPa

P1 = 100 kPa

T1 = 300 K

32 kW

P2 = 400 kPa

k=cpcv=1.10.8=1.375

The exit temperatures are determined from ideal gas isentropic relations to be,

T2=T1P2P1(k−1)k=27+273  K400  kPa100  kPa0.375/1.375=437.8  K

T3=T1P3P1(k−1)k=27+273  K600  kPa100  kPa0.375/1.375=489.0  K

(c) A mass balance on the control volume gives

P3

P2

P1

T

s

m1=m2+m3

where

m2=0.1m1m3=0.9m1

We take the compressor as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutm1h1+Win=m2h2+m3h3m1cpT1+Win=0.1m1cpT2+0.9m1cpT3

Solving for the inlet mass flow rate, we obtain

m1=Wincp0.1(T2−T1)+0.9(T3−T1)32 kW(1.1 kJ/kg⋅K)0.1(437.8-300)+0.9(489.0-300)0.158 kg/s

8-107 Air contained in a constant-volume tank s cooled to ambient temperature. The entropy changes of the air and the universe due to this process are to be determined and the process is to be sketched on a T-s diagram.

Air

5 kg

327C

100 kPa

Assumptions 1 Air is an ideal gas with constant specific heats.

Properties The specific heat of air at room temperature is             cv = 0.718 kJ/kg.K (Table A-2a).

Analysis (a) The entropy change of air is determined from

1

T

s

2

1

2

air

surr

327ºC

27ºC

ΔSair=mcvlnT2T1(5 kg)(0.718 kJ/kg.K)ln(27+273) K(327+273) K−2.488 kJ/K

(b) An energy balance on the system gives

Qout=mcv(T1−T2)(5 kg)(0.718 kJ/kg.K)(327−27)1077 kJ

The entropy change of the surroundings is

Δssurr=QoutTsurr=1077  kJ300  K=3.59  kJ/K

The entropy change of universe due to this process is

Sgen=ΔStotal=ΔSair+ΔSsurr=−2.488+3.59=1.10 kJ/K

# Reversible Steady-Flow Work

8-108C  The work associated with steady-flow devices is proportional to the specific volume of the gas. Cooling a gas during compression will reduce its specific volume, and thus the power consumed by the compressor.

8-109C  Cooling the steam as it expands in a turbine will reduce its specific volume, and thus the work output of the turbine.  Therefore, this is not a good proposal.

8-110C  We would not support this proposal since the steady-flow work input to the pump is proportional to the specific volume of the liquid, and cooling will not affect the specific volume of a liquid significantly.

8-111 Air is compressed isothermally in a reversible steady-flow device. The work required is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 There is no heat transfer associated with the process. 3 Kinetic and potential energy changes are negligible. 4 Air is an ideal gas with constant specific heats.

Compressor

120 psia

75°F

Air

16 psia

75°F

Properties The gas constant of air is R = 0.06855 Btu/lbm·R (Table A-1E).

Analysis Substituting the ideal gas equation of state into the reversible steady-flow work expression gives

win=12vdP=RT12dPP=RTlnP2P1(0.06855 Btu/lbm⋅R)(75+460 K)ln120 psia16 psia73.9 Btu/lbm

8-112 Saturated water vapor is compressed in a reversible steady-flow device. The work required is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 There is no heat transfer associated with the process. 3 Kinetic and potential energy changes are negligible.

Analysis The properties of water at the inlet state are

Compressor

1 MPa

Water

150°C

sat. vap.

11

Noting that the specific volume remains constant, the reversible steady-flow work expression gives

win=12vdP=v1(P2−P1)(0.39248 m3/kg)(1000-476.16)kPa1 kJ1 kPa⋅m3205.6 kJ/kg

8-113 The work produced for the process 1-3 shown in the figure is to be determined.

P (kPa)

v (m3/kg)

2

1

500

400

100

0.5             1.0

3

Assumptions  Kinetic and potential energy changes are negligible.

Analysis The work integral represents the area to the left of the reversible process line. Then,

win,1-3=12vdP+23vdPv1+v22(P2−P1)+v2(P3−P2)(0.5+1.0)m3/kg2(100−500)kPa+(1.0 m3/kg)(400-100)kPa0 kJ/kg

8-114E The work produced for the process 1-2 shown in the figure is to be determined.

Assumptions  Kinetic and potential energy changes are negligible.

P (psia)

v (ft3/lbm)

2

1

500

100

0.1           1.7

Analysis The work integral represents the area to the left of the reversible process line. Then,

win,1-2=12vdPv1+v22(P2−P1)(0.1+1.7)ft3/lbm2(500−100)psia1 Btu5.404 psia⋅ft366.6 Btu/lbm

8-115  Liquid water is to be pumped  by a 25-kW pump at a specified rate. The highest pressure the water can be pumped to is to be determined.

Assumptions 1 Liquid water is an incompressible substance. 2 Kinetic and potential energy changes are negligible. 3 The process is assumed to be reversible since we will determine the limiting case.

Properties The specific volume of liquid water is given to be v1 = 0.001 m3/kg.

Analysis The highest pressure the liquid can have at the pump exit can be determined from the reversible steady-flow work relation for a liquid,

PUMP

P2

100 kPa

25 kW

Thus,	Win=mvdP+ΔkeÃ0+ΔpeÃ0=mv1P2−P1321  kJ1  kPa⋅m3

It yields

P2=5100  kPa

8-116 A steam power plant operates between the pressure limits of 10 MPa and 20 kPa. The ratio of the turbine work to the pump work is to be determined.

Assumptions 1 Liquid water is an incompressible substance. 2 Kinetic and potential energy changes are negligible. 3 The process is reversible. 4 The pump and the turbine are adiabatic.

Properties The specific volume of saturated liquid water at 20 kPa is v1 = vf @ 20 kPa = 0.001017 m3/kg (Table A-5).

Analysis  Both the compression and expansion processes are reversible and adiabatic, and thus isentropic,

s1 = s2 and s3 = s4.  Then the properties of the steam are

H2O

3

4

H2O

2

1

P4=20  kPa  h4=hg @ 20  kPa=2608.9  kJ/kgs4=sg @ 20  kPa=7.9073  kJ/kg⋅KP3=10  MPa 3

Also, v1 = vf @ 20 kPa = 0.001017 m3/kg.

The work output to this isentropic turbine is determined from the steady-flow energy balance to be

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutmh3=mh4+WoutWout=m(h3−h4)

Substituting,

wturb, out=h3−h4=4707.2−2608.9=2098.3  kJ/kg

The pump work input is determined from the steady-flow work relation to be

wpump,in=v dP+ΔkeÃ0+ΔpeÃ0=v1P2−P1(0.001017  m3/kg)(10,000−20) kPa 1  kJ1 kPa⋅m310.15  kJ/kg

Thus,

wturb, outwpump, in=2098.3 10.15 =206.7

8-117 EES Problem 8-116 is reconsidered. The effect of the quality of the steam at the turbine exit on the net work output is to be investigated as the quality is varied from 0.5 to 1.0, and the net work output us to be plotted as a function of this quality.

Analysis The problem is solved using EES, and the results are tabulated and plotted below.

"System: control volume for the pump and turbine"

"Property relation: Steam functions"

"Process: For Pump and Turbine: Steady state, steady flow, adiabatic, reversible or isentropic"

"Since we don't know the mass, we write the conservation of energy per unit mass."

"Conservation of mass: m_dot[1]= m_dot[2]"

"Knowns:"

WorkFluid$ = 'Steam_IAPWS'

P[1] = 20  [kPa]

x[1] = 0

P[2] = 10000 [kPa]

x[4] = 1.0

"Pump Analysis:"

T[1]=temperature(WorkFluid$,P=P[1],x=0)

v[1]=volume(workFluid$,P=P[1],x=0)

h[1]=enthalpy(WorkFluid$,P=P[1],x=0)

s[1]=entropy(WorkFluid$,P=P[1],x=0)

s[2] = s[1]

h[2]=enthalpy(WorkFluid$,P=P[2],s=s[2])

T[2]=temperature(WorkFluid$,P=P[2],s=s[2])

"The Volume function has the same form for an ideal gas as for a real fluid."

v[2]=volume(WorkFluid$,T=T[2],p=P[2])

"Conservation of Energy - SSSF energy balance for pump"

"       -- neglect the change in potential energy, no  heat transfer:"

h[1]+W_pump = h[2]

"Also the work of pump can be obtained from the incompressible fluid, steady-flow result:"

W_pump_incomp = v[1]*(P[2] - P[1])

"Conservation of Energy - SSSF energy balance for turbine -- neglecting the change in potential energy, no  heat transfer:"

P[4] = P[1]

P[3] = P[2]

h[4]=enthalpy(WorkFluid$,P=P[4],x=x[4])

s[4]=entropy(WorkFluid$,P=P[4],x=x[4])

T[4]=temperature(WorkFluid$,P=P[4],x=x[4])

s[3] = s[4]

h[3]=enthalpy(WorkFluid$,P=P[3],s=s[3])

T[3]=temperature(WorkFluid$,P=P[3],s=s[3])

h[3] = h[4] + W_turb

W_net_out  = W_turb - W_pump

| Wnet,out [kJ/kg] | Wpump [kJ/kg] | Wpump,incomp [kJ/kg] | Wturb [kJ/kg] | x4 |
| --- | --- | --- | --- | --- |
| 557.1 | 10.13 | 10.15 | 567.3 | 0.5 |
| 734.7 | 10.13 | 10.15 | 744.8 | 0.6 |
| 913.6 | 10.13 | 10.15 | 923.7 | 0.7 |
| 1146 | 10.13 | 10.15 | 1156 | 0.8 |
| 1516 | 10.13 | 10.15 | 1527 | 0.9 |
| 2088 | 10.13 | 10.15 | 2098 | 1 |

8-118 Liquid water is pumped  by a 70-kW pump to a specified pressure at a specified level. The highest possible mass flow rate of water is to be determined.

PUMP

P2 = 5 MPa

Water

P1 = 120 kPa

Assumptions 1 Liquid water is an incompressible substance. 2 Kinetic energy changes are negligible, but potential energy changes may be significant. 3 The process is assumed to be reversible since we will determine the limiting case.

Properties The specific volume of liquid water is given to be                  v1 = 0.001 m3/kg.

Analysis The highest mass flow rate will be realized when the entire process is reversible.  Thus it is determined from the reversible steady-flow work relation for a liquid,

Thus,	Win=mv dP+ΔkeÃ0+Δpe=mvP2−P1+gz2−z1 { dot (0.001  m3/kg)(5000−120) kPa1  kJ1  kPa⋅m3+(9.8  m/s2)(10  m)1  kJ/kg1000  m2/s2

It yields

m=1.41  kg/s

8-119E Helium gas is compressed from a specified state to a specified pressure at a specified rate. The power input to the compressor is to be determined for the cases of isentropic, polytropic, isothermal, and two-stage compression.

Assumptions 1 Helium is an ideal gas with constant specific heats. 2 The process is reversible. 3 Kinetic and potential energy changes are negligible.

He

2

1

W

·

5 ft3/s

Properties The gas constant of helium is R = 2.6805 psia.ft3/lbm.R = 0.4961 Btu/lbm.R. The specific heat ratio of helium is k = 1.667 (Table A-2E).

Analysis  The mass flow rate of helium is

m=P1V1RT1=14  psia5  ft3/s2.6805  psia⋅ft3/lbm⋅R530  R=0.0493  lbm/s

(a)  Isentropic compression with k = 1.667:

Wcomp,in=m kRT1k−1P2P1k−1k−10.0493  lbm/s1.6670.4961  Btu/lbm⋅R530  R1.667−1

(b)  Polytropic compression with n = 1.2:

Wcomp,in=m nRT1n−1P2P1n−1n−10.0493  lbm/s1.20.4961  Btu/lbm⋅R530  R1.2−1

(c)  Isothermal compression:

Wcomp,in=mRTlnP2P1=0.0493  lbm/s0.4961  Btu/lbm⋅R530  Rln120  psia14  psia=27.83  Btu/s=39.4 hp

(d) Ideal two-stage compression with intercooling (n = 1.2):  In this case, the pressure ratio across each stage is the same, and its value is determined from

Px=P1P2=14  psia120  psia=41.0  psia

The compressor work across each stage is also the same, thus total compressor work is twice the compression work for a single stage:

Wcomp,in=2mwcomp, I=2m nRT1n−1PxP1n−1n−10.0493  lbm/s1.20.4961  Btu/lbm⋅R530  R1.2−1

8-120E EES Problem 8-119E is reconsidered. The work of compression and entropy change of the helium is to be evaluated and plotted as functions of the polytropic exponent as it varies from 1 to 1.667.

Analysis The problem is solved using EES, and the results are tabulated and plotted below.

Procedure FuncPoly(m_dot,k, R, T1,P2,P1,n:W_dot_comp_polytropic,W_dot_comp_2stagePoly,Q_dot_Out_polytropic,Q_dot_Out_2stagePoly)

If n =1 then

T2=T1

W_dot_comp_polytropic= m_dot*R*(T1+460)*ln(P2/P1)*convert(Btu/s,hp) "[hp]"

W_dot_comp_2stagePoly = W_dot_comp_polytropic "[hp]"

Q_dot_Out_polytropic=W_dot_comp_polytropic*convert(hp,Btu/s) "[Btu/s]"

Q_dot_Out_2stagePoly = Q_dot_Out_polytropic*convert(hp,Btu/s) "[Btu/s]"

Else

C_P = k*R/(k-1) "[Btu/lbm-R]"

T2=(T1+460)*((P2/P1)^((n+1)/n)-460)"[F]"

W_dot_comp_polytropic = m_dot*n*R*(T1+460)/(n-1)*((P2/P1)^((n-1)/n) - 1)*convert(Btu/s,hp)"[hp]"

Q_dot_Out_polytropic=W_dot_comp_polytropic*convert(hp,Btu/s)+m_dot*C_P*(T1-T2)"[Btu/s]"

Px=(P1*P2)^0.5

T2x=(T1+460)*((Px/P1)^((n+1)/n)-460)"[F]"

W_dot_comp_2stagePoly = 2*m_dot*n*R*(T1+460)/(n-1)*((Px/P1)^((n-1)/n) - 1)*convert(Btu/s,hp)"[hp]"

Q_dot_Out_2stagePoly=W_dot_comp_2stagePoly*convert(hp,Btu/s)+2*m_dot*C_P*(T1-T2x)"[Btu/s]"

endif

END

R=0.4961[Btu/lbm-R]

k=1.667

n=1.2

P1=14 [psia]

T1=70 [F]

P2=120 [psia]

V_dot = 5 [ft^3/s]

P1*V_dot=m_dot*R*(T1+460)*convert(Btu,psia-ft^3)

W_dot_comp_isentropic = m_dot*k*R*(T1+460)/(k-1)*((P2/P1)^((k-1)/k) - 1)*convert(Btu/s,hp)"[hp]"

Q_dot_Out_isentropic = 0"[Btu/s]"

Call FuncPoly(m_dot,k, R, T1,P2,P1,n:W_dot_comp_polytropic,W_dot_comp_2stagePoly,Q_dot_Out_polytropic,Q_dot_Out_2stagePoly)

W_dot_comp_isothermal= m_dot*R*(T1+460)*ln(P2/P1)*convert(Btu/s,hp)"[hp]"

Q_dot_Out_isothermal = W_dot_comp_isothermal*convert(hp,Btu/s)"[Btu/s]"

| n | Wcomp2StagePoly [hp] | Wcompisentropic [hp] | Wcompisothermal [hp] | Wcomppolytropic [hp] |
| --- | --- | --- | --- | --- |
| 1 | 39.37 | 62.4 | 39.37 | 39.37 |
| 1.1 | 41.36 | 62.4 | 39.37 | 43.48 |
| 1.2 | 43.12 | 62.4 | 39.37 | 47.35 |
| 1.3 | 44.68 | 62.4 | 39.37 | 50.97 |
| 1.4 | 46.09 | 62.4 | 39.37 | 54.36 |
| 1.5 | 47.35 | 62.4 | 39.37 | 57.54 |
| 1.667 | 49.19 | 62.4 | 39.37 | 62.4 |

8-121 Water mist is to be sprayed into the air stream in the compressor to cool the air as the water evaporates and to reduce the compression power. The reduction in the exit temperature of the compressed air and the compressor power saved are to be determined.

Assumptions 1 Air is an ideal gas with variable specific heats. 2 The process is reversible. 3 Kinetic and potential energy changes are negligible. 3 Air is compressed isentropically. 4 Water vaporizes completely before leaving the compressor.  4 Air properties can be used for the air-vapor mixture.

Properties The gas constant of air is R = 0.287 kJ/kg.K (Table A-1). The specific heat ratio of air is k = 1.4. The inlet enthalpies of water and air are (Tables A-4 and A-17)

hw1 = hf@20C = 83.29 kJ/kg , hfg@20C = 2453.9 kJ/kg and ha1 = h@300 K =300.19 kJ/kg

Analysis  In the case of isentropic operation (thus no cooling or water spray), the exit temperature and the power input to the compressor are

T2T1=P2P1(k−1)k   →  T2=(300 K)1200 kPa100 kPa(1.4−1)1.4=610.2 K

Wcomp,in=m kRT1k−1P2/P1k−1k−1(2.1 kg/s)1.40.287  kJ/kg⋅K300  K1.4−11200 kPa/100 kPa0.4/1.4−1=654.3 kW

He

2

1

W

·

100 kPa

300 K

1200 kPa

Water

20C

When water is sprayed, we first need to check the accuracy of the assumption that the water vaporizes completely in the compressor. In the limiting case, the compression will be isothermal at the compressor inlet temperature, and the water will be a saturated vapor. To avoid the complexity of dealing with two fluid streams and a gas mixture, we disregard water in the air stream (other than the mass flow rate), and assume air is cooled by an amount equal to the enthalpy change of water.

The rate of heat absorption of water as it evaporates at the inlet temperature completely is

Qcooling,max=mwhfg@20°C=(0.2 kg/s)(2453.9  kJ/kg)=490.8 kW

The minimum power input to the compressor is

Wcomp,in,min=mRTlnP2P1=(2.1 kg/s)(0.287  kJ/kg⋅K)(300  K) ln1200  kPa100  kPa=449.3 kW

This corresponds to maximum cooling from the air since, at constant temperature, h = 0 and thus Qout=Win=449.3 kW, which is close to 490.8 kW. Therefore, the assumption that all the water vaporizes is approximately valid. Then the reduction in required power input due to water spray becomes

ΔWcomp,in=Wcomp, isentropic−Wcomp, isothermal=654.3−449.3=205  kW

Discussion (can be ignored): At constant temperature, h = 0 and thus Qout=Win=449.3 kW corresponds to maximum cooling from the air, which is less than 490.8 kW. Therefore, the assumption that all the water vaporizes is only roughly valid. As an alternative, we can assume the compression process to be polytropic and the water to be a saturated vapor at the compressor exit temperature, and disregard the remaining liquid. But in this case there is not a unique solution, and we will have to select either the amount of water or the exit temperature or the polytropic exponent to obtain a solution. Of course we can also tabulate the results for different cases, and then make a selection.

Sample Analysis: We take the compressor exit temperature to be T2 = 200C = 473 K. Then,

hw2 = hg@200C = 2792.0 kJ/kg and ha2 = h@473 K = 475.3 kJ/kg

Then,

T2T1=P2P1(n−1)n   →  473 K300 K=1200 kPa100 kPa(n−1)n  →  n=1.224

Wcomp,in=m nRT1n−1P2/P1n−1n−1=m nRn−1(T2−T1)(2.1 kg/s)1.2240.287  kJ/kg⋅K1.224−1(473−300)K=570 kW

Energy balance:

Wcomp,in−Qout=m(h2−h1)  →Qout=Wcomp,in−m(h2−h1)                                                           =569.7 kW−(2.1 kg/s)(475.3−300.19)=202.0 kW

Noting that this heat is absorbed by water, the rate at which water evaporates in the compressor becomes

Qout,air=Qin,water=mw(hw2−hw1)      {mw=Qin,waterhw2−hw1=202.0 kJ/s(2792.0−83.29) kJ/kg=0.0746 kg/s

Then the reductions in the exit temperature and compressor power input become

Note that selecting a different compressor exit temperature T2 will result in different values.

8-122 A water-injected compressor is used in a gas turbine power plant. It is claimed that the power output of a gas turbine will increase when water is injected into the compressor because of the increase in the mass flow rate of the gas (air + water vapor) through the turbine. This, however, is not necessarily right since the compressed air in this case enters the combustor at a low temperature, and thus it absorbs much more heat. In fact, the cooling effect will most likely dominate and cause the cyclic efficiency to drop.

# Isentropic Efficiencies of Steady-Flow Devices

8-123C  The ideal process for all three devices is the reversible adiabatic (i.e., isentropic) process.  The adiabatic efficiencies of these devices are defined as

ηT=actual  work  outputinsentropic  work  output, ηC=insentropic  work inputactual  work input,  and  η=actual  exit  kinetic energyinsentropic  exit  kinetic  energy

8-124C  No, because the isentropic process is not the model or ideal process for compressors that are cooled intentionally.

8-125C  Yes.  Because the entropy of the fluid must increase during an actual adiabatic process as a result of irreversibilities.  Therefore, the actual exit state has to be on the right-hand side of the isentropic exit state

8-126 Saturated steam is compressed in an adiabatic process with an isentropic efficiency of 0.90. The work required is to be determined.

Assumptions 1 Kinetic and potential energy changes are negligible. 2 The device is adiabatic and thus heat transfer is negligible.

Analysis  We take the steam as the system. This is a closed system since no mass enters or leaves. The energy balance for this stationary closed system can be expressed as

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesWin=ΔU=m(u2−u1)

From the steam tables (Tables A-5 and A-6),

P1=100  kPa  u1=ug @ 100  kPa=2505.6 kJ/kgs1=sg @ 100  kPa=7.3589 kJ/kg⋅KP2=1  MPa  u2s=2903.0 kJ/kg

The work input during the isentropic process is

Ws,in=m(u2s−u1)= (100 kg)(2903.0−2505.6)kJ/kg=39,740 kJ

The actual work input is then

Wa,in=Ws,inηC=39,740 kJ0.90=44,160 kJ

8-127E R-134a is expanded in an adiabatic process with an isentropic efficiency of 0.95. The final volume is to be determined.

Assumptions 1 Kinetic and potential energy changes are negligible. 2 The device is adiabatic and thus heat transfer is negligible.

Analysis  We take the R-134a as the system. This is a closed system since no mass enters or leaves. The energy balance for this stationary closed system can be expressed as

T

s

1

2s

120 psia

20 psia

2a

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energies−Wout=ΔU=m(u2−u1)

From the R-134a tables (Tables A-11E through A-13E),

P1=120 psia  u1=108.48 Btu/lbms1=0.22362 Btu/lbm⋅RP2=20 psia  x2s=s2s−sfsfg=0.22362−0.026050.19962=0.9897u2s=uf+x2sufg=11.401+(0.9897)(82.898)=93.445 Btu/lbm

The actual work input is

wa,out=ηTws,out=ηT(u1−u2s)=(0.95)(108.48−93.448)Btu/lbm=14.28 Btu/lbm

The actual internal energy at the end of the expansion process is

wa,out=(u1−u2a)u2a=u1−wa,out=108.48−14.28=94.20 Btu/lbm

The specific volume at the final state is (Table A-12E)

P2=20 psia  x2=u2a−ufufg=94.20−11.40182.898=0.9988v2=vf+x2vfg=0.01182+(0.9988)(2.2772−0.01182)=2.2745 ft3/lbm

The final volume is then

V2=mv2=(10 lbm)(2.2745 ft3/lbm)=22.75 ft3

8-128 Steam is expanded in an adiabatic turbine with an isentropic efficiency of 0.92. The power output of the turbine is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible.

Analysis There is only one inlet and one exit, and thus m1=m2=m. We take the actual turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Steam turbine

T =92%

P1 = 3 MPa

T1 = 400C

P2 = 30 kPa

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

mh1=Wa,out+mh2     (since  {Q≃Δke≃Δpe≃0)Wa,out=m(h1−h2)

From the steam tables (Tables A-4 through A-6),

P1=3  MPa    h1=3231.7 kJ/kgs1=6.9235 kJ/kg⋅KP2s=30  kPa    x2s=s2s−sfsfg=6.9235−0.94416.8234=0.8763h2s=hf+x2shfg=289.27+(0.8763)(2335.3)=2335.7 kJ/kg

The actual power output may be determined by multiplying the isentropic power output with the isentropic efficiency. Then,

Wa,out=ηTWs,outηTm(h1−h2s)(0.92)(2 kg/s)(3231.7−2335.7)kJ/kg1649 kW

8-129 Steam is expanded in an adiabatic turbine with an isentropic efficiency of 0.90. The power output of the turbine is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible.

Analysis There is only one inlet and one exit, and thus m1=m2=m. We take the actual turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Steam turbine

T =90%

P1 = 3 MPa

T1 = 400C

P2 = 30 kPa

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

mh1=Wa,out+mh2     (since  {Q≃Δke≃Δpe≃0)Wa,out=m(h1−h2)

From the steam tables (Tables A-4 through A-6),

P1=3  MPa    h1=3231.7 kJ/kgs1=6.9235 kJ/kg⋅KP2s=30  kPa    x2s=s2s−sfsfg=6.9235−0.94416.8234=0.8763h2s=hf+x2shfg=289.27+(0.8763)(2335.3)=2335.7 kJ/kg

The actual power output may be determined by multiplying the isentropic power output with the isentropic efficiency. Then,

Wa,out=ηTWs,outηTm(h1−h2s)(0.90)(2 kg/s)(3231.7−2335.7)kJ/kg1613 kW

8-130 Argon gas is compressed by an adiabatic compressor. The isentropic efficiency of the compressor is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible. 4 Argon is an ideal gas with constant specific heats.

Argon compressor

P1 = 200 kPa

T1 = 27C

P2 = 2 MPa

T2 = 550C

Properties The specific heat ratio of argon is k = 1.667 (Table A-2).

Analysis The isentropic exit temperature is

T2s=T1P2P1(k−1)k=(300  K)2000  kPa200  kPa0.667/1.667=753.8 K

From the isentropic efficiency relation,

ηC=wswa=h2s−h1h2a−h1=cp(T2s−T1)cp(T2a−T1)=T2s−T1T2a−T1753.8−300823−300=0.867=86.7%

8-131 R-134a is compressed by an adiabatic compressor. The isentropic efficiency of the compressor is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible.

Analysis From the R-134a tables (Tables A-11 through A-13),

R-134a compressor

0C

sat. vapor

600 kPa

50C

T1=0°C     h1=250.45 kJ/kgs1=0.93139 kJ/kg⋅KP2=600 kPa    h2a=290.28 kJ/kgP2=600 kPa    h2s=265.25 kJ/kg

From the isentropic efficiency relation,

ηC=wswa=h2s−h1h2a−h1=265.25−250.45290.28−250.45=0.372=37.2%

8-132 Steam enters an adiabatic turbine at a specified state, and leaves at a specified state. The mass flow rate of the steam and the isentropic efficiency are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible.

Analysis (a)  From the steam tables (Tables A-4 and A-6),

P1=7 MPa  h1=3650.6  kJ/kgs1=7.0910  kJ/kg⋅KP2=50  kPa  h2a=2780.2  kJ/kg

There is only one inlet and one exit, and thus . We take the actual turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

H2O

1

2

6 MW

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=Wa,out+m(h2+V12/2)   (since   {Q≃Δpe≃0)Wa,out=−mh2−h1+V22−V122

Substituting, the mass flow rate of the steam is determined to be

6000  kJ/s=−m2780.2−3650.6+(140  m/s)2−(80  m/s)221 kJ/kg1000 m2/s2m=6.95   kg/s

(b)  The isentropic exit enthalpy of the steam and the power output of the isentropic turbine are

P2s=50  kPa  x2s=s2s−sfsfg=7.0910−1.09126.5019=0.9228h2s=hf+x2shfg=340.54+0.92282304.7=2467.3  kJ/kg

and

Ws,out=−mh2s−h1+V22−V122Ws,out=−6.95  kg/s2467.3−3650.6+(140  m/s)2−(80  m/s)221 kJ/kg1000 m2/s28174 kW

Then the isentropic efficiency of the turbine becomes

ηT=WaWs=6000  kW8174  kW=0.734=73.4%

8-133 Argon enters an adiabatic turbine at a specified state with a specified mass flow rate, and leaves at a specified pressure. The isentropic efficiency of the turbine is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible. 4 Argon is an ideal gas with constant specific heats.

Properties The specific heat ratio of argon is k = 1.667. The constant pressure specific heat of argon is cp = 0.5203 kJ/kg.K (Table A-2).

Analysis There is only one inlet and one exit, and thus . We take the isentropic turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ar

T

1

2

370 kW

Ein=Eoutmh1=Ws,out+mh2s     (since   {Q≃Δke≃Δpe≃0)Ws,out=m(h1−h2s)

From the isentropic relations,

T2s=T1P2sP1k−1k=1073  K200  kPa1500  kPa0.667/1.667=479  K

Then the power output of the isentropic turbine becomes

Ws,out=mcpT1−T2s=80/60  kg/min0.5203  kJ/kg⋅K1073−479=412.1  kW

Then the isentropic efficiency of the turbine is determined from

ηT=WaWs=370  kW412.1  kW=0.898=89.8%

8-134E Combustion gases enter an adiabatic gas turbine with an isentropic efficiency of 82% at a specified state, and leave at a specified pressure. The work output of the turbine is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible. 4 Combustion gases can be treated as air that is an ideal gas with variable specific heats.

AIR

T  = 82%

1

2

Analysis  From the air table and isentropic relations,

Pr2=P2P1Pr1=60  psia120  psia174.0=87.0  h2s=417.3  Btu/lbm

There is only one inlet and one exit, and thus . We take the actual turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed as

Ein=Eoutmh1=Wa,out+mh2     (since   {Q≃Δke≃Δpe≃0)Wa,out=m(h1−h2)

Noting that wa = Tws, the work output of the turbine per unit mass  is determined from

wa=0.82504.71−417.3 Btu/lbm=71.7  Btu/lbm

8-135 CD EES  Refrigerant-134a enters an adiabatic compressor with an isentropic efficiency of 0.80 at a specified state with a specified volume flow rate, and leaves at a specified pressure. The compressor exit temperature and power input to the compressor are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible.

R-134a

C = 80%

2

1

0.3 m3/min

Analysis (a) From the refrigerant tables (Tables A-11E through A-13E),

P1=120  kPa  h1=hg @ 120  kPa=236.97  kJ/kgs1=sg @ 120  kPa=0.94779  kJ/kg⋅Kv1=vg @ 120  kPa=0.16212  m3/kgP2=1  MPa  h2s=281.21  kJ/kg

From the isentropic efficiency relation,

Thus,	ηC=h2s−h1h2a−h1  h2a=h1+h2s−h1ηC=236.97+281.21−236.97/0.80=292.26  kJ/kgP2a=1  MPa T2a=58.9°C

(b)  The mass flow rate of the refrigerant is determined from

m=V1v1=0.3/60  m3/s0.16212  m3/kg=0.0308  kg/s

There is only one inlet and one exit, and thus . We take the actual compressor as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

Wa,in+mh1=mh2     (since   {Q≃Δke≃Δpe≃0)Wa,in=m(h2−h1)

Substituting, the power input to the compressor becomes,

Wa,in=0.0308  kg/s292.26−236.97 kJ/kg=1.70  kW

8-136 EES Problem 8-135 is reconsidered. The problem is to be solved by considering the kinetic energy and by assuming an inlet-to-exit area ratio of 1.5 for the compressor when the compressor exit pipe inside diameter is 2 cm.

Analysis The problem is solved using EES, and the solution is given below.

"Input Data from diagram window"

{P[1] = 120 "kPa"

P[2] = 1000 "kPa"

Vol_dot_1 = 0.3 "m^3/min"

Eta_c = 0.80 "Compressor adiabatic efficiency"

A_ratio = 1.5

d_2 = 2/100 "m"}

"System: Control volume containing the compressor, see the diagram window.

Property Relation:  Use the real fluid properties for R134a.

Process: Steady-state, steady-flow, adiabatic process."

Fluid$='R134a'

"Property Data for state 1"

T[1]=temperature(Fluid$,P=P[1],x=1)"Real fluid equ. at the sat. vapor state"

h[1]=enthalpy(Fluid$, P=P[1], x=1)"Real fluid equ. at the sat. vapor state"

s[1]=entropy(Fluid$, P=P[1], x=1)"Real fluid equ. at the sat. vapor state"

v[1]=volume(Fluid$, P=P[1], x=1)"Real fluid equ. at the sat. vapor state"

"Property Data for state 2"

s_s[1]=s[1]; T_s[1]=T[1] "needed for plot"

s_s[2]=s[1] "for the ideal, isentropic process across the compressor"

h_s[2]=ENTHALPY(Fluid$, P=P[2], s=s_s[2])"Enthalpy 2 at the isentropic state 2s and pressure P[2]"

T_s[2]=Temperature(Fluid$, P=P[2], s=s_s[2])"Temperature of ideal state - needed only for plot."

"Steady-state, steady-flow conservation of mass"

m_dot_1 = m_dot_2

m_dot_1 = Vol_dot_1/(v[1]*60)

Vol_dot_1/v[1]=Vol_dot_2/v[2]

Vel[2]=Vol_dot_2/(A[2]*60)

A[2] = pi*(d_2)^2/4

A_ratio*Vel[1]/v[1] = Vel[2]/v[2] "Mass flow rate: = A*Vel/v, A_ratio = A[1]/A[2]"

A_ratio=A[1]/A[2]

"Steady-state, steady-flow conservation of energy, adiabatic compressor, see diagram window"

m_dot_1*(h[1]+(Vel[1])^2/(2*1000)) + W_dot_c= m_dot_2*(h[2]+(Vel[2])^2/(2*1000))

"Definition of the compressor adiabatic efficiency, Eta_c=W_isen/W_act"

Eta_c = (h_s[2]-h[1])/(h[2]-h[1])

"Knowing h[2], the other properties at state 2 can be found."

v[2]=volume(Fluid$, P=P[2], h=h[2])"v[2] is found at the actual state 2, knowing P and h."

T[2]=temperature(Fluid$, P=P[2],h=h[2])"Real fluid equ. for T at the known outlet h and P."

s[2]=entropy(Fluid$, P=P[2], h=h[2])   "Real fluid equ. at the known outlet h and P."

T_exit=T[2]

"Neglecting the kinetic energies, the work is:"

m_dot_1*h[1] + W_dot_c_noke= m_dot_2*h[2]

SOLUTION

| A[1]=0.0004712 [m^2] A[2]=0.0003142 [m^2] A_ratio=1.5 d_2=0.02 [m] Eta_c=0.8 Fluid$='R134a' h[1]=237 [kJ/kg] h[2]=292.3 [kJ/kg] h_s[2]=281.2 [kJ/kg] m_dot_1=0.03084 [kg/s] m_dot_2=0.03084 [kg/s] P[1]=120.0 [kPa] P[2]=1000.0 [kPa] s[1]=0.9478 [kJ/kg-K] s[2]=0.9816 [kJ/kg-K] | s_s[1]=0.9478 [kJ/kg-K] s_s[2]=0.9478 [kJ/kg-K] T[1]=-22.32 [C] T[2]=58.94 [C] T_exit=58.94 [C] T_s[1]=-22.32 [C] T_s[2]=48.58 [C] Vol_dot_1=0.3 [m^3 /min] Vol_dot_2=0.04244 [m^3 /min] v[1]=0.1621 [m^3/kg] v[2]=0.02294 [m^3/kg] Vel[1]=10.61 [m/s] Vel[2]=2.252 [m/s] W_dot_c=1.704 [kW] W_dot_c_noke=1.706 [kW] |
| --- | --- |

8-137 Air enters an adiabatic compressor with an isentropic efficiency of 84% at a specified state, and leaves at a specified temperature. The exit pressure of air and the power input to the compressor are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible. 4 Air is an ideal gas with variable specific heats.

AIR

C = 84%

2

1

2.4 m3/s

Properties The gas constant of air is R = 0.287 kPa.m3/kg.K (Table A-1)

Analysis  (a) From the air table (Table A-21),

T1=290  K  h1=290.16  kJ/kg, Pr1=1.2311T2=530  K  h2a=533.98  kJ/kg

From the isentropic efficiency relation  ηC=h2s−h1h2a−h1,

h2s=h1+ηCh2a−h1=290.16+0.84533.98−290.16=495.0  kJ/kg  Pr2=7.951

Then from the isentropic relation ,

P2P1=Pr2Pr1  P2=Pr2Pr1P1=7.9511.2311100 kPa=646 kPa

(b) There is only one inlet and one exit, and thus . We take the actual compressor as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

Wa,in+mh1=mh2     (since   {Q≃Δke≃Δpe≃0)Wa,in=m(h2−h1)

where

m=P1V1RT1=(100  kPa)(2.4  m3/s)(0.287  kPa⋅m3/kg⋅K)(290  K)=2.884  kg/s

Then the power input to the compressor is determined to be

Wa,in=(2.884  kg/s)(533.98−290.16) kJ/kg=703 kW

8-138 Air is compressed by an adiabatic compressor from a specified state to another specified state. The isentropic efficiency of the compressor and the exit temperature of air for the isentropic case are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible. 4 Air is an ideal gas with variable specific heats.

AIR

2

1

Analysis  (a) From the air table (Table A-21),

From the isentropic relation,

Pr2=P2P1Pr1=600  kPa95  kPa1.386=8.754  h2s=508.72  kJ/kg

Then the isentropic efficiency becomes

(b)  If the process were isentropic, the exit temperature would be

8-139E Argon enters an adiabatic compressor with an isentropic efficiency of 80% at a specified state, and leaves at a specified pressure. The exit temperature of argon and the work input to the compressor are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible. 4 Argon is an ideal gas with constant specific heats.

Ar

C = 80%

2

1

Properties The specific heat ratio of argon is             k = 1.667. The constant pressure specific heat of argon is cp = 0.1253 Btu/lbm.R (Table A-2E).

Analysis (a) The isentropic exit temperature T2s is determined from

T2s=T1P2sP1k−1k=550  R200  psia20  psia0.667/1.667=1381.9  R

The actual kinetic energy change during this process is

Δkea=V22−V122=240  ft/s2−60  ft/s221  Btu/lbm25,037  ft2/s2=1.08  Btu/lbm

The effect of kinetic energy on isentropic efficiency is very small. Therefore, we can take the kinetic energy changes for the actual and isentropic cases to be same in efficiency calculations. From the isentropic efficiency relation, including the effect of kinetic energy,

ηC=wswa=(h2s−h1)+Δke(h2a−h1)+Δke=cpT2s−T1+ΔkescpT2a−T1+Δkea  0.8=0.12531381.9−550+1.080.1253T2a−550+1.08

It yields

T2a  =  1592 R

(b) There is only one inlet and one exit, and thus . We take the actual compressor as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

Wa,in+m(h1+V122)=m(h2+V22/2)   (since   {Q≃Δpe≃0)Wa,in=mh2−h1+V22−V122      wa,in=h2−h1+Δke

Substituting, the work input to the compressor is determined to be

wa,in=0.1253  Btu/lbm⋅R1592−550R+1.08  Btu/lbm=131.6 Btu/lbm

8-140E Air is accelerated in a 90% efficient adiabatic nozzle from low velocity to a specified velocity. The exit temperature and pressure of the air are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible. 4 Air is an ideal gas with variable specific heats.

Analysis From the air table (Table A-21E),

There is only one inlet and one exit, and thus . We take the nozzle as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed as

AIR

N = 90%

1

2

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=m(h2+V22/2)   (since   {W=Q≃Δpe≃0)h2=h1−V22−V12Ã02

Substituting, the exit temperature of air is determined to be

h2=363.89  kJ/kg−800  ft/s2−021  Btu/lbm25,037  ft2/s2=351.11  Btu/lbm

From the air table we read

T2a  =  1431.3 R

From the isentropic efficiency relation

ηN=h2a−h1h2s−h1

h2s=h1+h2a−h1ηN=363.89+351.11−363.890.90=349.69  Btu/lbm  Pr2=46.04

Then the exit pressure is determined from the isentropic relation to be

P2P1=Pr2Pr1  P2=Pr2Pr1P1=46.0453.0460  psia=52.1   psia

8-141E EES Problem 8-140E is reconsidered. The effect of varying the nozzle isentropic efficiency from 0.8 to 1.0 on the exit temperature and pressure of the air is to be investigated, and the results are to be plotted.

Analysis The problem is solved using EES, and the results are tabulated and plotted below.

"Knowns:"

WorkFluid$ = 'Air'

P[1] = 60  [psia]

T[1] = 1020 [F]

Vel[2] = 800 [ft/s]

Vel[1] = 0 [ft/s]

eta_nozzle = 0.9

"Conservation of Energy - SSSF energy balance for turbine -- neglecting the change in potential energy, no  heat transfer:"

h[1]=enthalpy(WorkFluid$,T=T[1])

s[1]=entropy(WorkFluid$,P=P[1],T=T[1])

T_s[1] = T[1]

s[2] =s[1]

s_s[2] = s[1]

h_s[2]=enthalpy(WorkFluid$,T=T_s[2])

T_s[2]=temperature(WorkFluid$,P=P[2],s=s_s[2])

eta_nozzle = ke[2]/ke_s[2]

ke[1] = Vel[1]^2/2

ke[2]=Vel[2]^2/2

h[1]+ke[1]*convert(ft^2/s^2,Btu/lbm) = h[2] + ke[2]*convert(ft^2/s^2,Btu/lbm)

h[1] +ke[1]*convert(ft^2/s^2,Btu/lbm) = h_s[2] + ke_s[2]*convert(ft^2/s^2,Btu/lbm)

T[2]=temperature(WorkFluid$,h=h[2])

P_2_answer = P[2]

T_2_answer = T[2]

| nozzle | P2 [psia | T2 [F | Ts,2 [F] |
| --- | --- | --- | --- |
| 0.8 | 51.09 | 971.4 | 959.2 |
| 0.85 | 51.58 | 971.4 | 962.8 |
| 0.9 | 52.03 | 971.4 | 966 |
| 0.95 | 52.42 | 971.4 | 968.8 |
| 1 | 52.79 | 971.4 | 971.4 |

8-142 Air is expanded in an adiabatic nozzle with an isentropic efficiency of 0.96. The air velocity at the exit is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 There is no heat transfer or shaft work associated with the process. 3 Air is an ideal gas with constant specific heats.

Properties The properties of air at room temperature are cp = 1.005 kJ/kg·K and k = 1.4 (Table A-2a).

Analysis For the isentropic process of an ideal gas, the exit temperature is determined from

T2s=T1P2P1(k−1)k=(180+273 K)100 kPa300 kPa0.41.4=331.0 K

There is only one inlet and one exit, and thus m1=m2=m. We take nozzle as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Air

300 kPa

180C

0 m/s

100 kPa

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

mh1+V122=mh2+V222h1+V122=h2+V222h1−h2=V22−V122cp(T1−T2)=V22−V122=Δke

The kinetic energy change for the isentropic case is

Δkes=cp(T1−T2s)=(1.005 kJ/kg⋅K)(453−331)K=122.6 kJ/kg

The kinetic energy change for the actual process is

Δkea=ηNΔkes=(0.96)(122.6 kJ/kg)=117.7 kJ/kg

Substituting into the energy balance and solving for the exit velocity gives

V2=(2Δkea)0.5=2(117.7 kJ/kg)1000 m2/s21 kJ/kg0.5=485 m/s

8-143E Air is decelerated in an adiabatic diffuser with an isentropic efficiency of 0.82. The air velocity at the exit is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 There is no heat transfer or shaft work associated with the process. 3 Air is an ideal gas with constant specific heats.

Properties The properties of air at room temperature are cp = 0.240 Btu/lbm·R and k = 1.4 (Table A-2Ea).

Analysis For the isentropic process of an ideal gas, the exit temperature is determined from

T2s=T1P2P1(k−1)k=(30+460 R)20 psia13 psia0.41.4=554.2 R

There is only one inlet and one exit, and thus m1=m2=m. We take nozzle as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Air

13 psia

30F

1000 ft/s

20 psia

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

mh1+V122=mh2+V222h1+V122=h2+V222h1−h2=V22−V122cp(T1−T2)=V22−V122=Δke

The kinetic energy change for the isentropic case is

Δkes=cp(T2s−T1)=(0.240 Btu/lbm⋅R)(554.2−490)R=15.41 Btu/lbm

The kinetic energy change for the actual process is

Δkea=ηNΔkes=(0.82)(15.41 Btu/lbm)=12.63 Btu/lbm

Substituting into the energy balance and solving for the exit velocity gives

V2=(V12−2Δkea)0.5=(1000 ft/s)2−2(12.63 Btu/lbm)25,037 ft2/s21 Btu/lbm0.5=606 ft/s

## Entropy Balance

8-144 Heat is lost from Refrigerant-134a as it is throttled. The exit temperature of the refrigerant and the entropy generation are to be determined.

Assumptions 1 Steady operating conditions exist. 2 Kinetic and potential energy changes are negligible.

Analysis The properties of the refrigerant at the inlet of the device are (Table A-13)

R-134a

900 kPa

35C

200 kPa

q

P1=900  kPa h1=100.86 kJ/kgs1=0.37135 kJ/kg

The enthalpy of the refrigerant at the exit of the device is

h2=h1−qout=100.86−0.8=100.06 kJ/kg

Now, the properties at the exit state may be obtained from the R-134a tables

P2=200  kPa T2=−10.09°Cs2=0.38884 kJ/kg.K

The entropy generation associated with this process may be obtained by adding the entropy change of R-134a as it flows in the device and the entropy change of the surroundings.

ΔsR-134a=s2−s1=0.38884−0.37135=0.01749 kJ/kg.K

Δssurr=qoutTsurr=0.8 kJ/kg(25+273) K=0.002685 kJ/kg.K

sgen=Δstotal=ΔsR−134a+Δssurr=0.01749+0.002685=0.0202 kJ/kg.K

8-145 Liquid water is withdrawn from a rigid tank that initially contains saturated water mixture until no liquid is left in the tank. The quality of steam in the tank at the initial state, the amount of mass that has escaped, and the entropy generation during this process are to be determined.

Assumptions 1 Kinetic and potential energy changes are zero. 2 There are no work interactions.

Analysis (a) The properties of the steam in the tank at the final state and the properties of exiting steam are (Tables A-4 through A-6)

Water mixture

7.5 kg

400 kPa

Q

P2=400 kPa u2=2553.1 kJ/kgv2=0.46242 m3/kgs2=6.8955 kJ/kg.KP2e=400 kPa he=604.66 kJ/kgse=1.7765 kJ/kg.K

The relations for the volume of the tank and the final mass in the tank are

V=m1v1=(7.5 kg)v1m2=Vv2=(7.5 kg)v10.46242 m3/kg=16.219v1

The mass, energy, and entropy balances may be written as

me=m1−m2

Qin−mehe=m2u2−m1u1

QinTsource−mese+Sgen=m2s2−m1s1

Substituting,

me=7.5−16.219v1							(1)

5−(7.5−16.219v1)(604.66)=16.219v1(2553.1)−7.5u1			(2)

5500+273−(7.5−16.219v1)(1.7765)+Sgen=16.219v1(6.8955)−7.5s1		(3)

Eq. (2) may be solved by a trial-error approach by trying different qualities at the inlet state. Or, we can use EES to solve the equations to find

x1 = 0.8666

Other properties at the initial state are

P1=400 kPa u1=2293.2 kJ/kgv1=0.40089 m3/kgs1=6.2129 kJ/kg.K

Substituting into Eqs (1) and (3),

(b)	me=7.5−16.219(0.40089)=0.998 kg

(c)

5500+273−7.5−16.219(0.40089)(1.7765)+Sgen=16.219(0.40089)(6.8955)−7.5(6.2129)Sgen=0.00553 kJ/K

8-146 Each member of a family of four take a 5-min shower every day. The amount of entropy generated by this family per year is to be determined.

Assumptions 1 Steady operating conditions exist. 2 The kinetic and potential energies are negligible.  3 Heat losses from the pipes and the mixing section are negligible and thus  4 Showers operate at maximum flow conditions during the entire shower. 5 Each member of the household takes a 5-min shower every day. 6 Water is an incompressible substance with constant properties at room temperature. 7 The efficiency of the electric water heater is 100%.

Properties  The density and specific heat of water at room temperature are  = 1 kg/L and c = 4.18 kJ/kg.C (Table A-3).

Cold

water

1

Hot

water

2

Mixture

3

Analysis The mass flow rate of water at the shower head is

m=ρV=(1 kg/L)(12 L/min)=12 kg/min

The mass balance for the mixing chamber can be expressed in the rate form as

min−mout=Δmsystemä0  (steady)=0   min=moutm1+m2=m3

where the subscript 1 denotes the cold water stream, 2 the hot water stream, and 3 the mixture.

The rate of entropy generation during this process can be determined by applying the rate form of the entropy balance on a system that includes the electric water heater and the mixing chamber (the T-elbow). Noting that there is no entropy transfer associated with work transfer (electricity) and there is no heat transfer,   the entropy balance for this steady-flow system can be expressed as

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0(steady)underbracealignlRate of change   of entropym1s1+m2s2−m3s3+Sgen=0    (since  Q=0 and work is entropy free)Sgen=m3s3−m1s1−m2s2

Noting from mass balance that  and  s2 = s1 since hot water enters the system at the same temperature as the cold water, the rate of entropy generation is determined to be

Sgen=m3s3−(m1+m2)s1=m3(s3−s1)=m3cplnT3T1(12 kg/min)(4.18 kJ/kg.K)ln42+27315+273=4.495 kJ/min.K

Noting that 4 people take a 5-min shower every day, the amount of entropy generated per year is

Sgen=(Sgen)Δt(No. of people)(No. of days)(4.495 kJ/min.K)(5 min/person⋅day)(4 persons)(365 days/year)32,814 kJ/K  (per year)

# Discussion The value above represents the entropy generated within the water heater and the T-elbow in the absence of any heat losses. It does not include the entropy generated as the shower water at 42C is discarded  or cooled to the outdoor temperature. Also, an entropy balance on the mixing chamber alone (hot water entering at 55C instead of 15C) will exclude the entropy generated within the water heater.

8-147  Steam is condensed by cooling water in the condenser of a power plant. The rate of condensation of steam and the rate of entropy generation are to be determined.

Assumptions 1 Steady operating conditions exist. 2 The heat exchanger is well-insulated so that heat loss to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 3 Changes in the kinetic and potential energies of fluid streams are negligible. 4 Fluid properties are constant.

Properties The enthalpy and entropy of vaporization of water at 60C are hfg =2357.7 kJ/kg and sfg = 7.0769 kJ/kg.K  (Table A-4). The specific heat of water at room temperature is  cp = 4.18 kJ/kg.C (Table A-3).

Steam

60C

18C

Water

60C

27C

Analysis (a) We take the cold water tubes as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since  Δke≃Δpe≃0)Qin=mcp(T2−T1)

Then  the heat transfer rate to the cooling water in the condenser becomes

Q=[mcp(Tout−Tin)]cooling water(75 kg/s)(4.18 kJ/kg.°C)(27°C−18°C)=2822 kJ/s

The rate of condensation of steam is determined to be

Q=(mhfg)steammsteam=Qhfg=2822 kJ/s2357.7 kJ/kg=1.20  kg/s

(b) The rate of entropy generation within the condenser during this process can be determined by applying the rate form of the entropy balance on the entire condenser. Noting that the condenser is well-insulated and thus heat transfer is negligible, the entropy balance for this steady-flow system can be expressed as

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0(steady)underbracealignlRate of change   of entropym1s1+m3s3−m2s2−m4s4+Sgen=0   (since Q=0)mwaters1+msteams3−mwaters2−msteams4+Sgen=0Sgen=mwater(s2−s1)+msteam(s4−s3)

Noting that water is an incompressible substance and steam changes from saturated vapor to saturated liquid, the rate of entropy generation is determined to be

Sgen=mwatercplnT2T1+msteam(sf−sg)=mwatercplnT2T1−msteamsfg(75 kg/s)(4.18 kJ/kg.K)ln27+27318+273−(1.20 kg/s)(7.0769 kJ/kg.K)1.06 kW/K

8-148 Cold water is heated by hot water in a heat exchanger. The rate of heat transfer and the rate of entropy generation within the heat exchanger are to be determined.

Assumptions 1 Steady operating conditions exist. 2 The heat exchanger is well-insulated so that heat loss to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 3 Changes in the kinetic and potential energies of fluid streams are negligible.  4 Fluid properties are constant.

Properties The specific heats of cold and hot water are given to be 4.18 and 4.19 kJ/kg.C, respectively.

Hot water

100C

3 kg/s

Cold water

15C

0.25 kg/s

45C

Analysis We take the cold water tubes as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since  Δke≃Δpe≃0)Qin=mcp(T2−T1)

Then the rate of heat transfer to the cold water in this heat exchanger becomes

Qin=[mcp(Tout−Tin)]cold water=(0.25 kg/s)(4.18 kJ/kg.°C)(45°C−15°C)=31.35 kW

Noting that heat gain by the cold water is equal to the heat loss by the hot water, the outlet temperature of the hot water is determined to be

Q=[mcp(Tin−Tout)]hot waterTout=Tin−Qmcp=100°C−31.35 kW(3 kg/s)(4.19 kJ/kg.°C)=97.5°C

(b) The rate of entropy generation within the heat exchanger is determined by applying the rate form of the entropy balance on the entire heat exchanger:

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0(steady)underbracealignlRate of change   of entropym1s1+m3s3−m2s2−m3s4+Sgen=0   (since Q=0)mcolds1+mhots3−mcolds2−mhots4+Sgen=0Sgen=mcold(s2−s1)+mhot(s4−s3)

Noting that both fluid streams are liquids (incompressible substances), the rate of entropy generation is determined to be

Sgen=mcoldcplnT2T1+mhotcplnT4T3(0.25 kg/s)(4.18 kJ/kg.K)ln45+27315+273+(3 kg/s)(4.19 kJ/kg.K)ln97.5+273100+2730.0190 kWK

8-149 Air is preheated by hot exhaust gases in a cross-flow heat exchanger. The rate of heat transfer, the outlet temperature of the air, and the rate of entropy generation are to be determined.

Assumptions 1 Steady operating conditions exist. 2 The heat exchanger is well-insulated so that heat loss to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 3 Changes in the kinetic and potential energies of fluid streams are negligible.  4 Fluid properties are constant.

Properties The specific heats of air and combustion gases are given to be 1.005 and 1.10 kJ/kg.C, respectively. The gas constant of air is R = 0.287 kJ/kg.K (Table A-1).

Air

95 kPa

20C

1.6 m3/s

Exhaust gases

2.2 kg/s,  95C

Analysis We take the exhaust pipes as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutmh1=Qout+mh2   (since Δke≃Δpe≃0)Qout=mcp(T1−T2)

Then the rate of heat transfer from the exhaust gases becomes

Q=[mcp(Tin−Tout)]gas.=(2.2 kg/s)(1.1 kJ/kg.°C)(180°C−95°C)=205.7  kW

The mass flow rate of air is

m=PVRT=(95 kPa)(1.6 m3/s)(0.287 kPa.m3/kg.K)×(293 K)=1.808 kg/s

Noting that heat loss by the exhaust gases is equal to the heat gain by the air, the outlet temperature of the air becomes

Q=mcp(Tout−Tin)airTout=Tin+Qmcp=20°C+205.7 kW(1.808 kg/s)(1.005 kJ/kg.°C)=133.2°C

The rate of entropy generation within the heat exchanger is determined by applying the rate form of the entropy balance on the entire heat exchanger:

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0(steady)underbracealignlRate of change   of entropym1s1+m3s3−m2s2−m3s4+Sgen=0   (since Q=0)mexhausts1+mairs3−mexhausts2−mairs4+Sgen=0Sgen=mexhaust(s2−s1)+mair(s4−s3)

Then the rate of entropy generation is determined to be

Sgen=mexhaustcplnT2T1+maircplnT4T3(2.2 kg/s)(1.1 kJ/kg.K)ln95+273180+273+(1.808 kg/s)(1.005 kJ/kg.K)ln133.2+27320+2730.091 kWK

8-150 Water is heated by hot oil in a heat exchanger. The outlet temperature of the oil and the rate of entropy generation within the heat exchanger are to be determined.

Assumptions 1 Steady operating conditions exist. 2 The heat exchanger is well-insulated so that heat loss to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 3 Changes in the kinetic and potential energies of fluid streams are negligible.  4 Fluid properties are constant.

Oil

170C

10 kg/s

Water

20C

4.5 kg/s

70C

Properties The specific heats of  water and oil are given to be 4.18 and 2.3 kJ/kg.C, respectively.

Analysis (a) We take the cold water tubes as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since  Δke≃Δpe≃0)Qin=mcp(T2−T1)

Then the rate of heat transfer to the cold water in this heat exchanger becomes

Q=[mcp(Tout−Tin)]water=(4.5 kg/s)(4.18 kJ/kg.°C)(70°C−20°C)=940.5kW

Noting that heat gain by the water is equal to the heat loss by the oil, the outlet temperature of the hot oil is determined from

Q=[mcp(Tin−Tout)]oil  →  Tout=Tin−Qmcp=170°C−940.5 kW(10 kg/s)(2.3 kJ/kg.°C)=129.1°C

(b) The rate of entropy generation within the heat exchanger is determined by applying the rate form of the entropy balance on the entire heat exchanger:

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0(steady)underbracealignlRate of change   of entropym1s1+m3s3−m2s2−m3s4+Sgen=0   (since Q=0)mwaters1+moils3−mwaters2−moils4+Sgen=0Sgen=mwater(s2−s1)+moil(s4−s3)

Noting that both fluid streams are liquids (incompressible substances), the rate of entropy generation is determined to be

Sgen=mwatercplnT2T1+moilcplnT4T3(4.5 kg/s)(4.18 kJ/kg.K)ln70+27320+273+(10 kg/s)(2.3 kJ/kg.K)ln129.1+273170+2730.736kWK

8-151E Refrigerant-134a is expanded adiabatically from a specified state to another. The entropy generation is to be determined.

Assumptions 1 Steady operating conditions exist. 2 Kinetic and potential energy changes are negligible.

Analysis The rate of entropy generation within the expansion device during this process can be determined by applying the rate form of the entropy balance on the system. Noting that the system is adiabatic and thus there is no heat transfer, the entropy balance for this steady-flow system can be expressed as

R-134a

100 psia

100F

10 psia

sat. vapor

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0(steady)underbracealignlRate of change   of entropym1s1−m2s2+Sgen=0Sgen=m(s2−s1)sgen=s2−s1

The properties of the refrigerant at the inlet and exit states are (Tables A-11E through A-13E)

P1=100 psia   s1=0.22900 Btu/lbm⋅R

P2=10 psia   s2=0.22948 Btu/lbm⋅R

Substituting,

sgen=s2−s1=0.22948−0.22900=0.00048 Btu/lbm⋅R

8-152 In an ice-making plant, water is frozen by evaporating saturated R-134a liquid. The rate of entropy generation is to be determined.

Assumptions 1 Steady operating conditions exist. 2 Kinetic and potential energy changes are negligible.

Analysis We take the control volume formed by the R-134a evaporator with a single inlet and single exit as the system. The rate of entropy generation within this evaporator during this process can be determined by applying the rate form of the entropy balance on the system. The entropy balance for this steady-flow system can be expressed as

R-134a

10C

10C

sat. vapor

Q

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0(steady)underbracealignlRate of change   of entropym1s1−m2s2+QinTw+Sgen=0Sgen=mR(s2−s1)−QinTwSgen=mRsfg−QinTw

The properties of the refrigerant are (Table A-11)

hfg @ −10°C=205.96 kJ/kgsfg @ −10°C=0.78263 kJ/kg⋅K

The rate of that must be removed from the water in order to freeze it at a rate of 4000 kg/h is

Qin=mwhif=(40003600 kg/s)(333.7 kJ/kg)=370.8 kW

where the heat of fusion of water at 1 atm is 333.7 kJ/kg. The mass flow rate of R-134a is

mR=Qinhfg=370.8 kJ/s205.96 kJ/kg=1.800 kg/s

Substituting,

Sgen=mRsfg−QinTw=(1.800 kg/s)(0.78263 kJ/kg⋅K)−370.8 kW273 K=0.0505 kW/K

8-153 Air is heated by steam in a heat exchanger. The rate of entropy generation associated with this process is to be determined.

Air

20C

Steam

35C

sat. vap.

10,000 kg/h

32C

30C

Assumptions 1 Steady operating conditions exist. 2 The heat exchanger is well-insulated so that heat loss to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 3 Changes in the kinetic and potential energies of fluid streams are negligible.  4 Air is an ideal gas with constant specific heats.

Properties The specific heat of air at room temperature is cp = 1.005 kJ/kgC (Table A-2).

Analysis  The rate of entropy generation within the heat exchanger is determined by applying the rate form of the entropy balance on the entire heat exchanger:

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0(steady)underbracealignlRate of change   of entropymwaters1+mairs3−mwaters2−mairs4+Sgen=0Sgen=mwater(s2−s1)+mair(s4−s3)

The properties of the steam at the inlet and exit states are

T1=35°C    h1=2564.6 kJ/kgs1=8.3517 kJ/kg⋅K		(Table A-4)

T2=32°C    h2=134.10 kJ/kgs2=0.4641 kJ/kg⋅K	(Table A-4)

From an energy balance, the heat given up by steam is equal to the heat picked up by the air. Then,

Q=mwater(h1−h2)=(10,0003600 kg/s)(2564.6−134.10) kJ/kg=6751 kW

mair=Qcp(T4−T3)=6751 kW(1.005 kJ/kg⋅°C)(30−20)°C=671.7 kg/s

Substituting into the entropy balance relation,

Sgen=mwater(s2−s1)+mair(s4−s3)mwater(s2−s1)+maircplnT4T3(10,0003600 kg/s)(0.4641-8.3517) kJ/kg⋅K+(671.7 kg/s)(1.005 kJ/kg⋅K)ln303 K293 K0.745 kW/K

Note that the pressure of air remains unchanged as it flows in the heat exchanger. This is why the pressure term is not included in the entropy change expression of air.

8-154 Oxygen is cooled as it flows in an insulated pipe. The rate of entropy generation in the pipe is to be determined.

Assumptions 1 Steady operating conditions exist. 2 The pipe is well-insulated so that heat loss to the surroundings is negligible. 3 Changes in the kinetic and potential energies are negligible.  4 Oxygen is an ideal gas with constant specific heats.

Properties The properties of oxygen at room temperature are R = 0.2598 kJ/kgK,  cp = 0.918 kJ/kgK (Table A-2a).

Analysis  The rate of entropy generation in the pipe is determined by applying the rate form of the entropy balance on the pipe:

Oxygen

240 kPa

20C

70 m/s

200 kPa

18C

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0(steady)underbracealignlRate of change   of entropym1s1−m2s2+Sgen=0   (since Q=0)Sgen=m(s2−s1)

The specific volume of oxygen at the inlet and the mass flow rate are

v1=RT1P1=(0.2598 kPa⋅m3/kg⋅K)(293 K)240 kPa=0.3172 m3/kg

m=A1V1v1=πD2V14v1=π(0.12 m)2(70 m/s)4(0.3172 m3/kg)=2.496 kg/s

Substituting into the entropy balance relation,

Sgen=m(s2−s1)mcplnT2T1−RlnP2P1(2.496 kg/s)(0.918 kJ/kg⋅K)ln291 K293 K−(0.2598 kJ/kg⋅K)ln200 kPa240 kPa0.1025 kW/K

8-155 Nitrogen is compressed by an adiabatic compressor. The entropy generation for this process is to be determined.

Assumptions 1 Steady operating conditions exist. 2 The compressor is well-insulated so that heat loss to the surroundings is negligible. 3 Changes in the kinetic and potential energies are negligible.  4 Nitrogen is an ideal gas with constant specific heats.

Properties The specific heat of nitrogen at the average temperature of (17+227)/2=122C = 395 K is cp = 1.044 kJ/kgK (Table A-2b).

Analysis  The rate of entropy generation in the pipe is determined by applying the rate form of the entropy balance on the compressor:

Compressor

Air

100 kPa

17C

600 kPa

227C

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0(steady)underbracealignlRate of change   of entropym1s1−m2s2+Sgen=0   (since Q=0)Sgen=m(s2−s1)

Substituting per unit mass of the oxygen,

sgen=s2−s1cplnT2T1−RlnP2P1(1.044 kJ/kg⋅K)ln(227+273) K(17+273) K−(0.2968 kJ/kg⋅K)ln600 kPa100 kPa0.0369 kJ/kg⋅K

8-156E Steam is condensed by cooling water in a condenser. The rate of heat transfer and the rate of entropy generation within the heat exchanger are to be determined.

Assumptions 1 Steady operating conditions exist. 2 The heat exchanger is well-insulated so that heat loss to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 3 Changes in the kinetic and potential energies of fluid streams are negligible. 4 Fluid properties are constant.

Steam

120F

60F

Water

73F

120F

Properties The specific heat of water is 1.0 Btu/lbm.F (Table A-3E). The enthalpy and entropy of vaporization of water at 120F are 1025.2 Btu/lbm and sfg = 1.7686 Btu/lbm.R  (Table A-4E).

Analysis We take the tube-side of the heat exchanger where cold water is flowing as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since  Δke≃Δpe≃0)Qin=mcp(T2−T1)

Then the rate of heat transfer to the cold water in this heat exchanger becomes

Q=[mcp(Tout−Tin)]water=(92 lbm/s)(1.0  Btu/lbm.°F)(73°F−60°F)=1196  Btu/s

Noting that heat gain by the water is equal to the heat loss by the condensing steam, the rate of condensation of the steam in the heat exchanger is determined from

Q=(mhfg)steam=msteam=Qhfg=1196 Btu/s1025.2 Btu/lbm=1.167  lbm/s

(b) The rate of entropy generation within the heat exchanger is determined by applying the rate form of the entropy balance on the entire heat exchanger:

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0(steady)underbracealignlRate of change   of entropym1s1+m3s3−m2s2−m4s4+Sgen=0   (since Q=0)mwaters1+msteams3−mwaters2−msteams4+Sgen=0Sgen=mwater(s2−s1)+msteam(s4−s3)

Noting that water is an incompressible substance and steam changes from saturated vapor to saturated liquid, the rate of entropy generation is determined to be

Sgen=mwatercplnT2T1+msteam(sf−sg)=mwatercplnT2T1−msteamsfg(92 lbm/s)(1.0 Btu/lbm.R) ln73+46060+460−(1.167 lbm/s)(17686 Btu/lbm.R)0.209 Btu/s.R

8-157 A regenerator is considered to save heat during the cooling of milk in a dairy plant. The amounts of fuel and money such a generator will save per year and the annual reduction in the rate of entropy generation are to be determined.

Hot milk

72C

4C

Cold milk

Assumptions 1 Steady operating conditions exist. 2  The properties of the milk are constant.

Properties The average density and specific heat of milk can be taken to be milk1 kg/L  and     cp, milk= 3.79 kJ/kg.C  (Table A-3).

Analysis  The mass flow rate of the milk is

mmilk=ρVmilk(1 kg/L)(12 L/s)=12 kg/s=43,200 kg/h

Taking the pasteurizing section as the system, the energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0   →    {Ein=EoutQin+mh1=mh2   (since Δke≃Δpe≃0)Qin=mmilkcp(T2−T1)

Therefore, to heat the milk from 4 to 72C as being done currently, heat must be transferred to the milk at a rate of

Qcurrent=[mcp(Tpasturization−Trefrigeration)]milk=(12 kg/s)(3.79 kJ/kg.°C)(72−4)°C=3093 kJ/s

The proposed regenerator has an effectiveness of  = 0.82, and thus it will save 82 percent of this energy. Therefore,

Noting that the boiler has an efficiency of boiler = 0.82, the energy savings above correspond to fuel savings of

Noting that 1 year = 36524=8760 h and unit cost of natural gas is $0.52/therm, the annual fuel and money savings will be

Fuel Saved = (0.02931 therms/s)(87603600 s) = 924,450 therms/yr

Money saved =(Fuel saved)(Unit cost of fuel)=(924,450 therm/yr)($1.04/therm)=$961,400/yr

The rate of entropy generation during this process is determined by applying the rate form of the entropy balance on an extended system that includes the regenerator and the immediate surroundings so that the boundary temperature is the surroundings temperature, which we take to be the cold water temperature of 18C.:

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0(steady)underbracealignlRate of change   of entropy   →    {Sgen=Sout−Sin

Disregarding entropy transfer associated with fuel flow, the only significant difference between the two cases is the reduction is the entropy transfer to water due to the reduction in heat transfer to water, and is determined to be

Sgen, reduction=Sout, reduction=Qout,reductionTsurr=QsavedTsurr=2536 kJ/s18+273=8.715 kW/K

Sgen, reduction=Sgen, reductionΔt=(8.715 kJ/s.K)(8760×3600 s/year)=2.75×108 kJ/K(per year)

8-158 Stainless steel ball bearings leaving the oven at a uniform temperature of 900C at a rate of 1400 /min are exposed to air and are cooled to 850C before they are dropped into the water for quenching. The rate of heat transfer from the ball to the air and the rate of entropy generation due to this heat transfer are to be determined.

Assumptions 1 The thermal properties of the bearing balls are constant. 2 The kinetic and potential energy changes of the balls are negligible. 3 The balls are at a uniform temperature at the end of the process

Properties The density and specific heat of the ball bearings are given to be  = 8085 kg/m3 and cp = 0.480 kJ/kg.C.

Analysis  (a) We take a single bearing ball as the system. The energy balance for this closed system can be expressed as

Steel balls, 900C

Furnace

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energies−Qout=ΔUball=m(u2−u1)Qout=mc(T1−T2)

The total amount of heat transfer from a ball is

m=ρV=ρπD36=(8085 kg/m3)π(0.012 m)36=0.007315 kgQout=mc(T1−T2)=(0.007315 kg)(0.480 kJ/kg.°C)(900−850)°C=0.1756 kJ/ball

Then the rate of heat transfer from the balls to the air becomes

Qtotal=nballQout (per ball)=(1400 balls/min)×(0.1756 kJ/ball)=245.8 kJ/min=4.10 kW

Therefore, heat is lost to the air at a rate of 4.10 kW.

(b) We again take a single bearing ball as the system. The entropy generated during this process can be determined by applying an entropy balance on an extended system that includes the ball and its immediate surroundings so that the boundary temperature of the extended system is at 30C at all times:

Sin−SoutunderbracealignlNet entropy transfer  by heat and mass+Sgenunderbracealignl  Entropygeneration=ΔSsystemunderbracealignl Changein entropy−QoutTb+Sgen=ΔSsystem   →   Sgen=QoutTb+ΔSsystem

where

ΔSsystem=m(s2−s1)=mcavglnT2T1=(0.007315 kg)(0.480 kJ/kg.K)ln850+273900+273=−0.0001530 kJ/K

Substituting,

Sgen=QoutTb+ΔSsystem=0.1756 kJ303 K−0.0001530 kJ/K=0.0004265 kJ/K   (per ball)

Then the rate of entropy generation becomes

Sgen=Sgennball=(0.0004265 kJ/K⋅ball)(1400 balls/min)=0.597 kJ/min.K=0.00995 kW/K

8-159 An egg is dropped into boiling water. The amount of heat transfer to the egg by the time it is cooked and the amount of entropy generation associated with this heat transfer process are to be determined.

Assumptions 1 The egg is spherical in shape with a radius of r0 = 2.75 cm.  2 The thermal properties of the egg are constant. 3 Energy absorption or release associated with any chemical and/or phase changes within the egg is negligible. 4 There are no changes in kinetic and potential energies.

Properties The density and specific heat of the egg are given to be  = 1020 kg/m3 and cp = 3.32 kJ/kg.C.

Analysis  We take the egg as the system. This is a closes system since no mass enters or leaves the egg. The energy balance for this closed system can be expressed as

Egg

8C

Boiling

Water

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin=ΔUegg=m(u2−u1)=mc(T2−T1)

Then the mass of the egg and the amount of heat transfer become

m=ρV=ρπD36=(1020 kg/m3)π(0.055 m)36=0.0889 kgQin=mcp(T2−T1)=(0.0889 kg)(3.32 kJ/kg.°C)(70−8)°C=18.3  kJ

We again take a single egg as the system The entropy generated during this process can be determined by applying an entropy balance on an extended system that includes the egg and its immediate surroundings so that the boundary temperature of the extended system is at 97C at all times:

Sin−SoutunderbracealignlNet entropy transfer  by heat and mass+Sgenunderbracealignl  Entropygeneration=ΔSsystemunderbracealignl Changein entropyQinTb+Sgen=ΔSsystem   →   Sgen=−QinTb+ΔSsystem

where

ΔSsystem=m(s2−s1)=mcavglnT2T1=(0.0889 kg)(3.32 kJ/kg.K) ln70+2738+273=0.0588 kJ/K

Substituting,

Sgen=−QinTb+ΔSsystem=−18.3 kJ370 K+0.0588 kJ/K=0.00934  kJ/K  (per egg)

8-160 Long cylindrical steel rods are heat-treated in an oven. The rate of heat transfer to the rods in the oven and the rate of entropy generation associated with this heat transfer process are to be determined.

Assumptions 1 The thermal properties of the rods are constant. 2 The changes in kinetic and potential energies are negligible.

Properties The density and specific heat of the steel rods are given to be  = 7833 kg/m3 and cp = 0.465 kJ/kg.C.

Analysis  (a) Noting that the rods enter the oven at a velocity of 3 m/min and exit at the same velocity, we can say that a 3-m long section of the rod is heated in the oven in 1 min. Then the mass of the rod heated in 1 minute is

Oven, 900C

Steel rod, 30C

m=ρV=ρLA=ρL(πD24)(7833 kg/m3)(3 m)[π(0.1 m)24]=184.6 kg

We take the 3-m section of the rod in the oven as the system. The energy balance for this closed system can be expressed as

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin=ΔUrod=m(u2−u1)=mc(T2−T1)

Substituting,

Qin=mc(T2−T1)=(184.6 kg)(0.465 kJ/kg.°C)(700−30)°C=57,512 kJ

Noting that this much heat is transferred in 1 min, the rate of heat transfer to the rod becomes

Qin=QinΔt=(57,512 kJ)(1 min)=57,512 kJ/min=958.5 kW

(b) We again take the 3-m long section of the rod as the system. The entropy generated during this process can be determined by applying an entropy balance on an extended system that includes the rod and its immediate surroundings so that the boundary temperature of the extended system is at 900C at all times:

Sin−SoutunderbracealignlNet entropy transfer  by heat and mass+Sgenunderbracealignl  Entropygeneration=ΔSsystemunderbracealignl Changein entropyQinTb+Sgen=ΔSsystem   →   Sgen=−QinTb+ΔSsystem

where

ΔSsystem=m(s2−s1)=mcavglnT2T1=(184.6 kg)(0.465 kJ/kg.K)ln700+27330+273=100.1 kJ/K

Substituting,

Sgen=−QinTb+ΔSsystem=−57,512 kJ900+273 K+100.1 kJ/K=51.1 kJ/K

Noting that this much entropy is generated in 1 min, the rate of entropy generation becomes

Sgen=SgenΔt=51.1 kJ/K1 min=51.1 kJ/min.K=0.85  kW/K

8-161 The inner and outer surfaces of a brick wall are maintained at specified temperatures. The rate of entropy generation within the wall is to be determined.

Assumptions  Steady operating conditions exist since the surface temperatures of the wall remain constant at the specified values.

Analysis   We take the wall to be the system, which is a closed system. Under steady conditions, the rate form of the entropy balance for the wall simplifies to

Brick Wall

20 cm

20C

5C

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0underbracealignlRate of change   of entropy=0QinTb,in−QoutTb,out+Sgen,wall=01890 W293 K−1890 W278 K+Sgen,wall=0    Sgen,wall=0.348 W/K

Therefore, the rate of entropy generation in the wall is 0.348 W/K.

Ts = 34C

8-162 A person is standing in a room at a specified temperature. The rate of entropy transfer from the body with heat is to be determined.

Assumptions  Steady operating conditions exist.

Analysis  Noting that Q/T represents entropy transfer with heat, the rate of entropy transfer from the body of the person accompanying heat transfer is

Stransfer=QT=336  W307  K=1.094  W/K

8-163 A 1000-W iron is left on the iron board with its base exposed to the air at 20°C. The rate of entropy generation is to be determined in steady operation.

Iron

1000 W

Assumptions  Steady operating conditions exist.

Analysis We take the iron to be the system, which is a closed system.  Considering that the iron experiences no change in its properties in steady operation, including its entropy, the rate form of the entropy balance for the iron simplifies to

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0underbracealignlRate of change   of entropy=0−QoutTb,out+Sgen,iron=0

Therefore,

Sgen,iron=QoutTb,out=1000 W673 K=1.486 W/K

The rate of total entropy generation during this process is determined by applying the entropy balance on an extended system that includes the iron and its immediate surroundings so that the boundary temperature of the extended system is at 20C at all times. It gives

Sgen,total=QoutTb,out=QTsurr=1000 W293 K=3.413 W/K

Discussion Note that only about one-third of the entropy generation occurs within the iron.  The rest occurs in the air surrounding the iron as the temperature drops from 400C to 20C without serving any useful purpose.

8-164E A cylinder contains saturated liquid water at a specified pressure. Heat is transferred to liquid from a source and some liquid evaporates. The total entropy generation during this process is to be determined.

Assumptions 1 No heat loss occurs from the water to the surroundings during the process. 2 The pressure inside the cylinder and thus the water temperature remains constant during the process. 3 No irreversibilities occur within the cylinder during the process.

Analysis The pressure of the steam is maintained constant.  Therefore, the temperature of the steam remains constant also at

H2O

25 psia

Source

900F

400Btu

T=Tsat @ 25  psia=240.03°F=700 R     (Table A-5E)

Taking the contents of the cylinder as the system and noting that the temperature of water remains constant, the entropy change of the system during this isothermal, internally reversible process becomes

ΔSsystem=Qsys,inTsys=400  Btu700  R=0.572  Btu/R

Similarly, the entropy change of the heat source is determined from

ΔSsource=−Qsource,outTsource=−400  Btu900+460 R=−0.294  Btu/R

Now consider a combined system that includes the cylinder and the source. Noting that no heat or mass crosses the boundaries of this combined system, the entropy balance for it can be expressed as

Sin−SoutunderbracealignlNet entropy transfer   by heat and mass+Sgenunderbracealignl  Entropygeneration=ΔSsystemunderbracealignl Changein entropy0+Sgen,total=ΔSwater+ΔSsource

Therefore, the total entropy generated during this process is

Sgen,total=ΔSwater+ΔSsource=0.572−0.294=0.278  Btu/R

Discussion The entropy generation in this case is entirely due to the irreversible heat transfer through a finite temperature difference. We could also determine the total entropy generation by writing an energy balance on an extended system that includes the system and its immediate surroundings so that part of the boundary of the extended system, where heat transfer occurs, is at the source temperature.

8-165E Steam is decelerated in a diffuser from a velocity of 900 ft/s to 100 ft/s. The mass flow rate of steam and the rate of entropy generation are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy changes are negligible. 3 There are no work interactions.

Properties The properties of steam at the inlet and the exit of the diffuser are (Tables A-4E through A-6E)

Steam

P1 =20 psia

T1 = 240F

V1 = 900 ft/s

T2 = 240F

Sat. vapor

V2 = 100 ft/s

A2 = 1 ft2

Q

·

P1=20  psia  h1=1162.3  Btu/lbms1=1.7406  Btu/lbm⋅RT2=240°F  h2=1160.5  Btu/lbms2=1.7141  Btu/lbm⋅Rv2=16.316  ft3/lbm

Analysis (a) The mass flow rate of the steam can be determined from its definition to be

m=1v2A2V2=116.316  ft3/lbm1  ft2100 ft/s=6.129  lbm/s

(b) We take diffuser as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)−Qout=m(h2+V22/2)   (since  {W≃Δpe≃0)Qout=−mh2−h1+V22−V122

Substituting, the rate of heat loss from the diffuser is determined to be

Qout=−6.129  lbm/s1160.5−1162.3+100  ft/s2−900  ft/s221  Btu/lbm25,037  ft2/s2=108.42  Btu/s

The rate of total entropy generation during this process is determined by applying the entropy balance on an extended system that includes the diffuser and its immediate surroundings so that the boundary temperature of the extended system is 77F at all times. It gives

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0underbracealignlRate of change   of entropy=0ms1−ms2−QoutTb,surr+Sgen=0

Substituting, the total rate of entropy generation during this process becomes

Sgen=ms2−s1+QoutTb,surr=6.129  lbm/s1.7141−1.7406 Btu/lbm⋅R+108.42  Btu/s537  R=0.0395   Btu/s⋅R

8-166 Steam expands in a turbine from a specified state to another specified state. The rate of entropy generation during this process is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible.

STEAM TURBINE

P1 = 6 MPa

T1 = 450C

P2 = 20 kPa

sat. vapor

4 MW

Properties From the steam tables (Tables A-4 through 6)

P1=6 MPa  h1=3302.9  kJ/kgs1=6.7219  kJ/kg⋅KP2=20  kPa  h2=2608.9  kJ/kgs2=7.9073  kJ/kg⋅K

Analysis  There is only one inlet and one exit, and thus . We take the turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

mh1=Qout+Wout+mh2Qout=m(h1−h2)−Wout

Substituting,

Qout=(25,000/3600  kg/s)(3302.9−2608.9) kJ/kg−4000 kJ/s=819.3  kJ/s

The rate of total entropy generation during this process is determined by applying the entropy balance on an extended system that includes the turbine and its immediate surroundings so that the boundary temperature of the extended system is 25C at all times. It gives

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0underbracealignlRate of change   of entropy=0ms1−ms2−QoutTb,surr+Sgen=0

Substituting, the rate of entropy generation during this process is determined to be

Sgen=ms2−s1+QoutTb,surr=25,000/3600  kg/s7.9073−6.7219 kJ/kg⋅K+819.3  kW298  K=11.0  kW/K

8-167 A hot water stream is mixed with a cold water stream. For a specified mixture temperature, the mass flow rate of cold water stream and the rate of entropy generation are to be determined.

Assumptions 1 Steady operating conditions exist. 2 The mixing chamber is well-insulated so that heat loss to the surroundings is negligible. 3 Changes in the kinetic and potential energies of fluid streams are negligible.

Properties Noting that T < Tsat @ 200 kPa = 120.21C, the water in all three streams exists as a compressed liquid, which can be approximated as a saturated liquid at the given temperature. Thus from Table A-4,

2

H2O

200 kPa

70C

3.6 kg/s

20C

1

3

42C

P1=200  kPa  h1≃hf @ 70C=293.07  kJ/kgs1sf @ 70C=0.9551  kJ/kg⋅KP2=200  kPa  h2≃hf @ 20C=83.91  kJ/kgs2sf @ 20C=0.2965  kJ/kg⋅K

Analysis (a)  We take the mixing chamber as the system, which is a control volume. The mass and energy balances for this steady-flow system can be expressed in the rate form as

Mass balance:	min−mout=ΔEsystemä0  (steady)=0  m1+m2=m3

Energy balance:

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutmh1+m2h2=m3h3   (since   {Q=W=Δke≃Δpe≃0)

Combining the two relations gives       m1h1+m2h2=m1+m2h3

Solving for  and substituting, the mass flow rate of cold water stream is determined to be

m2=h1−h3h3−h2m1=(293.07−175.90) kJ/kg(175.90−83.91) kJ/kg(3.6  kg/s)=4.586  kg/s

Also,	m3=m1+m2=3.6+4.586=8.186  kg/s

(b) Noting that the mixing chamber is adiabatic and thus there is no heat transfer to the surroundings, the entropy balance of the steady-flow system (the mixing chamber) can be expressed as

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0underbracealignlRate of change   of entropy=0m1s1+m2s2−m3s3+Sgen=0

Substituting, the total rate of entropy generation during this process becomes

Sgen=m3s3−m2s2−m1s18.186  kg/s0.5990  kJ/kg⋅K−4.586  kg/s0.2965  kJ/kg⋅K−3.6  kg/s0.9551  kJ/kg⋅K

8-168 Liquid water is heated in a chamber by mixing it with superheated steam. For a specified mixing temperature, the mass flow rate of the steam and the rate of entropy generation are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 There are no work interactions.

2

MIXING CHAMBER

200 kPa

20C

2.5 kg/s

150C

1

3

60C

1200 kJ/min

Properties Noting that T < Tsat @ 200 kPa = 120.21C, the cold water and the exit mixture streams exist as a compressed liquid, which can be approximated as a saturated liquid at the given temperature. From Tables A-4 through A-6,

P1=200  kPa  h1≃hf @ 20C=83.91  kJ/kgs1sf @ 20C=0.2965  kJ/kg⋅KP2=200  kPa  h2=2769.1  kJ/kgs2=7.2810  kJ/kg⋅K

Analysis (a)  We take the mixing chamber as the system, which is a control volume. The mass and energy balances for this steady-flow system can be expressed in the rate form as

Mass balance:	min−mout=Δmsystemä0  (steady)=0  m1+m2=m3

Energy balance:

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutm1h1+m2h2=Qout+m3h3

Combining the two relations gives    Qout=m1h1+m2h2−m1+m2h3=m1h1−h3+m2h2−h3

Solving for  and substituting, the mass flow rate of the superheated steam is determined to be

m2=Qout−m1h1−h3h2−h3=(1200/60 kJ/s)−2.5  kg/s83.91−251.18 kJ/kg(2769.1−251.18)kJ/kg=0.166  kg/s

Also, 	m3=m1+m2=2.5+0.166=2.666  kg/s

(b) The rate of total entropy generation during this process is determined by applying the entropy balance on an extended system that includes the mixing chamber and its immediate surroundings so that the boundary temperature of the extended system is 25C at all times. It gives

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0underbracealignlRate of change   of entropy=0m1s1+m2s2−m3s3−QoutTb,surr+Sgen=0

Substituting, the rate of entropy generation during this process is determined to be

Sgen=m3s3−m2s2−m1s1+QoutTb,surr2.666  kg/s0.8313  kJ/kg⋅K0.166  kg/s

8-169 A rigid tank initially contains saturated liquid water. A valve at the bottom of the tank is opened, and half of mass in liquid form is withdrawn from the tank. The temperature in the tank is maintained constant.  The amount of heat transfer and the entropy generation during this process are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid leaving the device remains constant.  2 Kinetic and potential energies are negligible. 3 There are no work interactions involved. 4 The direction of heat transfer is to the tank (will be verified).

H2O

0.3 m3

150C

T = const.

Q

me

Properties The properties of water are (Tables A-4 through A-6)

T1=150°C  v1=vf @ 150C=0.001091  m3/kgu1=uf @150C=631.66  kJ/kgs1=sf @ 150C=1.8418  kJ/kg⋅KTe=150°C  he=hf @150C=632.18  kJ/kgse=sf @ 150C=1.8418  kJ/kg⋅K

Analysis (a) We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:

min−mout=Δmsystem   →    me=m1−m2

Energy balance:

Ein−Eoutunderbracealignl     Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin=mehe+m2u2−m1u1  (since  W≃ke≃pe≃0)

The initial and the final masses in the tank are

m1=Vv1=0.3  m30.001091  m3/kg=275.10  kg          m2=12m1=12275.10  kg=137.55  kg=me

Now we determine the final internal energy and entropy,

v2=Vm2=0.3  m3137.75  kg=0.002181  m3/kg          x2=v2−vfvfg=0.002181−0.0010910.39248−0.001091=0.002786T2=150°C u2=uf+x2ufg=631.66+0.0027861927.4=637.03  kJ/kgs2=sf+x2sfg=1.8418+0.0027864.9953=1.8557  kJ/kg⋅K

The heat transfer during this process is determined by substituting these values into the energy balance equation,

Qin=mehe+m2u2−m1u1137.55  kg632.18  kJ/kg137.55  kg637.03  kJ/kg275.10  kg

(b) The total entropy generation is determined by considering a combined system that includes the tank and the heat source. Noting that no heat crosses the boundaries of this combined system and no mass enters, the entropy balance for it can be expressed as

Sin−SoutunderbracealignlNet entropy transfer   by heat and mass+Sgenunderbracealignl  Entropygeneration=ΔSsystemunderbracealignl Changein entropy−mese+Sgen=ΔStank+ΔSsource

Therefore, the total entropy generated during this process is

Sgen=mese+ΔStank+ΔSsource=mese+(m2s2−m1s1)−Qsource,outTsource137.55  kg1.8418  kJ/kg⋅K+137.55  kg1.8557  kJ/kg⋅K275.10  kg1.8418  kJ/kg⋅K810  kJ423  K

8-170E An unknown mass of iron is dropped into water in an insulated tank while being stirred by a 200-W paddle wheel. Thermal equilibrium is established after 10 min. The mass of the iron block and the entropy generated during this process are to be determined.

Assumptions 1 Both the water and the iron block are incompressible substances with constant specific heats at room temperature. 2 The system is stationary and thus the kinetic and potential energy changes are zero. 3 The system is well-insulated and thus there is no heat transfer.

# Properties The specific heats of water and the iron block at room temperature are cp, water = 1.00 Btu/lbm.F    and cp, iron = 0.107 Btu/lbm.F (Table A-3E). The density of water at room temperature is 62.1 lbm/ft³.

Analysis (a) We take the entire contents of the tank, water + iron block, as the system. This is a closed system since no mass crosses the system boundary during the process. The energy balance on the system can be expressed as

WATER

70F

Iron

185FC

200 W

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesWpw,in=ΔU

or,

Wpw,in=[mcT2−T1]iron+[mcT2−T1]water

where

mwater=ρV=62.1   lbm/ft30.8  ft3=49.7  lbmWpw=WpwΔt=(0.2  kJ/s)(10×60  s)1  Btu1.055  kJ=113.7  Btu

Using specific heat values for iron and liquid water and substituting,

113.7  Btu=miron(0.107  Btu/lbm⋅F)(75−185)F+49.7  lbm(1.00  Btu/lbm⋅F)(75−70)Firon

(b) Again we take the iron + water in the tank to be the system. Noting that no heat or mass crosses the boundaries of this combined system, the entropy balance for it can be expressed as

Sin−SoutunderbracealignlNet entropy transfer   by heat and mass+Sgenunderbracealignl  Entropygeneration=ΔSsystemunderbracealignl Changein entropy0+Sgen,total=ΔSiron+ΔSwater

where

ΔSiron=mcavglnT2T1=11.4  lbm0.107  Btu/lbm⋅Rln535  R645  R=−0.228  Btu/RΔSwater=mcavglnT2T1=49.6  lbm1.0  Btu/lbm⋅Rln535  R530  R=0.466  Btu/R

Therefore, the entropy generated during this process is

ΔStotal=Sgen=ΔSiron+ΔSwater=−0.228+0.466=0.238 Btu/R

8-171E Air is compressed steadily by a compressor. The mass flow rate of air through the compressor and the rate of entropy generation are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy changes are negligible.  3 Air is an ideal gas with variable specific heats.

Properties The gas constant of air is 0.06855 Btu/lbm.R (Table A-1E). The inlet and exit enthalpies of air are (Table A-21E)

AIR

2

1

1,500 Btu/min

400 hp

T1=520  R h1=124.27 Btu/lbms1∘=0.59173 Btu/lbm⋅RT2=1080  Rh2=260.97 Btu/lbms2∘=0.76964 Btu/lbm⋅R

Analysis (a) We take the compressor as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

Win+m(h1+V122)=Qout+m(h2+V22/2)   (since Δpe≃0)Win−Qout=mh2−h1+V22−V122

Substituting, the mass flow rate is determined to be

Thus,	400  hp0.7068  Btu/s1  hp−1500  Btu60  s=m260.97−124.27+350  ft/s221  Btu/lbm25,037  ft2/s2

It yields		m=1.852   lbm/s

(b) Again we take the compressor to be the system. Noting that no heat or mass crosses the boundaries of this combined system, the entropy balance for it can be expressed as

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0underbracealignlRate of change   of entropy=0ms1−ms2−QoutTb,surr+Sgen=0  →   {Sgen=ms2−s1+QoutTb,surr

where

ΔSair=ms2−s1=ms2∘−s1∘−RlnP2P11.852  lbm/s0.76964−0.59173−0.06855  Btu/lbm⋅Rln150  psia15  psia=0.0372  Btu/s⋅R

Substituting, the rate of entropy generation during this process is determined to be

Sgen=ms2−s1+QoutTb,surr=0.0372 Btu/s.R+1500/60  Btu/s520  R=0.0853  Btu/s.R

8-172 Steam is accelerated in a nozzle from a velocity of 70 m/s to 320 m/s. The exit temperature and the rate of entropy generation are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy changes are negligible. 3 There are no work interactions. 4 The device is adiabatic and thus heat transfer is negligible.

Steam

P2 = 3  MPa

V2 = 320 m/s

P1 = 4  MPa

T1 = 450 C

V1 = 70 m/s

Properties From the steam tables (Table A-6),

P1=4  MPa  h1=3331.2  kJ/kgs1=6.9386  kJ/kg⋅Kv1=0.08004  m3/kg

Analysis  (a) There is only one inlet and one exit, and thus . We take nozzle as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=m(h2+V22/2)   (since  {Q≃W≃Δpe≃0)0=h2−h1+V22−V122

Substituting,,

or,	h2=3331.2  kJ/kg−320  m/s2−70  m/s221  kJ/kg1000  m2/s2=3282.4  kJ/kg

Thus,

P2=3 MPa T2=422.3°Cs2=6.9976  kJ/kg⋅K

The mass flow rate of steam is

m=1v1A1V1=10.08004  m3/kg7×10−4 m270  m/s=0.6122  kg/s

(b) Again we take the nozzle to be the system. Noting that no heat crosses the boundaries of this combined system, the entropy balance for it can be expressed as

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0underbracealignlRate of change   of entropy=0ms1−ms2+Sgen=0  Sgen=ms2−s1

Substituting, the rate of entropy generation during this process is determined to be

Sgen=ms2−s1=0.6122  kg/s6.9976−6.9386 kJ/kg⋅K=0.0361  kW/K

Review Problems

8-173E The source and sink temperatures and the thermal efficiency of a heat engine are given. The entropy change of the two reservoirs is to be calculated and it is to be determined if this engine satisfies the increase of entropy principle.

Assumptions  The heat engine operates steadily.

HE

QH

QL

Wnet

TH

TL

Analysis According to the first law and the definition of the thermal efficiency,

QL=(1−η)QH=(1−0.4)(1 Btu)=0.6 Btu

when the thermal efficiency is 40%. The entropy change of everything involved in this process is then

ΔStotal=ΔSH+ΔSLQHTH+QLTL=−1 Btu1300 R+0.6 Btu500 R=0.000431 Btu/R

Since the entropy of everything has increased, this engine is possible. When the thermal efficiency of the engine is 70%,

QL=(1−η)QH=(1−0.7)(1 Btu)=0.3 Btu

The total entropy change is then

ΔStotal=ΔSH+ΔSLQHTH+QLTL=−1 Btu1300 R+0.3 Btu500 R=−0.000169 Btu/R

which is a decrease in the entropy of everything involved with this engine.  Therefore, this engine is now impossible.

8-174 The source and sink temperatures and the COP of a refrigerator are given. The total entropy change of the two reservoirs is to be calculated and it is to be determined if this refrigerator satisfies the second law.

20°C

30°C

R

QH

1 kJ

Wnet

Assumptions  The refrigerator operates steadily.

Analysis  Combining the first law and the definition of the coefficient of performance produces

QH=QL1+1COPR=(1 kJ)1+14=1.25 kJ

when COP = 4. The entropy change of everything is then

ΔStotal=ΔSH+ΔSLQHTH+QLTL=1.25 kJ303 K+−1 kJ253 K=0.000173 kJ/K

Since the entropy increases, a refrigerator with COP = 4 is possible. When the coefficient of performance is increased to 6,

QH=QL1+1COPR=(1 kJ)1+16=1.167 kJ

and the net entropy change is

ΔStotal=ΔSH+ΔSLQHTH+QLTL=1.167 kJ303 K+−1 kJ253 K=−0.000101 kJ/K

and the refrigerator can no longer be possible.

8-175 The operating conditions and thermal reservoir temperatures of a heat pump are given. It is to be determined if the increase of entropy principle is satisfied.

260 K

300 K

HP

Assumptions  The heat pump operates steadily.

Analysis  Applying the first law to the cyclic heat pump gives

QL=QH−Wnet,in=25 kW−5 kW=20 kW

According to the definition of the entropy, the rate at which the entropy of the high-temperature reservoir increases is

ΔSH=QHTH=25 kW300 K=0.0833 kW/K

Similarly, the rate at which the entropy of the low-temperature reservoir decreases is

ΔSL=QLTL=−20 kW260 K=−0.0769 kW/K

The rate at which the entropy of everything changes is then

ΔStotal=ΔSH+ΔSL=0.0833−0.0769=0.0064 kW/K

which is positive and therefore it satisfies the increase in entropy principle.

8-176

T

s

2

1

Steam is expanded adiabatically in a closed system. The minimum internal energy that can be achieved during this process is to be determined.

Analysis The entropy at the initial state is

11

The internal energy will be minimum if the process is isentropic. Then,

221

8-177E Water is expanded in an isothermal, reversible process. It is to be determined if the process is possible.

Analysis The entropies at the initial and final states are (Tables A-5E and A-6E)

11

T

s

2

1

221

The heat transfer during this isothermal, reversible process is the area under the process line:

q=T(s2−s1)(250+460 K)(1.8277−1.3001) Btu/lbm⋅R374.6 Btu/lbm

The total entropy change (i.e., entropy generation) is the sum of the entropy changes of water and the reservoir:

Δstotal=Δswater+ΔsRs2−s1+−qTR(1.8277−1.3001) Btu/lbm⋅R+−374.6 Btu/lbm(300+460) R0.0347 Btu/lbm⋅R

Note that the sign of heat transfer is with respect to the reservoir.

8-178E Air is compressed adiabatically in a closed system. It is to be determined if this process is possible.

Assumptions 1 Changes in the kinetic and potential energies are negligible.  4 Air is an ideal gas with constant specific heats.

Properties The properties of air at room temperature are R = 0.3704 psiaft3/lbmR, cp = 0.240 Btu/lbmR (Table A-2Ea).

Analysis  The specific volume of air at the initial state is

v1=RT1P1=(0.3704 psia⋅ft3/lbm⋅R)(560 R)16 psia=12.96 ft3/lbm

The volume at the final state will be minimum if the process is isentropic. The specific volume for this case is determined from the isentropic relation of an ideal gas to be

v2,min=v1P1P21k=(12.96 ft3/lbm)16 psia100 psia11.4=3.500 ft3/lbm

and the minimum volume is

V2=mv2=(2 lbm)(3.500 ft3/lbm)=7.00 ft3

which is greater than the proposed volume 4 ft3/lbm. Hence, it is not possible to compress this air into 4 ft3/lbm.

8-179 Oxygen is expanded adiabatically in a piston-cylinder device. The maximum volume is to be determined.

Assumptions 1 Changes in the kinetic and potential energies are negligible.  4 Oxygen is an ideal gas with constant specific heats.

Properties The gas constant of oxygen is R = 0.2598 kPam3/kgK. The specific heat ratio at the room temperature is k = 1.395 (Table A-2a).

Analysis  The volume of oxygen at the initial state is

V1=mRT1P1=(3 kg)(0.2598 kPa⋅m3/kg⋅K)(373+273 K)950 kPa=0.5300 m3

The volume at the final state will be maximum if the process is isentropic. The volume for this case is determined from the isentropic relation of an ideal gas to be

V2,max=V1P1P21k=(0.5300 m3)950 kPa100 kPa11.395=2.66 m3

8-180E A solid block is heated with saturated water vapor. The final temperature of the block and water, and the entropy changes of the block, water, and the entire system are to be determined.

Assumptions 1 The system is stationary and thus the kinetic and potential energy changes are zero. 2 There are no work interactions involved. 3 There is no heat transfer between the system and the surroundings.

Analysis (a) As the block is heated, some of the water vapor will be condensed. We will assume (will be checked later) that the water is a mixture of liquid and vapor at the end of the process. Based upon this assumption, the final temperature of the water and solid block is 212F (The saturation temperature at 14.7 psia). The heat picked up by the block is

Qblock=mc(T2−T1)=(100 lbm)(0.5 Btu/lbm⋅R)(212−70)R=7100 Btu

The water properties at the initial state are

11	(Table A-5E)

The heat released by the water is equal to the heat picked up by the block. Also noting that the pressure of water remains constant, the enthalpy of water at the end of the heat exchange process is determined from

h2=h1−Qwatermw=1150.3 Btu/lbm−7100 Btu10 lbm=440.3 Btu/lbm

The state of water at the final state is saturated mixture. Thus, our initial assumption was correct. The properties of water at the final state are

22

The entropy change of the water is then

ΔSwater=mw(s2−s1)=(10 lbm)(0.69947−1.7566)Btu/lbm=−10.57 Btu/R

(b) The entropy change of the block is

ΔSblock=mclnT2T1=(100 lbm)(0.5 Btu/lbm⋅R)ln(212+460)R(70+460)R=11.87 Btu/R

(c) The total entropy change is

ΔStotal=Sgen=ΔSwater+ΔSblock=−10.57+11.87=1.30 Btu/R

The positive result for the total entropy change (i.e., entropy generation) indicates that this process is possible.

8-181 Air is compressed in a piston-cylinder device. It is to be determined if this process is possible.

Assumptions 1 Changes in the kinetic and potential energies are negligible.  4 Air is an ideal gas with constant specific heats. 3 The compression process is reversible.

Properties The properties of air at room temperature are R = 0.287 kPam3/kgK, cp = 1.005 kJ/kgK (Table A-2a).

Analysis We take the contents of the cylinder as the system. This is a closed system since no mass enters or leaves. The energy balance for this stationary closed system can be expressed as

Air

100 kPa

27°C

Heat

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesWb,in−Qout=ΔU=m(u2−u1)    Wb,in−Qout=mcp(T2−T1)Wb,in−Qout=0      (since T2=T1)Qout=Wb,in

The work input for this isothermal, reversible process is

win=RTlnP2P1=(0.287 kJ/kg⋅K)(300 K)ln250 kPa100 kPa=78.89 kJ/kg

That is,

qout=win=78.89 kJ/kg

The entropy change of air during this isothermal process is

Δsair=cplnT2T1−RlnP2P1=−RlnP2P1=−(0.287 kJ/kg⋅K)ln250 kPa100 kPa=−0.2630 kJ/kg⋅K

The entropy change of the reservoir is

ΔsR=qRTR=78.89 kJ/kg300 K=0.2630 kJ/kg⋅K

Note that the sign of heat transfer is taken with respect to the reservoir.  The total entropy change (i.e., entropy generation) is the sum of the entropy changes of air and the reservoir:

Δstotal=Δsair+ΔsR=−0.2630+0.2630=0 kJ/kg⋅K

Not only this process is possible but also completely reversible.

8-182 A paddle wheel does work on the water contained in a rigid tank. For a zero entropy change of water, the final pressure in the tank, the amount of heat transfer between the tank and the surroundings, and the entropy generation during the process are to be determined.

Assumptions The tank is stationary and the kinetic and potential energy changes are negligible.

Analysis (a) Using saturated liquid properties for the compressed liquid at the initial state (Table A-4)

Wpw

Water

120C

500 kPa

T1=120°C u1=503.60 kJ/kgs1=1.5279 kJ/kg.K

The entropy change of water is zero, and thus at the final state we have

T2=95°CP2=84.6 kPau2=492.63 kJ/kg

(b) The heat transfer can be determined from an energy balance on the tank

Qout=WPw,in−m(u2−u1)=22 kJ−(1.5 kg)(492.63−503.60)kJ/kg=38.5 kJ

(c) Since the entropy change of water is zero, the entropy generation is only due to the entropy increase of the surroundings, which is determined from

Sgen=ΔSsurr=QoutTsurr=38.5 kJ(15+273) K=0.134 kJ/K

8-183 A horizontal cylinder is separated into two compartments by a piston, one side containing nitrogen and the other side containing helium. Heat is added to the nitrogen side. The final temperature of the helium, the final volume of the nitrogen, the heat transferred to the nitrogen, and the entropy generation during this process are to be determined.

Assumptions 1 Kinetic and potential energy changes are negligible. 2 Nitrogen and helium are ideal gases with constant specific heats at room temperature. 3 The piston is adiabatic and frictionless.

Properties The properties of nitrogen at room temperature are R = 0.2968 kPa.m3/kg.K, cp = 1.039 kJ/kg.K, cv = 0.743 kJ/kg.K, k = 1.4. The properties for helium are R = 2.0769 kPa.m3/kg.K, cp = 5.1926 kJ/kg.K, cv = 3.1156 kJ/kg.K, k = 1.667 (Table A-2).

Q

He

0.1 kg

N2

0.2 m3

Analysis (a) Helium undergoes an isentropic compression process, and thus the final helium temperature is determined from

THe,2=T1P2P1(k−1)k=(20+273)K120 kPa95 kPa(1.667−1)1.667321.7 K

(b) The initial and final volumes of the helium are

VHe,1=mRT1P1=(0.1 kg)(2.0769  kPa⋅m3/kg⋅K)(20+273  K)95 kPa=0.6406 m3

VHe,2=mRT2P2=(0.1 kg)(2.0769  kPa⋅m3/kg⋅K)(321.7  K)120 kPa=0.5568 m3

Then, the final volume of nitrogen becomes

VN2,2=VN2,1+VHe,1−VHe,2=0.2+0.6406−0.5568=0.2838 m3

(c) The mass and final temperature of nitrogen are

mN2=P1V1RT1=(95 kPa)(0.2 m3)(0.2968  kPa⋅m3/kg⋅K)(20+273  K)=0.2185 kg

TN2,2=P2V2mR=(120 kPa)(0.2838 m3)(0.2185 kg)(0.2968  kPa⋅m3/kg⋅K)=525.1 K

The heat transferred to the nitrogen is determined from an energy balance

Qin=ΔUN2+ΔUHemcv(T2−T1)N2+mcv(T2−T1)He(0.2185 kg)(0.743 kJ/kg.K)(525.1−293)+(0.1 kg)(3.1156 kJ/kg.K)(321.7−293)46.6 kJ

(d) Noting that helium undergoes an isentropic process, the entropy generation is determined to be

Sgen=ΔSN2+ΔSsurr=mN2cplnT2T1−RlnP2P1+−QinTR(0.2185 kg)(1.039 kJ/kg.K)ln525.1 K293 K−(0.2968 kJ/kg.K)ln120 kPa95 kPa+−46.6 kJ(500+273) K0.057 kJ/K

8-184 An electric resistance heater is doing work on carbon dioxide contained an a rigid tank. The final temperature in the tank, the amount of heat transfer, and the entropy generation are to be determined.

Assumptions 1 Kinetic and potential energy changes are negligible. 2 Carbon dioxide is ideal gas with constant specific heats at room temperature.

Properties The properties of CO2 at an anticipated average temperature of 350 K are R = 0.1889 kPa.m3/kg.K, cp = 0.895 kJ/kg.K, cv = 0.706 kJ/kg.K (Table A-2b).

We

CO2

250 K

100 kPa

Analysis (a) The mass and the final temperature of CO2 may be determined from ideal gas equation

m=P1VRT1=(100 kPa)(0.8 m3)(0.1889  kPa⋅m3/kg⋅K)(250  K)=1.694 kg

T2=P2VmR=(175 kPa)(0.8 m3)(1.694 kg)(0.1889  kPa⋅m3/kg⋅K)=437.5 K

(b) The amount of heat transfer may be determined from an energy balance on the system

Qout=Ee,inΔt−mcv(T2−T1)(0.5 kW)(40×60 s)−(1.694 kg)(0.706 kJ/kg.K)(437.5-250)K=975.8  kJ

(c) The entropy generation associated with this process may be obtained by calculating total entropy change, which is the sum of the entropy changes of CO2 and the surroundings

Sgen=ΔSCO2+ΔSsurr=mcplnT2T1−RlnP2P1+QoutTsurr(1.694 kg)(0.895 kJ/kg.K)ln437.5 K250 K−(0.1889 kJ/kg.K)ln175 kPa100 kPa+975.8 kJ300 K3.92 kJ/K

8-185 Heat is lost from the helium as it is throttled in a throttling valve. The exit pressure and temperature of helium and the entropy generation are to be determined.

Helium

500 kPa

70C

q

Assumptions 1 Steady operating conditions exist. 2 Kinetic and potential energy changes are negligible. 3 Helium is an ideal gas with constant specific heats.

Properties The properties of helium are R = 2.0769 kPa.m3/kg.K, cp = 5.1926 kJ/kg.K (Table A-2a).

Analysis (a) The final temperature of helium may be determined from an energy balance on the control volume

qout=cp(T1−T2)T2=T1−qoutcp=70°C−2.5 kJ/kg5.1926 kJ/kg.°C=342.5 K=69.5°C

The final pressure may be determined from the relation for the entropy change of helium

ΔsHe=cplnT2T1−RlnP2P10.25 kJ/kg.K=(5.1926 kJ/kg.K)ln342.5 K343 K−(2.0769 kJ/kg.K)lnP2500 kPaP2=441.7 kPa

(b) The entropy generation associated with this process may be obtained by adding the entropy change of helium as it flows in the valve and the entropy change of the surroundings

sgen=ΔsHe+Δssurr=ΔsHe+qoutTsurr=0.25 kJ/kg.K+2.5 kJ/kg(25+273) K=0.258 kJ/kg.K

8-186 Refrigerant-134a is compressed in a compressor. The rate of heat loss from the compressor, the exit temperature of R-134a, and the rate of entropy generation are to be determined.

Assumptions 1 Steady operating conditions exist. 2 Kinetic and potential energy changes are negligible.

Analysis (a) The properties of R-134a at the inlet of the compressor are (Table A-12)

Compressor

R-134a

200 kPa

sat. vap.

700 kPa

Q

P1=200 kPa v1=0.09987 m3/kgh1=244.46 kJ/kgs1=0.93773 kJ/kg.K

The mass flow rate of the refrigerant is

m=V1v1=0.03 m3/s0.09987 m3/kg=0.3004 kg/s

Given the entropy increase of the surroundings, the heat lost from the compressor is

ΔSsurr=QoutTsurrQout=TsurrΔSsurr=(20+273 K)(0.008 kW/K)=2.344 kW

(b) An energy balance on the compressor gives

Win−Qout=m(h2−h1)10 kW-2.344 kW=(0.3004 kg/s)(h2-244.46) kJ/kgh2=269.94 kJ/kg

The exit state is now fixed. Then,

P2=700 kPa T2=31.5°Cs2=0.93620 kJ/kg.K

(c) The entropy generation associated with this process may be obtained by adding the entropy change of R-134a as it flows in the compressor and the entropy change of the surroundings

Sgen=ΔSR+ΔSsurr=m(s2−s1)+ΔSsurr(0.3004 kg/s)(0.93620-0.93773) kJ/kg.K+0.008 kW/K0.00754 kJ/K

8-187 Air flows in an adiabatic nozzle. The isentropic efficiency, the exit velocity, and the entropy generation are to be determined.

Properties The gas constant of air is R = 0.287 kJ/kg.K (Table A-1).

Assumptions 1 Steady operating conditions exist. 2 Potential energy changes are negligible.

Analysis (a) (b) Using variable specific heats, the properties can be determined from air table as follows

Air

500 kPa

400 K

30 m/s

300 kPa

350 K

T1=400 K    h1=400.98 kJ/kgs10=1.99194 kJ/kg.KPr1=3.806T2=350 K    h2=350.49 kJ/kgs20=1.85708 kJ/kg.KPr2=P2P1Pr1=300  kPa500  kPa3.806=2.2836h2s=346.31 kJ/kg

Energy balances on the control volume for the actual and isentropic processes give

h1+V122=h2+V222400.98 kJ/kg+(30 m/s)221 kJ/kg1000 m2/s2=350.49 kJ/kg+V2221 kJ/kg1000 m2/s2                                                    V2=319.1 m/s

h1+V122=h2s+V2s22400.98 kJ/kg+(30 m/s)221 kJ/kg1000 m2/s2=346.31 kJ/kg+V2s221 kJ/kg1000 m2/s2                                                    V2s=331.8 m/s

The isentropic efficiency is determined from its definition,

ηN=V22V2s2=(319.1 m/s)2(331.8 m/s)2=0.925

(c) Since the nozzle is adiabatic, the entropy generation is equal to the entropy increase of the air as it flows in the nozzle

sgen=Δsair=s20−s10−RlnP2P1(1.85708−1.99194)kJ/kg.K−(0.287 kJ/kg.K)ln300 kPa500 kPa=0.0118 kJ/kg.K

8-188 It is to be shown that the difference between the steady-flow and boundary works is the flow energy.

Analysis The total differential of flow energy Pv can be expressed as

dPv=P dv+v dP=δwb−δwflow=δwb−wflow

Therefore, the difference between the reversible steady-flow work and the reversible boundary work is the flow energy.

8-189 An insulated rigid tank is connected to a piston-cylinder device with zero clearance that is maintained at constant pressure. A valve is opened, and some steam in the tank is allowed to flow into the cylinder.  The final temperatures in the tank and the cylinder are to be determined.

Assumptions 1 Both the tank and cylinder are well-insulated and thus heat transfer is negligible.  2 The water that remains in the tank underwent a reversible adiabatic process. 3 The thermal energy stored in the tank and cylinder themselves is negligible. 4 The system is stationary and thus kinetic and potential energy changes are negligible.

Analysis (a) The steam in tank A undergoes a reversible, adiabatic process, and thus s2 = s1.  From the steam tables (Tables A-4 through A-6),

P1=500  kPa  v1=vg @ 500  kPa=0.37483  m3/kgu1=ug @ 500  kPa=2560.7  kJ/kgs1=sg @ 500  kPa=6.8207  kJ/kg⋅KP2=150  kPa s2=s1 T2, A=Tsat @ 150  kPa=111.35°Cx2, A=s2, A−sfsfg=6.8207−1.43375.7894=0.9305v2, A=vf+x2, Avfg=0.001053+(0.9305)(1.1594−0.001053)=1.0789  m3/kgu2, A=uf+x2, Aufg=466.97+(0.9305)(2052.3  kJ/kg)=2376.6  kJ/kg

The initial and the final masses in tank A are

Thus,	m1, A=VAv1, A=0.4  m30.37483  m3/kg=1.067  kg    and   m2, A=VAv2, A=0.4  m31.0789  m3/kg=0.371  kg2, B

(b) The boundary work done during this process is

Sat.

vapor

500 kPa

0.4 m3

150 kPa

Wb,out= P dV=PBV2, B−0=PBm2, Bv2, B

Taking the contents of both the tank and the cylinder to be the system, the energy balance for this closed system can be expressed as

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energies−Wb,out=ΔU=ΔUA+ΔUB

or,	Wb,out+ΔUA+ΔUB=0PBm2, Bv2, B+m2u2−m1u1A+m2u2B=02, B2, Bm2u2−m1u1A

Thus,

h2, B=m1u1−m2u2Am2, B=1.0672560.7−0.3712376.60.696=2658.8  kJ/kg

At 150 kPa, hf = 467.13 and hg = 2693.1 kJ/kg.  Thus at the final state, the cylinder will contain a saturated liquid-vapor mixture since hf < h2 < hg.  Therefore,

T2, B=Tsat @ 150  kPa=111.35°C

8-190 Carbon dioxide is compressed in a reversible isothermal process using a steady-flow device. The work required and the heat transfer are to be determined.

Assumptions 1 Steady operating conditions exist. 2 Changes in the kinetic and potential energies are negligible.  3 CO2 is an ideal gas with constant specific heats.

Properties The gas constant of CO2 is R = 0.1889 kPam3/kgK (Table A-2a).

Analysis There is only one inlet and one exit, and thus m1=m2=m. We take the compressor as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Compressor

400 kPa

20°C

100 kPa

20°C

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

mh1+Win−Qout=mh2Win−Qout=m(h2−h1)=mcp(T2−T1)Win=Qout    (since T2=T1)

The work input for this isothermal, reversible process is

win=RTlnP2P1=(0.1889 kJ/kg⋅K)(293 K)ln400 kPa100 kPa=76.7 kJ/kg

From the energy balance equation,

qout=win=76.7 kJ/kg

8-191 Carbon dioxide is compressed in an isentropic process using a steady-flow device. The work required and the heat transfer are to be determined.

Assumptions 1 Steady operating conditions exist. 2 Changes in the kinetic and potential energies are negligible.  3 CO2 is an ideal gas with constant specific heats.

Properties The gas constant of CO2 is R = 0.1889 kPam3/kgK. Other properties at room temperature are cp = 0.846 kJ/kgK and k =1.289 (Table A-2a).

Analysis There is only one inlet and one exit, and thus m1=m2=m. We take the compressor as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Compressor

400 kPa

100 kPa

20°C

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

mh1+Win=mh2Win=m(h2−h1)=mcp(T2−T1)

The temperature at the compressor exit for the isentropic process of an ideal gas is

T2=T1P2P1(k−1)k=(293 K)400 kPa100 kPa0.2891.289=399.8 K

Substituting,

win=cp(T2−T1)=(0.846 kJ/kg⋅K)(399.8−293)K=90.4 kJ

The work input increases from 76.7 kJ/kg to 90.4 kJ/kg when the process is executed isentropically instead of isothermally.  Since the process is isentropic (i.e., reversible, adiabatic), the heat transfer is zero.

8-192 R-134a is compressed in an isentropic compressor. The work required is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 The process is isentropic (i.e., reversible-adiabatic). 3 Kinetic and potential energy changes are negligible.

Analysis There is only one inlet and one exit, and thus m1=m2=m. We take the compressor as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Compressor

-10°C

sat. vapor

800 kPa

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

mh1+Win=mh2Win=m(h2−h1)

T

s

2

1

800 kPa

-10C

The inlet state properties are

11

For this isentropic process, the final state enthalpy is

221

Substituting,

win=h2−h1=(273.22−244.51) kJ/kg=28.7 kJ/kg

8-193 Refrigerant-134a is expanded adiabatically in a capillary tube. The rate of entropy generation is to be determined.

Assumptions 1 Steady operating conditions exist. 2 Kinetic and potential energy changes are negligible.

Analysis The rate of entropy generation within the expansion device during this process can be determined by applying the rate form of the entropy balance on the system. Noting that the system is adiabatic and thus there is no heat transfer, the entropy balance for this steady-flow system can be expressed as

R-134a

50C

sat. liq.

12C

Capillary tube

Sin−SoutunderbracealignlRate of net entropy transfer      by heat and mass+SgenunderbracealignlRate of entropy    generation=ΔSsystemÃ0(steady)underbracealignlRate of change   of entropym1s1−m2s2+Sgen=0Sgen=m(s2−s1)sgen=s2−s1

It may be easily shown with an energy balance that the enthalpy remains constant during the throttling process. The properties of the refrigerant at the inlet and exit states are (Tables A-11 through A-13)

T1=50°C    h1=123.50 kJ/kg⋅Ks1=0.44193 kJ/kg⋅K

T2=−12°C    x2=h2−hfhfg=123.50−35.92207.38=0.4223s2=sf+x2sfg=0.14504+(0.4223)(0.79406)=0.48038 kJ/kg⋅K

Substituting,

Sgen=m(s2−s1)=(0.2 kg/s)(0.48038−0.44193) kJ/kg⋅K=0.00769 kW/K

8-194 Steam is expanded in an adiabatic turbine. Six percent of the inlet steam is bled for feedwater heating. The isentropic efficiencies for two stages of the turbine are given. The power produced by the turbine and the overall efficiency of the turbine are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 The turbine is well-insulated, and there is no heat transfer from the turbine.

Analysis There is one inlet and two exits. We take the turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Turbine

4 MPa

350°C

30 kPa

800 kPa

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

T

s

3

1

4 MPa

30 kPa

0.8 MPa

2

3s

m1h1=m2h2+m3h3+WoutWout=m1h1−m2h2−m3h3wout=h1−0.06h2−0.94h3wout=(h1−h2)+0.94(h2−h3)

The isentropic and actual enthalpies at three states are determined using steam tables as follows:

11

221

ηT,1=h1−h2h1−h2s  h2=h1−ηT,1(h1−h2s)=3093.3−(0.97)(3093.3−2734.0)=2744.8 kJ/kg

22  x2=0.9885s2=6.6086 kJ/kg⋅K

ηT,2=h2−h3h2−h3s  h3=h2−ηT,2(h2−h3s)=2744.8−(0.95)(2744.8−2227.9)=2253.7 kJ/kg

Substituting,

wout=(h1−h2)+0.94(h2−h3)(3093.3−2744.8)+0.94(2744.8−2253.7)810.1 kJ/kg-inlet

The overall isentropic efficiency of the turbine is

ηT=(h1−h2)+0.94(h2−h3)(h1−h2s)+0.94(h2−h3s)=(3093.3−2744.8)+0.94(2744.8−2253.7)(3093.3−2734.0)+0.94(2744.8−2227.9)=810.1845.2=0.958=95.8%

8-195 Air is compressed steadily by a compressor from a specified state to a specified pressure. The minimum power input required is to be determined for the cases of adiabatic and isothermal operation.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 Air is an ideal gas with variable specific heats. 4 The process is reversible since the work input to the compressor will be minimum when the compression process is reversible.

Properties The gas constant of air is R = 0.287 kJ/kg.K (Table A-1).

Analysis (a) For the adiabatic case, the process will be reversible and adiabatic (i.e., isentropic),

thus the isentropic relations are applicable.

and	T1=290  K  Pr1=1.2311  and  h1=290.16  kJ/kgPr2=P2P1Pr1=700  kPa100  kPa(1.2311)=8.6177     →   T2=503.3  Kh2=506.45  kJ/kg

s =const.

2

1

T =const.

2

1

The energy balance for the compressor, which is a steady-flow system, can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

Win+mh1=mh2  →    {Win=m(h2−h1)

Substituting, the power input to the compressor is determined to be

Win=(5/60  kg/s)(506.45−290.16) kJ/kg=18.0 kW

(b)  In the case of the reversible isothermal process, the steady-flow energy balance becomes

Ein=Eout  →   {Win+mh1−Qout=mh2  →    {Win=Qout+m(h2−h1)Ã0=Qout

since h = h(T) for ideal gases, and thus the enthalpy change in this case is zero. Also, for a reversible isothermal process,

where  	Qout=mTs1−s2=−mTs2−s1s2−s1=s2∘−s1∘Ã0−RlnP2P1=−RlnP2P1=−0.287  kJ/kg⋅K ln700  kPa100  kPa=−0.5585  kJ/kg⋅K

Substituting, the power input for the reversible isothermal case becomes

Win=−(5/60  kg/s)(290  K)(−0.5585  kJ/kg⋅K)=13.5  kW

8-196 Refrigerant-134a is compressed by a 0.7-kW adiabatic compressor from a specified state to another specified state. The isentropic efficiency, the volume flow rate at the inlet, and the maximum flow rate at the compressor inlet are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible.

Properties From the R-134a tables (Tables A-11 through A-13)

R-134a

2

1

V1

0.7 kW

·

P1=140  kPa   v1=0.14605  m3/kgh1=246.36  kJ/kgs1=0.9724  kJ/kg⋅KP2=700  kPa h2=288.53  kJ/kg

Analysis (a) The isentropic efficiency is determined from its definition,

ηC=h2s−h1h2a−h1=281.16−246.36288.53−246.36=0.825=82.5%

(b) There is only one inlet and one exit, and thus . We take the actual compressor as the system, which is a control volume. The energy balance for this steady-flow system can be expressed as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

Wa,in+mh1=mh2     (since   {Q≃Δke≃Δpe≃0)Wa,in=m(h2−h1)

Then the mass and volume flow rates of the refrigerant are determined to be

m=Wa,inh2a−h1=0.7  kJ/s288.53−246.36 kJ/kg=0.0166  kg/sV1=mv1=0.0166  kg/s0.14605  m3/kg=0.00242  m3/s=145  L/min

(c)  The volume flow rate will be a maximum when the process is isentropic, and it is determined similarly from the steady-flow energy equation applied to the isentropic process. It gives

mmax=Ws,inh2s−h1=0.7  kJ/s281.16−246.36 kJ/kg=0.0201  kg/sV1,max=mmaxv1=0.0201  kg/s0.14605  m3/kg=0.00294  m3/s=176   L/min

Discussion Note that the raising the isentropic efficiency of the compressor to 100% would increase the volumetric flow rate by more than 20%.

8-197E Helium is accelerated by a 94% efficient nozzle from a low velocity to 1000 ft/s. The pressure and temperature at the nozzle inlet are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Helium is an ideal gas with constant specific heats. 3 Potential energy changes are negligible. 4 The device is adiabatic and thus heat transfer is negligible.

Properties The specific heat ratio of helium is k = 1.667. The constant pressure specific heat of helium is 1.25 Btu/lbm.R (Table A-2E).

Analysis We take nozzle as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

HELIUM

hN = 94%

1

2

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=m(h2+V22/2)   (since  {Q≃W≃Δpe≃0)0=h2−h1+V22−V122  0=cp,avgT2−T1+V22−V122

Solving for T1 and substituting,

T1=T2a+V2s2−V122CpÃ0=180°F+1000  ft/s221.25  Btu/lbm⋅R1  Btu/lbm25,037  ft2/s2=196.0°F=656 R

From the isentropic efficiency relation,

or,	ηN=h2a−h1h2s−h1=cPT2a−T1cPT2s−T1T2s=T1+T2a−T1ηN=656+640−6560.94=639  R

From the isentropic relation,  T2sT1=P2P1k−1k

P1=P2T1T2skk−1=14  psia656  R639  R1.667/0.667=14.9   psia

8-198 … 8-201  Design and Essay Problems

